3GPP TSG-RAN WG2 Meeting #112-e	R2-2009519

# Electronic, 2-13 November 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0931 | rev | - | Current version: | 16.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on TX UE handling of last transmission of MAC PDU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, InterDigital Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-10-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When Sidelink TX UE handles the last transmission of a TB, the description below from LTE-V2X spec is reused: 1> if this transmission corresponds to the last transmission of the MAC PDU: 2> decrement SL_RESOURCE_RESELECTION_COUNTER by 1, if available. However, in LTE-V2X, there is no HARQ feedback supported. The “last transmission of MAC PDU” simply means the UE has reached its limit of blind retransmissions. There is no preemption used in LTE SL resource selection, either. Therefore, the meaning of “last transmission”, if simply reused in NR SL mode 2, will be interpreted to a much wider scope and cover many different scenarios. This reuse will definitely create confusion to a reader, especially when there are descriptions in the same procedure which require the UE to flush HARQ buffer after receiving HARQ feedback, but not explicitly require UE to decrement the SL_RESOURCE_RESELECTION_COUNTER. Therefore, it is suggested to add a NOTE to clarify this. Also, the normative requirements for TX UE to flush HARQ TX buffer when receiving an affirmative HARQ feedback shall be only limited to mode 1 UE, Mode 2 UE does not need to flush HARQ TX buffer because UE implementation can simply use the newly arrived TB to replace the old data in the same buffer. Finally, in the description of the condition check whether mode 1 UE reaches the configured maximum retry limit of a MAC PDU, the text reads as to compare one maximal number with another maximum value, which is a grammar error and can cause confusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Added new “if-else” seapraiton to limit “flush HARQ buffer” operaiton to mode 1 UE and “SL_RESOURCE_RESELECTION_COUNTER change” behavior to mode 2 UE, respectively. 2. Added a NOTE explaining that the last transmisison of MAC PDU can be determined by mode 2 UE by a couple of different scenarios, incluiding HARQ feedback, congestion control, preemption, etc. 3. Removed “to sl-MaxTransNum” from the mode 1 UE condition to check whether it reaches maximum retry limit. Impact analysis Impacted architecture options: NR SL Impacted functionality: SL HARQ Process Inter-operability: No inter-operability impacts are foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect normative requirements will remain in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.22.1.3.1a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

##### 5.22.1.3.1a	Sidelink process

The Sidelink process is associated with a HARQ buffer.

New transmissions and retransmissions are performed on the resource indicated in the sidelink grant as specified in clause 5.22.1.1 and with the MCS selected as specified in clause 8.1.3.1 of TS 38.214 [7] and clause 5.22.1.1.

If the Sidelink process is configured to perform transmissions of multiple MAC PDUs with Sidelink resource allocation mode 2, the process maintains a counter SL_RESOURCE_RESELECTION_COUNTER. For other configurations of the Sidelink process, this counter is not available.

If the Sidelink HARQ Entity requests a new transmission, the Sidelink process shall:

1>	store the MAC PDU in the associated HARQ buffer;

1>	store the sidelink grant received from the Sidelink HARQ Entity;

1>	generate a transmission as described below.

If the Sidelink HARQ Entity requests a retransmission, the Sidelink process shall:

1>	store the sidelink grant received from the Sidelink HARQ Entity;

1>	generate a transmission as described below.

To generate a transmission, the Sidelink process shall:

1>	if there is no uplink transmission; or

1>	if the MAC entity is able to simultaneously perform uplink transmission(s) and sidelink transmission at the time of the transmission; or

1>	if the other MAC entity and the MAC entity are able to simultaneously perform uplink transmission(s) and sidelink transmission at the time of the transmission respectively; or

1>	if there is a MAC PDU to be transmitted for this duration in uplink, except a MAC PDU obtained from the Msg3 buffer, the MSGA buffer, or prioritized as specified in clause 5.4.2.2, and the sidelink transmission is prioritized over uplink transmission:

2>	instruct the physical layer to transmit SCI according to the stored sidelink grant with the associated Sidelink transmission information;

2>	instruct the physical layer to generate a transmission according to the stored sidelink grant;

2>	if HARQ feedback has been enabled the MAC PDU according to clause 5.22.1.4.2:

3>	instruct the physical layer to monitor PSFCH for the transmission and perform PSFCH reception as specified in clause 5.22.1.3.2.

2>	if sl-PUCCH-Config is configured by RRC for the stored sidelink grant:

3>	determine transmission of an acknowledgement on the PUCCH as specified in clause 5.22.1.3.2.

- if the MAC entity has been configured with Sidelink resource allocation mode 1:

2>	if sl-MaxTransNum corresponding to the highest priority of the logical channel(s) in the MAC PDU has been configured in sl-CG-MaxTransNumList for the sidelink grant by RRC and the maximum number of transmissions of the MAC PDU has been reached; or

2>	if a positive acknowledgement to a transmission of the MAC PDU has been received according to clause 5.22.1.3.2; or

2>	if negative-only acknowledgement was enabled in the SCI and no negative acknowledgement was received for the most recent (re-)transmission of the MAC PDU according to clause 5.22.1.3.2:

3>	flush the HARQ buffer of the associated Sidelink process.

- else:

2> if this transmission corresponds to the last transmission of the MAC PDU:

3> decrement SL_RESOURCE_RESELECTION_COUNTER by 1, if available.

NOTE 1:	The UE can determine whether this transmission corresponds to the last transmission of the MAC PDU when reaching the maximum number of retransmissions, receiving HARQ ACK, not receiving groupcast NACK, or when the retransmission is unable to be performed due to congestion control or pre-emption.

The transmission of the MAC PDU is prioritized over uplink transmissions of the MAC entity or the other MAC entity if the following conditions are met:

1>	if the MAC entity is not able to perform this sidelink transmission simultaneously with all uplink transmissions at the time of the transmission, and

1>	if uplink transmission is neither prioritized as specified in clause 5.4.2.2 nor prioritized by upper layer according to TS 23.287 [19]; and

1>	if sl-PrioritizationThres is configured and if the value of the highest priority of logical channel(s) or a MAC CE in the MAC PDU is lower than sl-PrioritizationThres.

NOTE 2:	If the MAC entity is not able to perform this sidelink transmission simultaneously with all uplink transmissions as specified in clause 5.4.2.2 of TS 36.321 [22] at the time of the transmission, and prioritization-related information is not available prior to the time of this sidelink transmission due to processing time restriction, it is up to UE implementation whether this sidelink transmission is performed.

End of Changes