3GPP TSG-RAN WG2 Meeting #112-e	R2-2009520

# Electronic, 2-13 November 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2085 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Stored Sidelink Measurement Configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-10-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When UE acquires SIB12, if the UE has active unicast connection to a peer UE and the SL measurement configuration has been modified in SIB12, the UE shall trigger a sidelink reconfiguration, but this part is missing in the spec. When UE is in coverage, UE will obtain Sidelink Measurement Configuration represented from either SIB 12 or from dedicated RRC signaling and store it. However, there is no explicit procedure to indicate that the stored configuration is discarded when UE moves out of coverage. When UE moves out of coverage, it shall not use the stored SL measurement configuration, but use the configuration from pre-configuration. Also, it is not clear how UE can obtain SL measurement configuration per destination if the configuration is not obtained via dedicated RRC signaling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. When UE acquires SIB12,added the action to perform sidelink reconfiguraiton. 2. Split the description on how to set SL measurement configuration in the RRCReconfiguraionSideink in three different cases: - Out of coverage case: UE shall use the setting in preconfiguration - SIB12: UE gets a common configuration for SL measurement w/o destination information. - Dedicated signaling: UE use the stored SL measurement configuration for this destination. . Impact analysis Impacted architecture options: NR SL Impacted functionality: Sidelink Reconfiguration Inter-operability: If a NR Sidelink UE implements the CR and the peer Sidelink UE does not, then the UE may not be updated by the peer UE with the latest configuration, and continue to use the incorrect configuration for SL measurement (e.g., perform measurements in wrong SL frequency). As a result, it may provide misleading/faulty UE measurements to the peer UE and cause unnecessery radio link maintenance problems |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong SL measurement configuration will be used in Sidelink measurements and reporting. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.4.13, 5.8.9.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

##### 5.2.2.4.13	Actions upon reception of SIB12

Upon receiving SIB12, the UE shall:

1>	if the UE has stored at least one segment of SIB12 and the value tag of SIB12 has changed since a previous segment was stored:

2>	discard all stored segments;

1>	store the segment;

1>	if all segments have been received:

2>	assemble SIB12-IEs from the received segments;

2>	if sl-FreqInfoList is included in sl-ConfigCommonNR:

3>	if configured to receive NR sidelink communication:

4>	use the resource pool(s) indicated by sl-RxPool for NR sidelink communication reception, as specified in 5.8.7;

3>	if configured to transmit NR sidelink communication:

4>	use the resource pool(s) indicated by sl-TxPoolSelectedNormal, or sl-TxPoolExceptional for NR sidelink communication transmission, as specified in 5.8.8;

4>	perform CBR measurement on the transmission resource pool(s) indicated by sl-TxPoolSelectedNormal and sl-TxPoolExceptional for NR sidelink communication transmission, as specified in 5.5.3.1;

4>	use the synchronization configuration parameters for NR sidelink communication on frequencies included in sl-FreqInfoList, as specified in 5.8.5;

2>	if sl-RadioBearerConfigList or sl-RLC-BearerConfigList is included in sl-ConfigCommonNR:

3>	perform sidelink DRB reconfiguration as specified in 5.8.9.1a;

2> if sl-MeasConfigCommon is included in sl-ConfigCommonNR:

3> store the NR sidelink measurement configuration;

3> if the UE is in RRC_IDLE or RRC_INACTIVE:

4> if sl-MeasConfigCommon is different from stored NR sidelink measurement configuration, and there is an active unicast connection to any peer UE, performs sidelink RRC reconfiguration procedure as specified in clause 5.8.9.1.

The UE should discard any stored segments for SIB12 if the complete SIB12 has not been assembled within a period of 3 hours. The UE shall discard any stored segments for SIB12 upon cell (re-) selection.

Next Change

##### 5.8.9.1.2	Actions related to transmission of RRCReconfigurationSidelink message

The UE shall set the contents of RRCReconfigurationSidelink message as follows:

1>	for each sidelink DRB that is to be released, according to sub-clause 5.8.9.1a.1.1, due to configuration by sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR or by upper layers:

2>	set the SLRB-PC5-ConfigIndex included in the slrb-ConfigToReleaseList corresponding to the sidelink DRB;

1>	for each sidelink DRB that is to be established or modified, according to sub-clause 5.8.9.1a.2.1, due to receiving sl-ConfigDedicatedNR, SIB12 or SidelinkPreconfigNR:

2>	set the SLRB-Config included in the slrb-ConfigToAddModList, according to the received sl-RadioBearerConfig and sl-RLC-BearerConfig corresponding to the sidelink DRB;

- set the sl-MeasConfig as follows:

2>	if UE is out of coverage:

3> set the sl-MeasConfig according to the sl-MeasPreconfig in SidelinkPreconfigNR;

2> else if UE is in RRC_CONNECTED and has stored NR sidelink measurement configuration information for this destination when receiving it via dedicated configuration:

3> set the sl-MeasConfig according to stored NR sidelink measurement configuration information for this destination;

2>	else if UE is in RRC_IDLE or RRC_INACTIVE:

3> set the sl-MeasConfig according to stored NR sidelink measurement received from SIB12.

1>	start timer T400 for the destination associated with the sidelink DRB;

1>	set the sl-CSI-RS-Config;

1>	set the sl-LatencyBound-CSI-Report,

NOTE 1:	How to set the parameters included in sl-CSI-RS-Config and sl-LatencyBound-CSI-Report is up to UE implementation.

The UE shall submit the RRCReconfigurationSidelink message to lower layers for transmission.

End of Changes