3GPP TSG-RAN WG2 Meeting #112 Electronic	R2-2009550

Elbonia, 02 – 13 November 2020

Agenda item:	6.8.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Dormant BWP and UE capabilities

WID/SID:	LTE_NR_DC_CA_enh-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

In this paper we discuss SCell dormancy and BWP requirements to support the feature.

# 2	SCell dormancy and BWPs

When looking at current release 16 feature groups (annex B) we can see that group 18-4 indicates whether UE support SCell dormancy indication within active time. It also indicates:

- One dormant BWP and one non-dormant BWP is supported per carrier

- More than one non-dormant BWP per carrier is supported only if UE feature 6-3/6-4 is also supported

18-4 does not explicitly write that UE would need to support two dedicated BWPs but it says that UE needs to support at least one dormant and one non-dormant BWP.

On the other hand the capability indicates as well that if more than one non-dormant BWP is to be supported then UE needs to support 6-3/6-4 (see annex A) which requires up to 4 dedicated DL BWPs to be supported. So this seems to assume that UE would need to support at least 6-2 (two dedicated DL BWPs) for the basic 18-4 capability. If only one dedicated DL BWP would be required then NW would always need to set up initial BWP as dormant/non-dormant BWP, which would make the feature not that useful as initial BWP supports only CORESET#0. Thus we think that 6-2 is missing as requirement for 18-4 capability:

Proposal 1: 6-2 should be prerequisite for 18-4 capability i.e. UE needs to support DCI-based BWP switching and two dedicated DL BWPs to support Scell dormancy.

Generally, it would be possible to consider that dormant BWP are not part of “BWP budget” but additional to what UE supports. That is,  if UE supports 6-2 (two dedicated DL BWPs) then UE could still support having a dormant BWP configured as additional one. But it should at least be possible that UE supporting dormancy can always be configured with one (dedicated) non-dormant BWP and one (dedicated) dormant BWP. For UEs supporting only 6-2, we would also be fine to consider that UE supporting SCell dormancy can always support 2 (dedicated) non-dormant BWP and 1 (dedicated) dormant BWP (i.e. 3 BWPs in total in terms of signalling), but at least currently this is not how the capability is written and this could be see as a change to the interpretatin of the Rel-15 capabilities. The RAN1 wording also seems to suggest that dormancy doesn't really change the "BWP budget" by itself, so it seems reasonable to assume that the dormancy just utilizes what UE already supports for BWP operation via the Rel-15 capabilities.

Proposal 2: Dormant BWPs are counted in the “BWP budget” for dedicated BWPs, e.g. UE supporting 2 dedicated BWPs could be configured with 1 dormant BWP in addition to one non-dormant BWP (i.e. within the limits of Rel-15 signalling).

We would note that it's currently allowed to configure at most 4 dedicated BWPs Rel-15. In theory the signalling supports 5 dedicated BWPs (using configuration option where BWP#0 uses a dedicated configuration, see Annex B.2) so no new signalling may be needed, but this would not be possible with all BWP configuration options.

Finally, we would note that it would be good to inform RAN1 of the RAN2 conclusions to avoid misalignments.

Proposal 3: Inform RAN1 on the decisions concerning above proposals.

# 3	Conclusion

Proposal 1: 6-2 should be prerequisite for 18-4 capability i.e. UE needs to support DCI-based BWP switching and two dedicated DL BWPs to support Scell dormancy.

Proposal 2: Dormant BWPs are counted in the “BWP budget” for dedicated BWPs, e.g. UE supporting 2 dedicated BWPs could be configured with 1 dormant BWP in addition to one non-dormant BWP (i.e. within the limits of Rel-15 signalling).

Proposal 3: Inform RAN1 on the decisions concerning above proposals.

# Annex A: 38.822-15.0.1 on BWP feature groups

| 6. CA/DC, BWP, SUL; 6-1; Basic BWP operation with restriction; 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 3) RRC reconfiguration of any parameters related to BWP 4) BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell; n/a; n/a; n/a; n/a; This feature should be mandatory without capability signalling for at least BWPs which is the same as the set of specified channel BW UE-specific RRC configured DL/UL BWP can have the same or different numerology from the initial active DL/UL BWP; Mandatory without capability signalling. 6-1a; BWP operation without restriction on BW of BWP(s); BW of UE-specific RRC configured BWP may not include BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP may not include SSB for SCell; 6-1, 6-2, 6-3, or 6-4; bwp-WithoutRestriction; BandNR; n/a; n/a; 6-1a is applicable to 6-1, 6-2, 6-3, or 6-4.; Optional with capability signalling. 6-2; Type A BWP adaptation with same numerology; 1) Up to 2 UE-specific RRC configured DL BWPs per carrier 2) Up to 2 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) Same numerology for all the UE-specific RRC configured BWPs per carrier 5) BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell; 6-1; upto2 in bwp-SameNumerology; BandNR; n/a; n/a; Optional with capability signalling. 6-3; Type B BWP adaptation with same numerology; 1) Up to 4 UE-specific RRC configured DL BWPs per carrier 2) Up to 4 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) Same numerology for all the UE-specific RRC configured BWPs per carrier 5) BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell; 6-1; upto4 in bwp-SameNumerology; BandNR; n/a; n/a; Optional with capability signalling. 6-4; BWP adaptation with different numerologies; 1) Up to 4 UE-specific RRC configured DL BWPs per carrier 2) Up to 4 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) More than one numerologies for the UE-specific RRC configured BWPs per carrier 5) Same numerology between DL and UL per cell except for SUL at a given time 6) BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell; 6-1; upto4 in bwp-DiffNumerology; BandNR; n/a; n/a; Optional with capability signalling. 6-5; Basic DL NR-NR CA operation; 1) Up to16 DL carriers 2) Same numerology across carrier for data/control channel at a given time; supportedBandCombinationList; RF-Parameters; n/a; n/a; This is conditioned on the support of DL CA band combination(s).; Optional with capability signalling. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# Annex B: Dormant BWP related feature groups in 38.822 (R1-2007137)

| 18. MR-DC/CA enhancement | 18-4 | SCell dormancy indication within active time | Support for SCell dormancy indication sent within the active time on PCell with DCI format 0_1/1_1 | 6-5 | Yes | N/A |  | Per BC | N/A | N/A | N/A | One dormant BWP and one non-dormant BWP is supported per carrier More than one non-dormant BWP per carrier is supported only if UE feature 6-3/6-4 is also supported | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18. MR-DC/CA enhancement | 18-4a | SCell dormancy indication outside active time | Support for SCell dormancy indication sent outside the active time on PCell with DCI format 2_6 | 19-1 | Yes | N/A |  | Per BC | N/A | N/A | N/A | One dormant BWP and one non-dormant BWP is supported per carrier More than one non-dormant BWP per carrier is supported only if UE feature 6-3/6-4 is also supported | Optional with capability signalling |