3GPP TSG RAN WG2 Meeting #112 electronic                                	R2-2009595

Online, November 2020

Source: 	Asia Pacific Telecom

Title: 	Discussion on HARQ and RACH aspects in NTN

Agenda item:	8.10.2.1

Document for:	Discussion and Decision

# Introduction

In RAN2#111-e, the following agreements were achieved.

Agreement in RAN2#111-e - From RAN2 perspective, an offset is applied to the start of ra-ResponseWindow in NTN for both LEO and GEO scenarios. - An offset to the start of the ra-ContentionResolutionTimer is introduced for both LEO and GEO scenarios. - Modification of drx-LongCycleStartOffset, drx-StartOffset, drx-ShortCycle, drx-ShortCycleTimer, drx-onDurationTimer, drx-SlotOffset and drx-InactivityTimer is not needed in Rel-17 NTN. - From a RAN2 perspective, for DL, HARQ feedback can be enabled/disabled in Rel-17 NTN, but HARQ processes remain configured. The criteria and decision to enable/disable HARQ feedback is under network control and is signalled to the UE via RRC in a semi-static manner. FFS for UL Agreement in RAN2#111-e - Both 2-step and 4-step RACH are supported in Rel-17 NTN. FFS enhancements to RACH to accommodate the NTN environment.

In this contribution, we will provide further discussion on

- CG timer impact based on disabling of HARQ-ACK for UL retransmission

- Enhancements on RACH type selection

# Discussion

## CG timer impact based on disabling of HARQ-ACK for UL retransmission

From a RAN2 perspective, HARQ uplink retransmission at the UE transmitter can be enabled/disabled in Rel-17 NTN. HARQ uplink retransmission at the UE transmitter can be enabled/disabled per HARQ process. HARQ disabling means NW may schedule the same HARQ process ID in consecutive PDSCH/PUSCH allocations (using the same or toggled NDI) to decrease the delay caused by the stop-and-wait mechanism.

Based on the previous discussion, if HARQ feedback is disabled, the user plane impacts may be introduced on DRX related timers (e.g., drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL,  drx-RetransmissionTimerDL, drx-RetransmissionTimerUL). However, except for the DRX related timer, the configuredGrantTimer may also have an impact. It is noted that the configuredGrantTimer is configured per HARQ process, and the spirit of the configuredGrantTimer is to protect the TB in the corresponding HARQ process to give more time for waiting for the possible retransmission from NW. Nonetheless, if the HARQ process is disabled, it seems no need to protect the TB transmitted by CG.

configuredGrantTimer is not needed on top of the disabling of HARQ-ACK for UL retransmission.

## Enhancements on RA type selection

In Rel-16 NR, RA types can be either 2-step or 4-step RA types. The related spec context is provided below.

3GPP TS 38.321 V16.2.1 (2020-09) 1> else if the BWP selected for Random Access procedure is configured with both 2-step and 4-step RA type Random Access Resources and the RSRP of the downlink pathloss reference is above msgA-RSRP-Threshold; or 1> if the BWP selected for Random Access procedure is only configured with 2-step RA type Random Access resources (i.e. no 4-step RACH RA type resources configured); or 1> if the Random-Access procedure was initiated for reconfiguration with sync and if the contention-free Random Access Resources for 2-step RA type have been explicitly provided in rach-ConfigDedicated for the BWP selected for Random Access procedure: 2> set the RA_TYPE to 2-stepRA. 1> else: 2> set the RA_TYPE to 4-stepRA.

In NR, it is possible to only configure with 2-step RA resource in a serving cell or a UE (e.g., 4-step RA resource is not configured). However, considering the 2-step RA capacity is expected to be lower than the 4-step RA capacity. The 2-Step RA capability may not achieve the required UE density without 4-step RA resources.

If both 2-step and 4-step RA type Random Access Resources are configured in a cell, the legacy RA type selection is determined by UE based on the RSRP measurement. However, since no near-far effect in NTN, all UEs in a cell may have similar RSRP levels. Hence, all UEs in a cell may select 2-step RACH RA type or select 4-step RACH RA type only.

Since no near-far effect in NTN, measurement-based RA type selection leads to a single RA type.

Figure 1 shows the impact of elevation angles and the corresponding RSRP in a cell. Strong RSRP occurs when a satellite is on top of a UE, and weak RSRP for a small elevation angle. This is because 1) the propagation distance gets longer when the elevation angle decreases; 2) Line-Of-Sight (LOS) probability is smaller as the elevation angle decreases, which increases the chance of blocking. Especially, for an earth-fixed cell, UE may experience the best (90 degrees of elevation) and the worst (10 degrees of elevation) RSRP quality during the service. In other words, the NW would experience a burst request from the UEs for either a single RA type of 2-step or 4-step RA.

Figure 1: elevation angles of a serving cell and the RSRP quality

Enhancements on RA type selection shall be needed.

# Conclusion

In this contribution, we have the following observations.

Observation 1	Since no near-far effect in NTN, measurement-based RA type selection leads to a single RA type.

Based on the observation, we have the follwing proposals.

Proposal 1	configuredGrantTimer is not needed on top of the disabling of HARQ-ACK for UL retransmission.

Proposal 2	Enhancements on RA type selection shall be needed.

# Reference

Chairman’s Notes, 3GPP TSG RAN WG1 #102 e-meeting

Chairman’s Notes, 3GPP TSG RAN WG2 #111 e-meeting

3GPP TR 38.821 V16.0.0, Solutions for NR to support non-terrestrial networks, December 2019.

3GPP TS 38.321 V16.2.1, Medium Access Control (MAC) protocol specification.