3GPP TSG-RAN WG2 Meeting #112 e                  	R2-2009688

E-Meeting, 2nd – 13th Nov, 2020

Source:	vivo

Title:	Remaining issues on RACH and service continuity

Agenda Item:	8.8.3

Document for:	Discussion and Decision

- Introduction

At RAN2#111-e meeting, RAN2 has made the following agreements on RAN slicing discussion:

Agreements:

- Scenarios for now to be studied by RAN2:

- Multiple and different slices can be supported on different frequencies

- Multiple and different slices can be supported in the same frequency layer in different regions.

2	For each scenario we study both IDLE and INACTIVE and determine whether there is need for a solution and possible solutions.  Connected mode will also be considered but with a lower priority.

3	RAN2 will study both cell selection and cell re-selection

Further, RAN2 has agreed to the following email discussion:

- [Post111-e][916][NR RAN slicing] RAN slicing study questions (CMCC)

Scope: Based on online agreements. Discuss issues to address in the SI and in which deployment scenarios, meaning of “intended slice”. Can also discuss candidate solutions (including whether Rel-15 mechanisms can work), e.g. slice-based reselection or slice-based RACH.

At RAN3#109-e, on slice re-mapping at handover, RAN3 agreed that:

Re-mapping in connected mode shall be decided by the network (RAN and/or Core)

This contribution discusses potential solution on identified issues on RAN.s

- Discussion

  - Slice based RACH configuration

To allow UE direct and fast access to its intended slice, there could be some correlation between supported slice and RACH resources, for example to access slice 1 network reserves RACH resource 1 and for slice 2 network reserves RACH resource 2. So, when accessing slice 1, UE uses RACH resource 1. Thus, the correlation can be RACH resources per slice.

RACH configuration per slice would also allow to enforce UE access control, for UE cannot uses RACH resource 2 to access slice 1, alternatively, if network does not allocate RACH resource 1, UE cannot access to slice 1.

- RACH configuration per slice is a way to enforce UE access control.

Therefore,

- Dedicated RACH resource should be configured per supported slice.

During the email discussion, for the purpose of slice-based RACH configuration, the following to cases were widely acknowledged:

- Intention 1: RA resource isolation. From marketing point of view, some of the industrial customers have the requirement for access resource isolation, in order to provide guaranteed RA resources for their sensitive slices.

- Intention 2: Slice access prioritization. In R15/16, all slices are sharing the same RA resources and cannot be differentiated by network side. But some slices may need to be prioritized during the RA procedure.

For the above intentions, the following candidate solutions available:

- Solution 1: Separate RACH resources pool can be configured per slice or per slice group, in addition to the existing common RACH resources.

For this solution, RACH fragmentation/partitioning would be required. Particularly for the case of huge number of slices supported by the cell. Without strong requirement, RACH fragmentation should be avoided due to separate RACH resource pools. Thus, avoid too many parameters to be sent to UE.

- Solution 2: RACH parameters prioritization can be configured per slice.

This solution is simple and does not required RACH fragmentation.

Thus,

- Solution 2 with RACH prioritization configured per slice is simple and does not require RACH fragmentation

Therefore,

- RACH parameters prioritization configured per slice is baseline solution for slice based RACH configuration

  - Service continuity

Figure 3: Slice based UE handover

When UE is performing handover, to guarantee smooth UE service continuity, if the target cell supports the slice that the UE accessed in source cell (e.g., URLLC slice), after handover UE should have access to URLLC slice. Thus, during handover preparation, source cell and target cell should exchange RAN slice information on the slice UE accessed prior to handover. Then, target cell can reserve and allocate the necessary RACH resource for allowing UE direct access to URLLC slice at target cell.

If the target cell does not support the slice UE accessed in source cell, the target cell can perform slice remapping to slice that can, as best effort, satisfy UE service in target cell.

Therefore,

- RAN slice information are exchanged during handover preparation to allow adequate slice remapping for UE service continuity

- Conclusion

This paper discussed some remaining issues on RAN slices. The paper concludes with:

Observation 1:	RACH configuration per slice is a way to enforce UE access control.

Observation 2:	Solution 2 with RACH prioritization configured per slice is simple and does not require RACH fragmentation

Proposal 1：	Dedicated RACH resource should be configured per supported slice.

Proposal 2：	RACH parameters prioritization configured per slice is baseline solution for slice based RACH configuration

Proposal 3：	RAN slice information are exchanged during handover preparation to allow adequate slice remapping for UE service continuity

- References

- S2-2006526, Ls on Cell Configuration within TA/RA to Support Allowed NSSAI