3GPP TSG-RAN WG2 Meeting #112e	R2-2009717

# Electronic Meeting, 2nd – 13th November, 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2121 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on setting frequencyBandListFilterSidelink over PC5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Over PC5, the field frequencyBandListFilterSidelink is exchanged between two sidelink UEs when the capability trasfer procedure is triggered. However, the filter should be aligned to what has been requested already by the network in the UECapabilityEnquiry message (if the UE that triggered the sidelink capability procedure already exchanged its capabilities with the network) in order to not cause any misalignment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 5.8.9.2.3 - Added a note in the procedural text where is clarified that the initiating UE, if in coverage on the frequency used for the NR sidelink communication, should set the content of frequencyBandListFilterSidelink according to the UECapabilityEnquiry message, if previously received by the network. Impact Analysis Impacted 5G architecture options: Sidelink Impacted functionality: Sidelink capability transfer Inter-operability: 1. If the network is implemented according to the CR and the UE is not, the UE may request to its peer UE capabilities for certain frequencies bands that are not the same of that one requested by the network (or may be not supported by the network). 2. If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. 3. If a UE is implemented according to the CR and the other UE is not, the initiating UE may request to its peer UE capabilities for certain frequencies bands that are not the same of that one requested by the network (or may be not supported by the network). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not approved, it may happen that the initiating UE will request to the peer UE capability for frequencies bands that are not in line with the capability filter requested by the network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.9.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 5.8.9.2	Sidelink UE capability transfer

#### 5.8.9.2.1	General

This clause describes how the UE compiles and transfers its sidelink UE capability information for unicast to the initiating UE.

Figure 5.8.9.2.1-1: Sidelink UE capability transfer

#### 5.8.9.2.2	Initiation

The UE may initiate the sidelink UE capability transfer procedure upon indication from upper layer when it needs (additional) UE radio access capability information.

#### 5.8.9.2.3	Actions related to transmission of the UECapabilityEnquirySidelink by the UE

The initiating UE shall set the contents of UECapabilityEnquirySidelink message as follows:

1>	include in UE radio access capabilities for sidelink within ue-CapabilityInformationSidelink, if needed;

NOTE:	It is up to initiating UE to decide whether ue-CapabilityInformationSidelink should be included.

1>	set frequencyBandListFilterSidelink to include frequency bands for which the peer UE is requested to provide supported bands and band combinations;

1>	submit the UECapabilityEnquirySidelink message to lower layers for transmission.

NOTE:	The initiating UE, if in coverage on the frequency used for the NR sidelink communication, should set the content of frequencyBandListFilterSidelink according to the UECapabilityEnquiry message, if previously received from the network.

END OF CHANGES