3GPP TSG-RAN WG2 #112e	R2-2009720

Electronic meeting, 2nd – 13th November, 2020

Agenda Item:	8.7.3.1

Source:	Ericsson

Title:	Discussion on L3 UE-to-NW relay architecture

Document for:	Discussion, Decision

# 1	Introduction

In RAN2#111-e, RAN2 made the following agreements on L3 relay:

Agreements: - On user plane protocol stacks of L3 UE-to-NW relay, capture the followings in RAN2 TR: - SA2 captured two user plane protocol stacks for L3 UE-to-NW relay in TR 23.752 (Figure 6.6.1-2 of solution#6 and Figure 6.23.2-3 of solution#23). No impacts are identified to support them from RAN2 perspective. - In RAN2 TR, capture Figure 6.6.2-1 of TR 23.752 with a reference to SA2 TR with identified RAN2 impacts analysis. Relay (re)selection is added after the step of “Discovery”. Other procedures identified with RAN2 impact can also be added in the Figure. RAN2 will further consider procedures with RAN2 impact. - Leave discussion on Relay / Remote UE authorization in email discussion#606 - In TR, add one editor note “whether new PC5-S signaling is also introduced depends on SA2” - On QoS support, capture in TR: SA2 captured two solutions for QoS support of L3 UE-to-NW relay: - PCF sets separate Uu QoS parameters and PC5 QoS parameters in solution#25 of TR 23.752. - End-to-End QoS support in solution#24 of TR 23.752, where relay can obtain a mapping between PQI and 5QI from SMF/PCF - After relay obtains the mapping between PQI and 5QI from SMF/PCF (in solution#24 of [1]), RAN2 further discuss whether it is sufficient to enforce E2E QoS via legacy PC5 RRC reconfiguration of SLRB and resource allocation. - RAN2 don’t intend to study QoS enhancement for L3 UE-to-NW relay to SA2 (e.g. whether gNB can perform PDB split). RAN2 can discuss AS impacts related to SA2 specified QoS solutions. - Remote UE doesn’t need to provide information on which QoS flows need to be relayed to relay in AS layer. - On security, capture in TR: SA2 captured two solutions for security support of L3 UE-to-NW relay: - Via legacy Uu security and PC5 security - Via N3IWF in solution #23 of TR 23.752 - RAN2 will evaluate any impact in RAN2 scope from these solutions. - In TR, capture that “Rel-16 NR V2X PC5-RRC establishment procedure is reused to setup a secure unicast link between Remote UE and Relay UE before unicast traffic relaying”. - RAN2 leaves control plane protocol stacks of L3 UE-to-NW relay to SA2. And RAN2 TR adds a reference to SA2 TR. FFS if there is RAN2 impact to support the related control plane procedures. - RAN2 leaves protocol stacks of L3 UE-to-UE relay to SA2. And RAN2 TR adds a reference to SA2 TR. - First focus on the study of control plane procedures which are similar to L3 UE-to-NW relay and L3 UE-to-UE relay - Postpone the discussion on whether sending LS to SA3 for RAN specific security questions of L3 UE-to-NW relay

In this paper, we will address further aspects on the L3 UE-to-NW relay architecture.

# 2	Discussion

## 2.1	IP router based L3 UE to NW relay

Solution #6 in [1] defines an IP router based L3 UE to NW relay mechanism, Figure 1 below shows the corresponding protocol stack. For IP PDU Session Type and IP traffic over PC5 reference point, the relay UE allocates IPv6 prefix or IPv4 address for the remote UE, this implies that the remote UE’s IP address will be changed during path switching, either when switched to a direct Uu path or switched to an indirect path.

Figure 1. Protocol stack for IP router based L3 UE-to-Network Relay

For PDU Session of IPv4 or IPv6 or IPv4v6 type, to achieve session continuity the IP address needs to be preserved for the lifetime of the PDU Session. This implies that IP router based L3 UE to NW relay cannot achieve session continuity. However, according to [2], it can be seen that "service continuity" is different from "session continuity" by definition. More specifically, service continuity means the uninterrupted user experience of a service, including the cases where the IP address and/or anchoring point change. An uninterrupted user experience does not necessarily mean no data loss, rather it depends on the service type, for instance, for video service the user experience will not impacted as long as the data loss does not exceed a certain level.

Service continuity is different from "session continuity and IP preservation is not necessary.

Service continuity does not necessarily mean no data loss, the acceptable data loss level depends on service type.

RAN2 to capture the SA2 service continuity definition in [2] into the TR 38.836, and clarify that it is not equal to no data loss.

Service continuity could be achieved at application layer. For instance, [3] defines service continuity between on-network Mission Critical service and UE-to-network relay Mission Critical service. Figure 2 below shows the procedure when the UE switches from on-network Mission Critical service to UE-to-network relay Mission Critical service, i.e. switching from direct path to indirect path. It should be noted that step 2 could be started before the current connection is lost, i.e. session level make before break is supported at the application layer, by which the service interruption can be made unnoticeable to the end user.

To ensure that step 2 is started before the current connection is lost, some AS layer optimization may be needed, on the other hand, this can be implemented by tuning the existing parameter(s) that control the path switching with the presence of sidelink relay, i.e. no new AS mechanism is needed and no RAN impact is foreseen.

Service continuity could be achieved at application layer with already defined mechanism where session level make before break is supported.

The mechanism has no RAN impact.

RAN2 to capture in TR 38.836 that session level make before break is one way to achieve service continuity.

Figure 2. Service continuity when UE switches from on-network Mission Critical service to UE-to-network relay Mission Critical service

## 2.2	Non-3GPP interworking function (N3IWF) based L3 UE to NW relay

- Solution #23 in [1] defines N3IWF based L3 UE to NW relay. It is to support end-to-end traffic confidentiality and/or IP address preservation. The solution is optional and complementary to baseline L3 UE-to-Network Relay solutions (e.g. solution #6), i.e. it is primarily for the services that requires end-to-end traffic confidentiality and/or IP address preservation.

- To provide end-to-end security for the remote UE traffic, the design of "untrusted non-3GPP access to 5GC via N3IWF" in clause 4.2.8 of [2] or "Access to PLMN services via stand-alone non-public networks" in clause 5.30.2.7 of [2] is leveraged. Remote UE registers to 5GC via N3IWF and establishes corresponding PDU sessions. The data traffic over the PDU sessions are protected by IPSec between the Remote UE and N3IWF.

- To provide IP address preservation, remote UE follows the procedure specified in clause 4.9.2 of [4] (Handover of a PDU Session procedure between 3GPP and untrusted non-3GPP access) when the UE moves between direct communication path and indirect communication path. Session continuity can be achieved since IP address is preserved during path switching.

- When access to N3IWF is used, the UE-to-Network relay is able to relay both control plane (NAS) and user plane unicast traffic (UL and DL) between the remote UE and the network towards N3IWF. Figure 3 below shows the user plane protocol stack with N3IWF based L3 UE to NW relay while Figure 4 shows the control plane protocol stack.

- The solution is transparent to NG-RAN. The NG-RAN does not have any different treatment for the remote UE's traffic comparing to that in baseline L3 UE-to-Network Relay solutions (e.g. solution #6).

Figure 3. User plane protocol stacks for N3IWF router based L3 UE-to-Network Relay

Figure 4. Control plane protocol stacks for N3IWF router based L3 UE-to-Network Relay

N3IWF based L3 UE to NW relay supports end-to-end traffic confidentiality and/or IP address preservation.

N3IWF based L3 UE to NW relay can achieve session continuity.

The solution has no RAN impact.

RAN2 to capture in TR 38.836 the following:

  - N3IWF based L3 UE to NW relay supports end-to-end traffic confidentiality and/or IP address preservation;

  - N3IWF based L3 UE to NW relay can achieve session continuity;

  - N3IWF based L3 UE to NW relay has no RAN impact.

- In [5] it was argued that the increased overhead for N3IWF based L3 UE to NW relay is an issue in terms of decreased capacity. This is true for service with small packet size such as voice, for service with large packet size such as video the impact is much less significant. Besides, the capacity decrease is inevident in typical PS scenarios such as Fire Brigade and Connected Ambulance, where there are a mix of voice and video services.

Overhead caused by N3IWF is limited for service with large packet size.

The capacity decrease is inevident in typical PS scenarios with a mix of voice and video services.

- In [5] it was also argued that setup of IPsec tunnel would cause additional some latency. This is true, but on the other hand, with N3IWF there is no need to do end to end session reestablishment, which is needed in IP router based L3 UE to NW relay, and end to end session reestablishment likely needs more time than establishing an IPsec tunnel. Besides, with session continuity the application is not required to have the capability to handle IP address changes, which means service continuity could be achieved even for such kind of application.

N3IWF based L3 UE to NW relay could provide better performance than IP router based L3 UE to NW relay in terms of service continuity.

RAN2 to capture in TR 38.836 that N3IWF based L3 UE to NW relay could provide better performance than IP router based L3 UE to NW relay in terms of service continuity.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Service continuity is different from "session continuity and IP preservation is not necessary.

Observation 2	Service continuity does not necessarily mean no data loss, the acceptable data loss level depends on service type.

Observation 3	Service continuity could be achieved at application layer with already defined mechanism where session level make before break is supported.

Observation 4	The mechanism has no RAN impact.

Observation 5	N3IWF based L3 UE to NW relay supports end-to-end traffic confidentiality and/or IP address preservation.

Observation 6	N3IWF based L3 UE to NW relay can achieve session continuity.

Observation 7	The solution has no RAN impact.

Observation 8	Overhead caused by N3IWF is limited for service with large packet size.

Observation 9	The capacity decrease is inevident in typical PS scenarios with a mix of voice and video services.

Observation 10	N3IWF based L3 UE to NW relay could provide better performance than IP router based L3 UE to NW relay in terms of service continuity.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to capture the SA2 service continuity definition in [2] into the TR 38.836, and clarify that it is not equal to no data loss.

Proposal 2	RAN2 to capture in TR 38.836 that session level make before break is one way to achieve service continuity.

Proposal 3	RAN2 to capture in TR 38.836 the following:

a.	N3IWF based L3 UE to NW relay supports end-to-end traffic confidentiality and/or IP address preservation;

b.	N3IWF based L3 UE to NW relay can achieve session continuity;

c.	N3IWF based L3 UE to NW relay has no RAN impact.

Proposal 4	RAN2 to capture in TR 38.836 that N3IWF based L3 UE to NW relay could provide better performance than IP router based L3 UE to NW relay in terms of service continuity.

# References

3GPP TR 23.752: “Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS) (Release 17)”

3GPP TS 23.501: “System Architecture for the 5G System; Stage 2 (Release 16)”

3GPP TS 23.280: “Common functional architecture to support mission critical services; Stage 2 (Release 16)”

3GPP TS 23.502: “Procedures for the 5G System; Stage 2 (Release 16)”

R2-2006572, MediaTek Inc., “Architecture options for Sidelink Relay”, RAN2#111e