3GPP TSG-RAN WG2 meeting#112-e	R2-2009729

# Online,  2nd  - 13th November 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.304 | CR | 0814 | rev |  | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification to the last used cell for GWUS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh3-Core, LTE_eMTC5-Core |  |  |  |  |  | Date: |  |  | 2020-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A GWUS capable eNB indicates to the CN the last used cell when releasing the S1/NG Connection. In case, this is not done the eNB include the field nolastcellUpdate in RRCConnectionRelease message to inform the UE that it should not update the cell that was last used for GWUS. However, the description in section 7.5.1 only mentions WUS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 7.5.1, clarify that after reception of RRCConnectionRelease including noLastCellUpdate, the UE monitors GWUS in the cell it was using GWUS prior to this RRC connection attempt. Impact Analysis Impacted functionality: GWUS Inter-operability: The CR only impacts the UE, no inter-operability issue is foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may not monitor GWUS after noLastCellUpdate was included in the RRCConnectionRelease even though it was previoulsy using GWUS in this cell. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.331 CR 4479 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 7.5.1	General

Paging with Group Wake Up Signal is only used in the cell in which the UE most recently entered RRC_IDLE triggered by:

-	reception of RRCEarlyDataComplete; or

-	reception of RRCConnectionRelease not including noLastCellUpdate; or

-	reception of RRCConnectionRelease including noLastCellUpdate and the UE was using GWUS in this cell prior to this RRC connection attempt.

When all of the following conditions are met then the UE shall monitor GWUS using the GWUS parameters provided in system information:

-	the UE is in RRC_IDLE;

-	the UE supports GWUS;

-	GWUS configuration (gwus-Config) is provided in system information;

-	groupAlternation is present in gwus-Config and UE supports GWUS with group resource alternation; or

-	groupAlternation is not present in gwus-Config.

A UE supporting GWUS can be configured to monitor a WUS group and a common WUS. Upon detecting either of them, UE shall monitor POs as defined in clause 7.4.

For NB-IoT, E-UTRAN may configure up to 2 WUS resources (numbered 0 and 1). The timeoffset, g0, from the end of WUS resource 0 to the start of corresponding PO is determined as defined in clause 7.4. When both wus-Config and gwus-Config are present, WUS resource 0 shares radio resources with wus-Config.The timeoffset from the end of WUS resource 1 to the start of corresponding PO is sum of the timeoffset g0 and the maximum WUS duration.

After the UE has determined the applicable gap between end of WUS resource and associated PO as specified in clause 7.4, UE selects the WUS group set for the corresponding gap as specified in clause 7.5.2. From the selected WUS group set, UE selects one WUS group as defined in clause 7.5.3. If groupAlternation is not present in gwus-Config, the UE monitors the selected WUS group with the corresponding timeoffset for each PO. If groupAlternation is present in gwus-Config and UE supports GWUS with group resource alternation, the UE determines the WUS group to monitor for each PO and the corresponding timeoffset as specified in clause 7.5.4.

For BL UEs and UEs in enhanced coverage, E-UTRAN may configure up to 4 WUS resources. The resource number, time and frequency location of these resources is determined as specified in clause 7.5.5.