3GPP TSG-RAN WG2 Meeting #112 Electronic	R2-2009775

Elbonia, 02 – 13 November 2020

Agenda item:	6.15

Source:	Nokia, Nokia Shanghai Bell

Title:	On increasing the number of CSI-RS resources for L3 mobility

WID/SID:	NR_CSIRS_L3meas - Release 16

Document for:	Discussion and Decision

# 1	Introduction

RAN WG4 has worked on the requirements concerning CSI-RS based mobility. As a result, it was agreed to send the LS [1] to RAN WG1 and WG2, asking to ensure the number of configurable CSI-RSs can be increased. This paper analyses RAN WG2 impacts of this RAN WG4 decisions.

# 2	Discussion

The support of CSI-RSs for L3 mobility (CSI-RS-Resource-Mobility) has been defined in Rel-15 and the following can be found in NR RRC [2], section 6.3.2:

CSI-RS-Resource-Mobility ::= SEQUENCE { csi-RS-Index CSI-RS-Index, slotConfig CHOICE { ms4 INTEGER (0..31), ms5 INTEGER (0..39), ms10 INTEGER (0..79), ms20 INTEGER (0..159), ms40 INTEGER (0..319) }, associatedSSB SEQUENCE { ssb-Index SSB-Index, isQuasiColocated BOOLEAN } OPTIONAL, -- Need R frequencyDomainAllocation CHOICE { row1 BIT STRING (SIZE (4)), row2 BIT STRING (SIZE (12)) }, firstOFDMSymbolInTimeDomain INTEGER (0..13), sequenceGenerationConfig INTEGER (0..1023), ... }

There can be up to maxNrofCSI-RS-ResourcesRRM of such CSI-RS-Resource-Mobility. maxNrofCSI-RS-ResourcesRRM = 96.

CSI-RS-CellMobility ::= SEQUENCE { cellId PhysCellId, csi-rs-MeasurementBW SEQUENCE { nrofPRBs ENUMERATED { size24, size48, size96, size192, size264}, startPRB INTEGER(0..2169) }, density ENUMERATED {d1,d3} OPTIONAL, -- Need R csi-rs-ResourceList-Mobility SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesRRM)) OF CSI-RS-Resource-Mobility

Such list of CSI-RS resources (i.e. csi-rs-ResourceList-Mobility) is present within the CSI-RS-CellMobility. There can be up to maxNrofCSI-RS-CellsRRM of CSI-RS-CellMobility, as per the following excerpt. maxNrofCSI-RS-CellsRRM = 96.

CSI-RS-ResourceConfigMobility ::= SEQUENCE { subcarrierSpacing SubcarrierSpacing, csi-RS-CellList-Mobility SEQUENCE (SIZE (1..maxNrofCSI-RS-CellsRRM)) OF CSI-RS-CellMobility, ..., [[ refServCellIndex ServCellIndex OPTIONAL -- Need S ]] }

While CSI-RS-ResourceConfigMobility is included in the measurement object (MeasObjectNR).

In a nutshell, from RAN2 signalling point of view, there can be up to 96 cells in a measurement object and up to 96 CSI-RS resources per each cell. In total, 9216 CSI-RSs for mobility. There is a single reference frequency (ARFCN) per measurement object for CSI-RSs, so all CSI-RSs within a single measurement object are assumed to be for the same (reference) frequency carrier.

Observation 1: As per RAN2 signalling, a single NR measurement object can comprise up to 9216 CSI-RSs (up to 96 CSI-RSs per cell and up to 96 different cells within MO).

However, when reading RAN1 specification (TS 38.214, section 5.1.6.1.3, [3]) one can find the following restriction:

A UE configured with the higher layer parameters CSI-RS-Resource-Mobility may expect to be configured - with no more than 96 CSI-RS resources per higher layer parameter MeasObjectNR when all CSI-RS resources configured by the same higher layer parameter MeasObjectNR have been configured with associatedSSB, or, - with no more than 64 CSI-RS resources per higher layer parameter MeasObjectNR when all CSI-RS resources have been configured without associatedSSB or when only some of the CSI-RS resources have been configured with associatedSSB by the same higher layer parameter MeasObjectNR - For frequency range 1 the associatedSSB is optionally present for each CSI-RS resource - For frequency range 2 the associatedSSB is either present for all configured CSI-RS resources or not present for any configured CSI-RS resource per higher layer parameter MeasObjectNR.

So practically, when RAN1 and RAN2 specifications are analysed jointly, the UE can be configured with just up to 96 CSI-RSs in total, per measurement object. This is for the case when associatedSSB is configured as well, otherwise this maximum number is reduced to 64 per MO.

Observation 2: When RAN1 (TS 38.214) restriction is considered, a UE can be configured with up to 96 or 64 CSI-RSs per MO for the case with and without associatedSSB, correspondingly.

Measurement object is equivalent to a frequency layer in RAN1’s understanding. However, in RAN2 specification there seems to be nothing that restricts a single MO per frequency layer, so the UE can be configured with more than a single MO for the same frequency layer if the MO is provided for CSI-RS measurements (comprises CSI-RS-ResourceConfigMobility).

Observation 3: As per RAN2 specification, the UE can be configured with more than one measurement object for the same frequency carrier, if the MO is for CSI-RS measurements.

However, even in RAN2 specification there could be still an issue due to the RAN4 decisions and the associated LS [1], namely the UE capabilities. Currently in [2], within MeasAndMobParametersCommon there is a parameter defining how many CSI-RSs for RRM the UE may can support. It is designed as follows [2]:

maxNumberCSI-RS-RRM-RS-SINR             ENUMERATED {n4, n8, n16, n32, n64, n96} OPTIONAL

As per the definition in TS 38.306, this capability concerns the number of CSI-RSs per single slot. So if only the total number of CSI-RSs one UE can be configured with shall be increased in line with RAN4 request [1], a new UE capability for the number of CSI-RSs per slot may be left intact.

Proposal 1: The extension of maxNumberCSI-RS-RRM-RS-SINR is not defined, if the number of CSI-RSs per slot remains the same.

However, there could be a confusion if the maxNumberCSI-RS-RRM-RS-SINR is kept with the current values, while the UE could be actually configured with a different (larger) total number of CSI-RSs to be measured. Thus, this shall be captured somewhere – either as a new UE capability or via clarification in NR RRC specification [2].

Observation 4: The increase of the total number of configurable CSI-RSs per MO shall be reflected in NR RRC specification or subject to a new UE capability.

We believe that a clarification in the NR RRC specification [2] could be sufficient.

Proposal 2: RAN2 is asked to discuss how to capture the increase of CSI-RSs per MO in the RAN2 specifications. A CR in R2-2009777 [4] can be considered.

RAN1 and RAN4 should also be aware that current signalling in RAN2 still restricts the configuration to up to 96 CSI-RSs per cell in one MO. Thus, additional 96 CSI-RSs for that frequency layer would have to be linked with other intra-frequency cell(s).

Observation 5: RAN2 MO design allows to have up to 96 CSI-RSs per cell. The remaining up to 96 CSI-RSs would have to be linked to other intra-frequency cell(s).

These RAN2 specific aspects would have to be shared with other affected working groups, i.e. RAN1 and RAN4. Thus, a response LS in [5] is suggested for discussion and subsequent approval.

Proposal 3: RAN2 is asked to send a response LS to RAN4 and RAN1. Draft is available in R2-2009776 [5].

# 3	Conclusion

In this paper we have elaborated on the number of CSI-RSs per L3 mobility that may be configured per measurement object/frequency layer. The following observations and proposals have been made:

Observation 1: As per RAN2 signalling, a single NR measurement object can comprise up to 9216 CSI-RSs (up to 96 CSI-RSs per cell and up to 96 different cells within MO).

Observation 2: When RAN1 (TS 38.214) restriction is considered, a UE can be configured with up to 96 or 64 CSI-RSs per MO for the case with and without associatedSSB, correspondingly.

Observation 3: As per RAN2 specification, the UE can be configured with more than one measurement object for the same frequency carrier, if the MO is for CSI-RS measurements.

Proposal 1: The extension of maxNumberCSI-RS-RRM-RS-SINR is not defined, if the number of CSI-RSs per slot remains the same.

Observation 4: The increase of the total number of configurable CSI-RSs per MO shall be reflected in NR RRC specification or subject to a new UE capability.

Proposal 2: RAN2 is asked to discuss how to capture the increase of CSI-RSs per MO in the RAN2 specifications. A CR in R2-2009777 [4] can be considered.

Observation 5: RAN2 MO design allows to have up to 96 CSI-RSs per cell. The remaining up to 96 CSI-RSs would have to be linked to other intra-frequency cell(s).

Proposal 3: RAN2 is asked to send a response LS to RAN4 and RAN1. Draft is available in R2-2009776 [5].

# References

[1] R4-2012179 LS on number of configurable CSI-RS resources per MO 3GPP TSG-RAN WG4 Meeting#96-e Electronic Meeting, 17 – 28 August 2020

[2] 3GPP TS 38.331 NR Radio Resource Control (RRC) protocol specification, v16.2.0

[3] 3GPP TS 38.214 NR Physical layer procedures for data, (Release 16)

[4] R2-2009777 Draft CR to NR RRC on increasing the number of CSI-RSs 3GPP TSG-RAN WG2 Meeting #112 Elbonia, 02 – 13 November 2020

[5] R2-2009776 Draft response LS to RAN4 on increasing the number of CSI-RSs 3GPP TSG-RAN WG2 Meeting #112 Elbonia, 02 – 13 November 2020