3GPP TSG-RAN WG2 Meeting #112-e		R2-2009782

Online, 2nd – 13th November 2020

Agenda Item:	5.4.4

Source: 	MediaTek Inc.

Title:  	Clarifications for Inter-RAT Cell Reselection and Mobility State

Document for:	Discussion and decision

Introduction

In this contribution, we discuss an issue about whether inter-RAT cell reselections are counted for idle/inactive-mode UE to determine its mobility state.

Discussion

Problem description

Description in TS 38.304

In NR, an idle/inactive-mode UE determines its mobility state based on number of cell reselections, if corresponding parameters are broadcasted. This is described in TS 38.304, as captured below.

______________________________________________________________________________________

#### 5.2.4.3	Mobility states of a UE

##### 5.2.4.3.0	Introduction

The UE mobility state is determined if the parameters (TCRmax, NCR_H, NCR_M and TCRmaxHyst) are broadcasted in system information for the serving cell.

State detection criteria:

Normal-mobility state criteria:

-	If number of cell reselections during time period TCRmax is less than NCR_M.

Medium-mobility state criteria:

-	If number of cell reselections during time period TCRmax is greater than or equal to NCR_M but less than or equal to NCR_H.

High-mobility state criteria:

-	If number of cell reselections during time period TCRmax is greater than NCR_H.

The UE shall not consider consecutive reselections where a cell is reselected again right after one reselection for mobility state detection criteria.

State transitions:

The UE shall:

-	if the criteria for High-mobility state is detected:

-	enter High-mobility state.

-	else if the criteria for Medium-mobility state is detected:

-	enter Medium-mobility state.

-	else if criteria for either Medium- or High-mobility state is not detected during time period TCRmaxHyst:

-	enter Normal-mobility state.

If the UE is in High- or Medium-mobility state, the UE shall apply the speed dependent scaling rules as defined in clause 5.2.4.3.1.

______________________________________________________________________________________

For a multi-RAT UE, cell reselection may be intra-RAT (from one NR cell to another NR cell) or inter-RAT (e.g. from NR to LTE). The text in TS 38.304 does not specify explicitly whether inter-RAT cell reselections should be counted when determining UE mobility state based on the number of cell reselections within a given duration.

Observation 1:	The text in idle/inactive-mode procedure specification (TS 38.304) does not specify explicitly whether inter-RAT cell reselections should be counted when determining UE mobility state based on the number of cell reselections within a given duration.

Test case in TS 38.523

In UE conformance specification (TS 38.523[2]), we have the following test case related to inter-RAT

______________________________________________________________________________________

6.2.3.9.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state and in high mobility state }

ensure that {

when { UE detects higher priority E-UTRA cell as candidate for reselection }

then { UE reselects to the new cell taking into account the scaling factor }

}

…

Table 6.2.3.9.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | E-UTRA Cell 1 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | - | - | -98 | The power level values are assigned to ensure the UE registered on NR Cell 2. |
|  | SS/PBCH SSS EPRE | dBm/SCS | -98 | -84 | - |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | - | - | -84 | E-UTRA Cell 1 becomes the highest ranked cell. |
|  | SS/PBCH SSS EPRE | dBm/SCS | -115 | -106 | - |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | - | - | -98 | NR Cell 1 becomes the highest ranked cell. |
|  | SS/PBCH SSS EPRE | dBm/SCS | -84 | -98 | - |  |
| T4 | Cell-specific RS EPRE | dBm/15kHz | - | - | -84 | E-UTRA Cell 1 becomes the highest ranked cell. |
|  | SS/PBCH SSS EPRE | dBm/SCS | -115 | -106 | - |  |
| Note 1: Power level “Off” of NR cell is defined in TS 38.508-1 [4] Table 6.2.2.1-3. Note 2: Power level “Off” of E-UTRA cell is defined in TS 36.508 [7] Table 6.2.2.1-3. |  |  |  |  |  |  |

…

Table 6.2.3.9.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-2 | Void | - | - | - | - |
| 3 | The SS changes the power level setting according to the row "T2" in table 6.2.3.9.3.2-1/2. | - | - | - | - |
| 4 | Wait for 40s for the UE to perform cell reselection procedure. (Note 1, Note 3) | - | - | - | - |
| 5 | The SS changes the power level setting according to the row "T3" in table 6.2.3.9.3.2-1/2. | - | - | - | - |
| 6 | Wait for 41s for FR1 or 137s for FR2 for the UE to perform cell reselection procedure. (Note 2, Note 4) | - | - | - | - |
| 7 | The SS changes the TAC of NR Cell 2 to 2 according to table 6.2.3.9.3.3-2 and waits for 6 s to ensure that SIB1 is modified. | - | - | - | - |
| 8 | The SS changes the power level setting according to the row "T1" in table 6.2.3.9.3.2-1/2. | - | - | - | - |
| 9-14 | Steps 1 to 6 of the mobility registration updating procedure described in TS 38.508-1 [4] Table 4.9.5.2.2-1 are performed on NR Cell 2. ( Note 5) | - | - | - | - |
| 15 | The SS changes the power level setting according to the row "T4" in table 6.2.3.9.3.2-1/2. | - | - | - | - |
| 16 | Check: Does the UE transmit an RRCConnectionRequest message on E-UTRA cell 1 within 2s? ( Note 6) | --> | E-UTRAN RRC: RRCConnectionRequest | 1 | F |
| 17 | Check: Does the UE send an RRCConnectionRequest on E-UTRA Cell 1 within 33s for after step 16? (Note 7) | --> | E-UTRAN RRC: RRCConnectionRequest | 1 | P |
| 18-22 | Steps 2 to 6 of the generic test procedure in TS 38.508-1 [4] Table 4.9.7.2.2-1 are performed on E-UTRA cell 1. | - | - | - | - |
| Note 1: NCR (number of cell reselection) is 1. Note 2: NCR is 2. Note 3: The wait time for reselection to a newly detected Inter-RAT E-UTRA cell is selected to cover Tdetect, EUTRAN (32s) + Treselection (7s) + 640 ms system information blocks scheduling = 39.64s rounded up to 40s. Note 4: The wait time for reselection to an newly detected Inter-RAT NR cell is selected to cover Tdetect,NR (32s for FR1 and 4*32s=128s for FR2) + Treselection (7s) + 1280 ms system information block type scheduling = 40.28s rounded up to 41s for FR1 and 136.28s rounded up to 137s for FR2. Note 5: NCR is 3 and the UE enters high mobility state because number of cell reselections during time period TCRmax is greater than NCR_H. Note 6: The wait time for evaluating an inter-RAT cell is Treselection after scaling (2s). Note 7: The wait time for cell reselection procedure is selected to cover Tdetect, EUTRAN (32s) + Treselection after scaling (7s*0.25) + 640 ms system information blocks scheduling = 34.39s rounded up to 35s. The actual wait time is 33s which covers 35s - 2s (wait time in step 16). |  |  |  |  |  |

______________________________________________________________________________________

In the above test case, UE is initially registered on a NR Cell. NCR is increased whenever UE performs cell reselection from NR to LTE or from LTE to NR. That is, inter-RAT cell reselections are counted in the procedure. UE enters high mobility state once the number of cell reselections during time period TCRmax is greater than NCR_H.

Observation 2:	In UE conformance specification (TS 38.523), inter-RAT cell reselections are counted when determining UE mobility state based on the number of cell reselections within a given duration.

Our view

Generally speaking, the scope of a specification is limited its own RAT. For example, we have the following NOTE in clause 1 (Scope) of TS 38.304 [1].

NOTE: When the UE is camped on or searching for a cell to camp on belonging to other RATs, the UE behaviour is described in the specifications of the other RATs.

From network deployment perspective, intra-RAT cells are geographically separated with possibly small overlaps, while the cell coverages of different RATs are likely to overlap with each other, or even the base stations are co-located. The reason why the number of cell reselections can be used to determine UE mobility is that UE can estimate the distance it has moved during a given duration based on the number of geographically separated cells it has crossed, with the assumption that average distance UE travels in a cell is similar for different deployments. Counting inter-RAT cell reselection may result in a much higher estimation of UE mobility and makes the mobility state determination more imprecise.

Therefore, we believe that inter-RAT cell reselections should NOT be counted when determining UE mobility state based on the number of cell reselections within a given duration. If RAN2 agrees to this view, we also need to discuss how to clarify this in 3GPP specifications, e.g., modification of TS 38.304 and/or LS to other WGs.

Proposal 1:	Inter-RAT cell reselections should NOT be counted when determining UE mobility state based on the number of cell reselections within a given duration.

Proposal 2:	If Proposal 1 is agreed, we discuss how to clarify this in 3GPP specifications.

Conclusion

This contribution discusses whether inter-RAT cell reselections are counted for idle/inactive-mode UE to determine its mobility state. We have the following observation:

Observation 1:	The text in idle/inactive-mode procedure specification (TS 38.304) does not specify explicitly whether inter-RAT cell reselections should be counted when determining UE mobility state based on the number of cell reselections within a given duration.

Observation 2:	In UE conformance specification (TS 38.523), inter-RAT cell reselections are counted when determining UE mobility state based on the number of cell reselections within a given duration.

It is proposed to discuss and decide on the following proposals:

Proposal 1:	Inter-RAT cell reselections should NOT be counted when determining UE mobility state based on the number of cell reselections within a given duration.

Proposal 2:	If Proposal 1 is agreed, we discuss how to clarify this in 3GPP specifications.

Reference

- 3GPP TS 38.304 V16.2.0, September 2020

- 3GPP TS 38.523-1 V16.5.0, September 2020