3GPP TSG-RAN WG2 Meeting #112-e		R2-2009786

Online, 2nd – 13th November 2020

Agenda Item:	8.3.2

Source: 	MediaTek Inc.

Title:  	Support for Multi-SIM Devices – Paging Collision

Document for:	Discussion and decision

Introduction

A new work item about “Support for Multi-SIM devices in Rel-17” has been agreed in RP#86 [1]. In this contribution, we focus on the following objective about paging collision avoidance:

- Specify, if necessary, enhancement(s) to address the collision due to reception of paging when the UE is in IDLE/INACTIVE mode in both the networks associated with respective SIMs [RAN2] - RAT Concurrency: Network A can be NR. Network B can either be LTE or NR. - Applicable UE architecture: Single-Rx/Single-Tx.

Many solutions for supporting multi-SIM devices involve NAS procedures. SA2 already had some studies and their proposed solutions are captured in TR 23.761 [2]. Here we discuss selected solutions from RAN perspective.

Discussion

PF/PO calculation

The description in TS 38.304 about how to determine paging frame (PF) and paging occasion (PO) is copied below for reference.

TS 38.304 (Subclause 7.1) The PF and PO for paging are determined by the following formulae: SFN for the PF is determined by: (SFN + PF_offset) mod T = (T div N)*(UE_ID mod N) Index (i_s), indicating the index of the PO is determined by: i_s = floor (UE_ID/N) mod Ns The PDCCH monitoring occasions for paging are determined according to pagingSearchSpace as specified in TS 38.213 [4] and firstPDCCH-MonitoringOccasionOfPO if configured as specified in TS 38.331 [3]. When SearchSpaceId = 0 is configured for pagingSearchSpace, the PDCCH monitoring occasions for paging are same as for RMSI as defined in clause 13 in TS 38.213 [4]. When SearchSpaceId = 0 is configured for pagingSearchSpace, Ns is either 1 or 2. For Ns = 1, there is only one PO which starts from the first PDCCH monitoring occasion for paging in the PF. For Ns = 2, PO is either in the first half frame (i_s = 0) or the second half frame (i_s = 1) of the PF.

Potential Solutions

Shifted or additional PO (AS solution)

To avoid PO collision, an intuitive way is to shift the PF/PO of one network by applying an additional offset. We may also keep the original PO, and let the network transmit paging message for MUSIM UE in both the original PO and an additional (shifted) PO. Both methods are at AS level and involve paging formula modification. Modifying paging formula means that each gNB in the tracking area needs to know that there is such a “special UE”, and thus can send its paging message on a shifted or additional PO. This is a quite complicated procedure for the network. Thus, for paging collision avoidance, we suggest not considering AS solutions involving shifted or additional PO.

Proposal 1:	For paging collision avoidance, we do not consider AS solutions involving shifted or additional PO.

Re-assigned 5G-GUTI (NAS solution)

To shift PF/PO without modifying paging formula, UE may use another UE_ID. Since UE_ID = 5G-S-TMSI mod 1024, and 5G-S-TMSI is a part of 5G-GUTI assigned by AMF, changing UE_ID may be done by re-assigning 5G-GUTI. The Solution #20 in TR 23.761[2] proposes that the UE triggers a Mobility Registration Update (MRU) when a collision risk is detected by the UE, which prompts the AMF to assign a new 5G-GUTI to the UE. The procedure is shown in the figure below (Figure 6.20.3-1 in TR 23.761).

Figure 1. MRU trigger upon PO collision risk detection

This solution does not introduce new procedures, and is thus our preferred solution for paging collision avoidance. There are some details to be further studied, for example, whether a specific indication need to be provided by the UE in the Registration Request message or an existing indication can be reused. However, these details should be discussed in other WGs, and we expect no RAN impacts.

Proposal 2:	For paging collision avoidance, we may consider a NAS solution in which UE requests a new 5G-GUTI from the network.

Proposal 3:	The details for re-assigning 5G-GUTI for paging collision avoidance should be discussed in other WGs, and we expect no RAN impacts.

Alternative UE_ID (Cross NAS-AS solution)

Another solution for paging collision avoidance is to use Alternative UE_ID in paging procedure (Solution#15 in TR 23.761 [2]). Here is the summary of this solution:

- UE indicates an “Alternative UE_ID” via NAS signalling (e.g., Registration Request)

- RAN uses the Alternative UE_ID to determine the next PF/PO and pages the UE.

  - RAN does not derive the UE_ID from the IMSI or 5G-S-TMSI.

This is a cross NAS-AS solution. The Alternative UE_ID introduces unnecessary complexity while the benefit is similar to the NAS solution which changes 5G-S-TMSI. Also, the PO formula is somehow changed, i.e., UE_ID is not derived from IMSI/5G-S-TMSI, but from Alternative UE_ID. Therefore, we do not prefer the Alternative UE_ID solution.

Proposal 4:	For paging collision avoidance, we do not consider the solution of introducing an Alternative UE_ID which replaces the UE_ID derived from IMSI/5G-S-TMSI for PF/PO calculation.

UE implementation

Although we are discussing multi-SIM support in Rel-17, multi-SIM devices are already available. Paging collision may actually be resolved with UE implementation. For example, UE with PO collision may monitor POs of the two networks alternately.

If PO collision can be resolved by UE implantation, and the latency due to reduced potentially reduced paging monitoring is acceptable, the UE may not want to report paging collision. Therefore, we suggest leaving it for UE implementation whether to report paging collision.

Proposal 5:	It is up to UE implementation whether to report paging collision.

Conclusion

It is proposed to discuss and decide on the following proposals:

Proposal 1:	For paging collision avoidance, we do not consider AS solutions involving shifted or additional PO.

Proposal 2:	For paging collision avoidance, we may consider a NAS solution in which UE requests a new 5G-GUTI from the network.

Proposal 3:	The details for re-assigning 5G-GUTI for paging collision avoidance should be discussed in other WGs, and we expect no RAN impacts.

Proposal 4:	For paging collision avoidance, we do not consider the solution of introducing an Alternative UE_ID which replaces the UE_ID derived from IMSI/5G-S-TMSI for POF/PO calculation.

Proposal 5:	It is up to UE implementation whether to report paging collision.

Reference

- RP-193263, New WID: Support for Multi-SIM devices in Rel-17, vivo, China Telecom, China Unicom, Dec. 2019

- 3GPP TR 23.761, Study on system enablers for devices having multiple Universal Subscriber Identity Modules (USIM)