3GPP TSG-RAN WG2 Meeting #112 electronic	R2-2009821

Online, 2nd-13th November, 2020

Agenda Item:	8.10.3.3

Source:	Ericsson

Title:	Connected mode aspects for NTN

Document for:	Discussion, Decision

# 1	Introduction

A new WI on solutions for NR to support non-terrestrial networks (NTN) was approved at RAN#86, with an updated WID approved at RAN#88 [1]. The WI aims to specify the following control plane enhancements:

- Idle mode:

  - Definition of additional assistance information for cell selection/reselection (e.g. using UE location information, satellite Ephemeris information)

  - Definition of NTN (satellite/HAPS) cell specific information in SIB

- Connected mode

  - Enhancement necessary to take into account location information (UE & Satellite/HAPS) and/or ephemeris in determining when to perform hand-over, in order to have a high degree of hand-over control for hand-over robustness and coverage management.

  - Enhancement to existing measurement configurations to address absolute propagation delay difference between satellites (e.g. SMTC measurement gap adaptation to the SSB/CSI-RS measurement window) [RAN2/4].

- Service continuity for mobility from TN to NTN and from NTN to TN systems (to be addressed when connected mode mobility has sufficiently progressed)

- Identify potential issues associated to the use of the existing Location Services (LCS) application protocols to locate UE in the context of NTN and specify adaptations if any [RAN2/3]

In the email discussions [910] and [911] the following scenarios were categorized for NTN:

- Scenario 1: Feeder link switch for earth fixed beam, with/without service link switch due to satellite switch

- Scenario 2: Feeder link switch for earth moving beam, with/without service link switch due to satellite switch

- Scenario 3: Service link switch for earth fixed beam due to satellite switch

- Scenario 4: Connected mode mobility for earth moving beam when the beam no longer serves the UE

- Scenario 5: Connected mode mobility for both earth moving and earth fixed beam due to UE movement

In this contribution, we provide our view on connected mode aspects for NTN with respect to Scenarios 4 and 5.

# 2 Connected-state mobility

In connected state, a UE has a connection established to the network. The aim of connected-state mobility is to ensure that the connectivity is retained with no interruption or noticeable degradation as the device moves within the network. The UE is required to do searches for new cells both on the current carrier frequency (intra-frequency) and different carrier frequencies (inter-frequency) that are informed by the network. The UE does not make any decisions on its own regarding when it is time to trigger a handover procedure to a different cell. In general, the UE reports the results of any configured measurements to the network so that the network can make a decision on whether or not it is time for handover to a new cell.

In 5G NR, handover is a special case of a procedure called “reconfiguration with sync”. In addition, a variety of handover mechanisms, such as Dual Active Protocol Stack (DAPS) handover and conditional handover (CHO), have been introduced in specifications to enhance the mobility performance for challenging scenarios that require short interruption time, low-latency and high reliability performance. There has also been discussion to introduce RACH-less HO in NR however due to lack of time it has been postponed thus so far it is supported only in LTE.

2.1 Conditional Handover (CHO)

The main motivation of this handover mechanism is to reduce the number of failure occurrences while a UE is moving, e.g., when a handover between cells fails, or when a connection fails even before a handover (HO) is triggered. In conditional handover, instead of preparing one target cell as in the legacy case, multiple candidate target cells can be prepared in advance in the network. Further, the handover command is sent to the UE earlier than at legacy handover when the radio conditions are still good, rather than when conditions start to get degraded. When received, the UE stores the command, instead of applying it immediately. The UE only applies the stored command when a condition configured by the network is satisfied for one of the configured candidate target cells. Then the UE executes the handover and connects to the target node as in legacy handover.

When the UE executes the handover in one of the prepared cells, the gNB hosting this cell can inform the source cell that the UE successfully performed the handover in its cell, so that the source cell can cancel the resources reserved by the remaining target candidate cells. Considering that the time between the handover preparation, and thus resource reservation, is not known, the source cell can release such resources by informing target candidate cells before the UE executes the handover.

The network may support early or late data forwarding on the user plane depending on performance requirements of that particular connection. In early data forwarding, data is forwarded during the preparation phase and the main benefit is to enable similar interruption performance as legacy, while increasing robustness. However, the complexity increases with the number of target cell candidates and the time it takes until the handover is actually performed. Late data forwarding is a simpler alternative, when data starts to be forwarded by the source cell when the UE accesses the target cell. The benefit is that the serving cell only forwards data to a single neighbour cell, i.e., target cell, even if multiple cells have been prepared, and forwarding only starts after the UE accesses the target cell once the condition is fulfilled.

It is also possible that a failure is detected while the UE is monitoring the configured conditions. In legacy, the UE would perform cell selection and continue with a re-establishment procedure. However, with conditional handover, when the same type of failure  is detected, e.g. a radio link failure or handover failure, the UE can prioritize a cell for which it has a stored handover command and, instead of performing re-establishment, it performs a conditional handover, which reduces the interruption time and the signalling over air interface.

Figure 6: Simplified message diagram for an inter-gNB Conditional Handover. The RRCReconfiguration* indicated with an asterisk (‘*’) is the Handover Command containing the RRC reconfiguration the UE shall apply if/when connecting to the candidate target gNB.

The following NTN specific execution conditions for CHO has been studied in the SI phase.

- Location (UE and Satellite) triggering: additional triggering conditions based on UE and satellite location can be considered in NTN and may be considered independently or jointly with another trigger (e.g. measurement based). Location-based conditional HO in LEO scenarios should consider deterministic satellite movement. For example, the location triggering condition may be expressed as distance between the UE and the satellite.

- Time(r)-based triggering: Several triggering conditions considering the time a region is served can be considered. This may be based on UTC time, or a timer-based solution, and may be considered independently or jointly with another trigger (e.g. measurement based). Time-based conditional HO in LEO scenarios should consider deterministic satellite movement.

- Timing advance value based triggering: additional triggering conditions based on timing advance value to the target cell can be considered in NTN and may be considered independently or jointly with another trigger.

- Elevation angles of source and target cells based triggering: additional triggering conditions based on elevation angles of source and target cells can be considered in NTN and may be considered independently or jointly with another trigger.

In the Connected mode email discussion the following conclusion is reached

In the Connnected mode email discussion [911], the following has been conlcuded where the fisrt three proposals are marked ”to be agreed” and the last one ”to be discussed”

Proposal 2.1: The CHO can be used in NTN for both moving cell and fixed cell scenarios, and the CHO procedure and execution condition defined in Rel-16 shall be considered as baseline.

Proposal 2.2: NTN specific CHO execution condition should be introduced.

Proposal 2.3a: location based CHO execution condition should be introduce for both moving cell and fixed cell scenario.

Proposal 2.3b: Timer based CHO execution condition should be introduced for moving cell scenario.

In [4], we discuss the CHO from the perspective of Scenarios 1 and 3. For Scenario 4 and 5 we agree with the proposals of email discussion [911], however, it is too preliminary to exclude e.g. the timing advance based trigger. For sending a preamble, UE needs to have some timing advance deduced in order the network to be able to receive the preambles within the reception window. For this reason, we think the timing advance based trigger is important for CHO. More specifically, timing advance, RTT and the elevation angle are all related. What exactly is used as trigger can be discussed further. Thus, we call it here “timing advance based” trigger. It can also be discussed further whether it is needed as actual trigger in CHO command or if that is needed as a more general prerequisite for sending a preamble.

For NTN, the It is too preliminary to exclude e.g. the “timing advance based” trigger as for sending a preamble, UE needs to have some timing advance deduced

RAN2 to discuss whether it is needed as actual trigger in CHO command.

2.2 Dual Active Protocol Stack (DAPS) handover

DAPS handover was introduced in Rel-16 as a mean to reduce handover interruption time, especially for new type of URLLC services within factory automation and transport industry requiring high reliability, low latency and extremely short interruption times close to 0ms.

In a “normal” handover, the UE typically releases the connection to the source cell before the link to the target cell is established, i.e., uplink and downlink transmission is finalized in the source cell before the UE starts to communicate with the target cell. This evidently causes an interruption in the range of a few tens of milliseconds in a terrestrial network. However, in an NTN network, the interruption time would be even larger due to longer RTT.

To improve this, a DAPS handover concept was specified in Rel-16 (for both 4G and 5G) where the UE is able to keep the connection to the source while performing the connection to the target cell. The main characteristics of the DAPS handover mechanism are:

- Continued transmission/reception of user data in the source cell after transmitting/receiving the handover request indicating DAPS handover

- Simultaneous reception of user data from both source and target cells at completion of random access procedure in the target cell

- Uplink transmission of user data switched from source cell to target cell at completion of random access procedure

- Release of source cell connection at reception of release indicator from target cell

Figure 3: An overview of Dual Active Protocol Stack (DAPS) handover

Upon receiving the request to perform a DAPS handover, the UE continues to send and receive user data in the source cell while a new connection to the target cell is established. The UE establishes a new user plane protocol stack for the target cell, containing PHY (Physical), MAC (Medium Access Control) and RLC (Radio Link Control) layers, while keeping the source user plane protocol stack active for transmission and reception of user data in the source cell.

Since the UE may receive user data simultaneously from both source and target cells, PDCP layer is reconfigured to a common PDCP entity for the source and target user plane protocol stacks. To secure in-sequence delivery of user data, PDCP Sequence Number (SN) continuation is maintained throughout the handover procedure. For an inter-node DAPS handover, it implies forwarding of  SDUs on the Xn interface according to a new “Early data forwarding” concept. The new concept, which is a key function for reducing the handover interruption time, enables data forwarding of DL SDUs to the target gNB before transmission to the UE is stopped in the source cell.

For a terrestrial 4G or 5G network, the delay caused by the transmission on the Xn interface is not considered as an issue, however for NTN the usefulness of DAPS handover will very much depend on the Xn interface delay and the distance between the two GWs(gNB).

For NTN, the usefulness of inter-node DAPS handover depends on the Xn interface delay and the distance between the source and target GWs/gNBs.

In NTN, the long propagation delay on the radio interface between the UE and the gNB, will also have a bad impact to the service interruption time when performing handover between two NTN cells or between NTN and a cell in the terrestrial network.

For LEO-based cells, the propagation delay on the radio interface can be significant longer compared to a terrestrial mobile communications system. For a LEO satellite with transparent payload, the round trip delay may vary between 25.77 and 41.77ms depending on satellite altitude. These figures can be compared to a terrestrial mobile communications system where the delay between the UE and its serving base station is typically within 1ms.

Considering the size and the fixed position of the fixed beam LEO cell, handover will typically only be triggered when the UE enters or leaves the coverage area of the LEO cell. Note that the discussion for Scenario 1 and 3 is in another paper in [4]. The size of the cell and the very small RSRP difference between the centre and the edge of the fixed beam LEO cell, also implies that many UEs will typically be considered as stationary within the cell. For these types of UEs, handover to a neighbor cell will only be triggered in the rare case of temporary coverage loss in the LEO cell.

Due to the expected low handover rate to/from Earth fixed LEO cells while service link switch does not happen, handover scenarios that involves this type of cells may not be of highest priority when to optimize for DAPS handover.

Handover rate to and from a Earth fixed LEO cell is expected to be low or even non-existent for some UEs when service link switch does not happen

Adopting DAPS handover for handover scenarios that involves Earth fixed LEO cells is therefore questionable

The beam footprints generated by a LEO satellite are either fixed relative to the surface of earth, or in motion with the satellite orbit. With the capability to steer beams towards fixed points on earth using beamforming techniques, the beam/cell can be earth fixed for a period of time corresponding to the visibility time of the satellite.

Satellites in LEO orbits move with high speed relative to a fixed position on earth, leading to frequent and unavoidable handovers for both stationary and moving UEs. TR 38.821 shows that a UE will be forced to perform handover as often as every 7th second if served by a cell with 50 km diameter, while for a cell diameter size of 1000km handover will be triggered at approximately every 130 second. The TR also identifies small variation in time depending on UE speed.

It seems quite obvious that a combination of frequent handovers and thus reoccurring service interruptions (which also are affected by the long propagation delay) caused by the earth moving cells, will have a significant impact to end user experience for a UE performing handover between this type of cells.

In this type of handover scenarios, DAPS handover can be of interest to at least shorten the constantly reoccurring interruptions and by that improve end user experience.

Handover rate between earth moving cells can be significant high depending on NTN deployments

DAPS handover can improve end user experience in handover scenarios where frequent and unavoidable handovers are constantly triggered

At the RAN2#111 meeting, following agreement was reached for mobility between NTN and a cell in the terrestrial network:

“For TN/NTN mobility, the UE is not required to connect to both TN and NTN at the same time”

Since simultaneous connection to source and target cell is a key characteristic for DAPS handover, TN/NTN mobility can be ruled out as a scenario for DAPS handover, at least for Rel-17.

DAPS handover is not supported in TN/NTN mobility scenarios in Rel-17

To sum up, adopting DAPS handover for NTN handover scenarios is not obvious since the usefulness depends on many aspects such as propagation delays on radio and Xn interfaces, handover scenarios, expected handover rate etc. The benefits from DAPS handover also need to be balanced to the potential increased resource utilization in the network, especially if a large number of UEs are performing handover between two cells at the same time.

It is also not clear whether NTN in Rel-17 will need to support URLLC-alike services requiring short handover interruption time.

RAN2 should therefore further study the usefulness of supporting DAPS handover for NTN taking above issues into consideration.

In the Connected mode email discussion [911], the following has been conlcuded  with ”to be discussed”-label

Proposal 3.2a: DAPS HO for NTN is de-prioritized in this release.

Even we think there could be some use cases for DAPS HO we are ok to discuss DAPS for now as second priority.

RAN2 to discuss as second priority the need for DAPS handover in Rel-17, taking issues as propagation delay, NTN deployment scenarios and potential use cases requiring short interruption time, into account.

2.3 RACH-less Handover

RACH-less handover was introduced as part of the mobility enhancements in LTE Rel-14 and consists of that Msg1 and Msg2 are skipped in the target cell. The UE’s first transmission in the target cell is instead the message confirming the completion of the handover, i.e. RRCConnectionReconfigurationComplete in LTE and RRCReconfigurationComplete in NR, i.e. the RRC message which is normally included in Msg3 in a regular RACH-based handover.

UL transmission resources for UE’s first message in the target cell are allocated either through pre-allocated UL grants or UL grants dynamically allocated through PDCCH in the target cell without preceding scheduling request due to the missing random access procedure in the target cell. If pre-allocated UL grants are configured, these are periodic UL transmission resources allocated in the RRCConnectionReconfiguration or RRCReconfiguration message constituting the HO Command. The pre-allocated UL grants potentially achieves shorter interruption, since the PDCCH allocation adds some additional delay, but PDCCH allocation is more flexible.

Since the random access procedure, and therefore also the Random Access Response message is skipped in the target cell, the target gNB cannot determine a precise timing advance (TA) for the UE. Hence, RACH-less HO is limited to scenarios where the TA is known in advance. In practice, this means scenarios where the TA in the target cell is the same as in UE’s source cell or when the target cells is so small that the TA can be assumed to always be 0. Use of RACH-less HO with a TA based on UE estimates is considered for NTN.

In the Connnected mode email discussion [911], the following has been conlcuded with a ”to be discussed”-label:

Proposal 3.1: From RAN2’s perspective, RACH-less HO should be introduced in NTN. An LS should be sent to RAN1 to confirm the feasibility of RACH-less HO in NTN.

We support this proposal.

# 3 Measurement aspects

In the Connected mode email discussion [911], the following has been concluded with a ”to be agreed”-label:

Proposal 4: The existing measurement framework (e.g. measurement configuration, execution and reporting) shall be considered as a baseline, and all the existing measurement criteria and event can be used in NTN.

Proposal 5.1: Permission from UE is needed for the network to collect the UE location information for the purpose other than SON/MDT. If the UE location information is collected for other purpose, the UE consent for SON/MDT cannot be reused and a similar but independent procedure for permission should be considered.

Proposal 6.1: The Location-based measurement event should be supported in NTN for both moving cell and fixed cell scenario.

Proposal 7.1: Some enhancement is needed in SMTC and measurement gap configuration, taken into consideration the propagation delay difference in NTN.

Proposal 7.2: SSB period other than 5ms shall be supported in NTN.

In the Connected mode email discussion [911], the following has been concluded with a ”to be discussed”-label:

Proposal 5.2: The location information report should be supported in NTN for the purpose other than SON/MDT.

Proposal 6.2a: For moving cell scenario, a relative area scope expressed as the distance between UE and satellite or cell center will be configured and measurement report will be triggered when UE moves out of or moves in the area scope configured.

Proposal 6.2b: For fixed cell scenario, an absolute area scope will be configured and measurement report will be triggered when UE moves out of or moves in the area scope configured.

In LTE, there is location reporting from the UE in Aerial context. It can be configured I the H1 and H2 events which trigger the measurement result when the UE crosses certain Hight. While this is in LTE and not in NR currently, it is not clear why NR would require the permission for location reporting and LTE not. Thus, it seems more discussion is needed whether permission for location reporting is needed in NTN context.

There is location reporting from the UE in Aerial context in LTE and it is not clear why NR would require the permission for location reporting and LTE not.

Proposals 6.2a/b expresses one example of configuring the event entry condition. It is also possible to define the event entry condition with respect to the target cell such that event entry condition is fulfilled when UE steps in the area. Further, in addition to circle, especially for fixed beam case, an elliptic shape might describe the actual area better. The radius, or in case of elliptic shape one or both of the radius may change as the satellite moves/elevation angle changes.

It is also possible to define the event entry condition with respect to the target cell such that event entry condition is fulfilled when UE steps in the area.

In addition to circle, especially for fixed beam case, an elliptic shape might describe the actual area better.

RAN2 to discuss further the most suitable way of defining the entry condition for the event based measurement trigger.

In addition to event based trigger, the network may configure a UE to report its location to the network periodically. If this period is sparse enough it avoids UE to keep constant up to date of its location. Disadvantage is that it consumes uplink resources. Regardless, it provides an option for the UE capability with respect to maintaining up to date location.

Periodic reporting of UE’s location may provide an option where UE does not need to maintain location info constantly.

RAN2 to discuss the feasibility of periodic location reporting as an addition to the event triggered based.

The SMTC configuration helps the UE to find the SSB but does not have implications to the network. Then, if the window size is extended it only means UE has less information from where to measure. Thus, this option does not seem worth the specification effort. We can discuss whether additional information is useful at the UE or whether UE would deduce itself when to measure. Then again, when measurement gaps are configured it means that during the gap period UE does not need to listen PDCCH needs to be considered. That is, gap window cannot be too long and network needs to be aware when UE is not monitoring PDCCH. There may also be need for the UE to inform network if current gap configuration is not enough.

For both SMTC and meas gap design extending the window length does not seem fruitful.

For SMTC, the mean consideration should be what information would be actually useful for the UE.

For measurement gap consideration, the total gap length should be minimized in order to minimize the time UE does not listen the PDCCH.

RAN2 should aim to minimize the total measurement gap length for NTN.

# Conclusion

In the previous sections we made the following observations:

Observation 1	For NTN, the It is too preliminary to exclude e.g. the “timing advance based” trigger as for sending a preamble, UE needs to have some timing advance deduced

Observation 2	For NTN, the usefulness of inter-node DAPS handover depends on the Xn interface delay and the distance between the source and target GWs/gNBs.

Observation 3	Handover rate to and from a Earth fixed LEO cell is expected to be low or even non-existent for some UEs when service link switch does not happen

Observation 4	Adopting DAPS handover for handover scenarios that involves Earth fixed LEO cells is therefore questionable

Observation 5	Handover rate between earth moving cells can be significant high depending on NTN deployments

Observation 6	DAPS handover can improve end user experience in handover scenarios where frequent and unavoidable handovers are constantly triggered

Observation 7	DAPS handover is not supported in TN/NTN mobility scenarios in Rel-17

Observation 8	There is location reporting from the UE in Aerial context in LTE and it is not clear why NR would require the permission for location reporting and LTE not.

Observation 9	It is also possible to define the event entry condition with respect to the target cell such that event entry condition is fulfilled when UE steps in the area.

Observation 10	In addition to circle, especially for fixed beam case, an elliptic shape might describe the actual area better.

Observation 11	Periodic reporting of UE’s location may provide an option where UE does not need to maintain location info constantly.

Observation 12	For both SMTC and meas gap design extending the window length does not seem fruitful.

Observation 13	For SMTC, the mean consideration should be what information would be actually useful for the UE.

Observation 14	For measurement gap consideration, the total gap length should be minimized in order to minimize the time UE does not listen the PDCCH.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to discuss whether it is needed as actual trigger in CHO command.

Proposal 2	RAN2 to discuss as second priority the need for DAPS handover in Rel-17, taking issues as propagation delay, NTN deployment scenarios and potential use cases requiring short interruption time, into account.

Proposal 3	RAN2 to discuss further the most suitable way of defining the entry condition for the event based measurement trigger.

Proposal 4	RAN2 to discuss the feasibility of periodic location reporting as an addition to the event triggered based.

Proposal 5	RAN2 should aim to minimize the total measurement gap length for NTN.

# References

RP-201256, “Solutions for NR to support non-terrestrial networks (NTN),” 3GPP TSG RAN #88e, June 2020.

TR 38.821, “Solutions for NR to support Non-Terrestrial Networks (NTN),” V16.0.0.

TR 38.811, “Study on New Radio (NR) to support non-terrestrial networks,” V15.3.0

R2-10xxxx, Earth fixed and moving beams for NTN, Ericsson

R2-2007537, “NTN scope, scenarios, architecture, and requirements,” 3GPP TSG RAN WG2 Meeting #111, Ericsson, August 2020.

Aida, S., Patzelt, T., Leushacke, L., Kirschner, M., and Kiehling, R., ‘Monitoring and Mitigation of Close Proximities in Low Earth Orbit’, 21st International Symposium on Space Flight Dynamics – 21st ISSFD, Toulouse, France, 2009.

R1-2005502, “On UL time and frequency synchronization enhancements for NTN,” 3GPP TSG RAN WG1 Meeting #102, Ericsson, August 2020.