3GPP TSG-RAN WG2 Meeting #112 electronic	R2-2009874

Online, November 2nd-13th, 2020

Agenda item:		8.6.5

Source:		Lenovo, Motorola Mobility

Title:		       Consideration on CG based small data transmission

Document for:		Discussion and Decision

- Introduction

By RP-201305 as in [1], the following content is proposed in the WI objective for CG based SDT:

- For the RRC_INACTIVE state:

  - Transmission of UL data on pre-configured PUSCH resources (i.e. reusing the configured grant type 1) – when TA is valid

    - General procedure for small data transmission over configured grant type 1 resources from INACTIVE state [RAN2]

    - Configuration of the configured grant type1 resources for small data transmission in UL for INACTIVE state [RAN2]

  - Specify RRM core requirements for small data transmission in RRC_INACTIVE, if needed [RAN4]

In RAN2#111e meeting, the following agreements on small data transmission were made as in [1]:

Agreements: - Small data transmission with RRC message is supported as baseline for RA-based and CG based schemes - RRC-less can be studied for limited use cases (e.g. same serving cell and/or for CG) with lower priority. - Context fetch and data forwarding with anchor re-location and without anchor re-location will be considered. FFS if there are problems with the scenario “without anchor relocation”. - From RAN2 perspective, stored “configuration” in the UE Context is used for the RLC bearer configuration for any SDT mechanism (RACH and CG). - The 2-step RACH or 4-step RACH should be applied to RACH based uplink small data transmission in RRC_INACTIVE - The uplink small data can be sent in MSGA of 2-step RACH or msg3 of 4-step RACH. - Small data transmission is configured by the network on a per DRB basis - Data volume threshold is used for the UE to decide whether to do SDT or not. FFS how we calculate data volume. - FFS if an “additional SDT specific” RSRP threshold is further used to determine whether the UE should do SDT. - UL/DL transmission following UL SDT without transitioning to RRC_CONNECTED is supported. - When UE is in RRC_INACTIVE, it should be possible to send multiple UL and DL packets as part of the same SDT mechanism and without transitioning to RRC_CONNECTED on dedicated grant. FFS on details and whether any indication to network is needed.

In this contribution, it mainly focuses on the CG based small data transmission, it includes the CG configuration, the validity of CG resource, the CG resource release, and the access control.

- Discussion

- 2.1 Procedure for CG based SDT

As in figure1. a general procedure for CG based SDT is shown in. UE will receive the CG configuration in RRC release when UE moves from connected mode to inactive mode. When UE will decide to use the configured grant type 1 resource to transmit the small data in inactive mode, UE will transmit the small data over CG resource if CG resource is valid, then UE will monitor the response message to the UL data transmission. Based on the response message, UE will transfer to legacy inactive mode or continue the subsequent UL/DL data transmission in inactive mode.

Figure.1 General procedure for CG based SDT

In order to make UE successfully transmit the UL data in SDT and avoid the complex procedure, the CG resource is desired to be UE specific, otherwise, UE needs to initiate the small data transmission again because of the collision from the contention based CG resource, it is power wasting for UE.

Proposal1: CG resource is UE specific in serving cell, no contention-based CG resource will be applied for small data transmission.

- 2.2 The details on CG based SDT

In this section, the details on above procedure for CG based SDT are given.

As in [1], multiple types of traffic could be applied to small data transmission, such as traffic from Instant Messaging services, Heart-beat/keep-alive traffic, traffic from wearables, sensor and so on. Those services have different traffic pattern and some of them could be applied in the same UE, so we should discuss the issue whether more than one CG configurations could be applied to inactive UE. Considering the possible multiple traffics in UE for small data transmission, the multiple CG resource configurations could be configured to UE for different service requirements, although it will introduce large resource overhead.

Proposal2: More than one CG configurations could be configured to UE for small data transmission considering multiple traffics are applied in UE inactive mode.

When UE transfers from connected mode to inactive mode, the CG configuration could be transmitted in RRC release message. If UE decides to initiate small data transmission over CG resource, it will send the UL data multiplexed with other information in CG resource, then UE will monitor the response message to UL data transmission. Since CG resource is UE specific, the CG configuration information should at least include type 1 CG resource, a X-RNTI for CG based small data transmission, and related information for PDCCH monitoring.

Proposal3: The CG configuration could be configured in RRC release information including the information of X-RNTI and PDCCH information for CG based small data transmission.

When UE decides to initiate small data transmission over CG resource, the TA should be valid for UE inactive mode, otherwise the network could not correctly receive the UE UL information. For TA validity, several methods shown in NB-IOT PUR could be discussed for CG based SDT, in NB-IOT PUR, one-shot small data is transmitted in preconfigured uplink resource in UE idle mode. For example, if TA timer is expired, the TA is invalid and UE could not apply the CG resource for SDT. Or, the changing of RSRP could be used to indicate the TA invalidity.

Proposal4: A TA timer or changing of RSRP could be considered to evaluate the TA validity for CG based SDT.

Besides TA validity, considering the CG configuration will configure the UE physical layer parameter such as MCS, hence, the RSRP for CG based SDT needs to be considered, otherwise the UE may fail to transmit the small data over CG resource with preconfigured physical layer parameters.

Proposal5: For CG validity, the TA validity and RSRP over a threshold needs to be considered when UE decides to apply the CG based small data transmission

Proposal6: The information on CG validity needs to be included in CG configuration in RRC release message.

Since radio resource is very precious, the CG resource needs to be released timely if it is not used by the UE. For example, UE could give CG resource release request to network if it moves out this serving cell or it decides not to use the CG resource for SDT, or the network needs to release the CG resource if it is not applied by the UE for a long time.

Proposal7: The CG resource release needs to be considered for CG based SDT.

Considering the UAC procedure, it should be applied for SDT and the existing UAC can be reused for the majority cases of SDT. However, for the CG based solution, there is some difference. The pre-configured CG resources can be considered as dedicated resources. And it has been agreed that small data transmission is configured by the network on a per DRB basis, which means there are allocated resources once a CG based SDT is initialized for a configured DRB. Then we can consider the access attempt as allowed for this case.

Proposal8: The access attempt is considered as allowed if the pre-configured CG resources are configured for SDT and a CG based SDT is initialized by the DRB which is configured to allow SDT.

The LCP and multiplexing of MAC CEs and MAC SDUs are introduced to guarantee the QoS and fairness among the UL traffics in LTE and NR. Although the UE is in RRC_INACTIVE state when performing SDT, the purpose is to transmit UL data which is the same with RRC_CONNECTED state. And no matter when state the UE is in, the requirements for the traffic performance are the same. Therefore, the existing procedure of LCP and multiplexing of MAC CEs and MAC SDUs when UE transmits UL data in RRC_CONNECTED state should be reused when performing SDT.

Proposal9: The legacy procedure of the LCP and the multiplexing of MAC CEs and MAC SDUs for UE in RRC_CONNECTED mode couuld be reused when performing SDT.

- Conclusion

In this contribution, the issues on the CG configuration, the validity of CG resource, the CG resource release and the access control are discussed, we have the following proposals:

Proposal1: CG resource is UE specific in serving cell, no contention-based CG resource will be applied for small data transmission.

Proposal2: More than one CG configurations could be configured to UE for small data transmission considering multiple traffics are applied in UE inactive mode.

Proposal3: The CG configuration could be configured in RRC release information including the information of X-RNTI and PDCCH information for CG based small data transmission.

Proposal4: A TA timer or changing of RSRP could be considered to evaluate the TA validity for CG based SDT.

Proposal5: For CG validity, the TA validity and RSRP over a threshold needs to be considered when UE decides to apply the CG based small data transmission

Proposal6: The information on CG validity needs to be included in CG configuration in RRC release message.

Proposal7: The CG resource release needs to be considered for CG based SDT.

Proposal8: The access attempt is considered as allowed if the pre-configured CG resources are configured for SDT and a CG based SDT is initialized by the DRB which is configured to allow SDT.

Proposal9: The legacy procedure of the LCP and the multiplexing of MAC CEs and MAC SDUs for UE in RRC_CONNECTED mode could be reused when performing SDT.

- References

RP-201305, “Revised WID on NR small data transmissions in INACTIVE state”, ZTE Corporation, RAN#88e, June 29 - July 3, 2020.

R2-2008124, “Report for Rel-16 (NR-U, Power Savings and 2-step RACH) and IIoT and Small Data”, InterDigital RAN2#111e, Aug. 17 – Aug. 28, 2020.