3GPP TSG-RAN2 Meeting #112-e	R2-2009944

eMeeting, 2nd – 13th November, 2020

Agenda Item:	5.4.3 UE capabilities and Capability Coordination

Source: 	Ericsson

Title:  	UE capability and cross-slot scheduling for Paging

Document for:	Discussion and Decision

# Introduction

In this contribution the possible K0 values, for cross-slot scheduling and Paging, that a Rel-15 UE is required to support is discussed further.

Cross slot scheduling for Paging is discussed in the REL-17 UE power saving work item [1]. To avoid any impact on legacy UE it is important to understand what K0 values are supported by the legacy UE.

# Background

RAN1 feature list in 38.822

During the Rel-15 time frame TR 38.822 was used to capture a snap-shot of the L1-UE feature lists. TR 38.822 is no longer maintained.

For Feature 5-1 Basic scheduling/HARQ operation it says that UE shall support K0 = 1 for Paging (Mandatory without capability signaling):

…

11) DL scheduling slot offset K0=1 for type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS

…

PDSCH monitoring in 38.214

For the PDSCH time domain resource allocation for Paging it says in Table 5.1.2.1.1-1:

| RNTI | PDCCH search space | SS/PBCH block and CORESET multiplexing pattern | pdsch-ConfigCommon includes pdsch-TimeDomainAllocationList | pdsch-Config includes pdsch-TimeDomainAllocationList | PDSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |
| P-RNTI | Type2 common | 1 | No | - | Default A |
|  |  | 2 | No | - | Default B |
|  |  | 3 | No | - | Default C |
|  |  | 1,2,3 | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-ConfigCommon |

The tables for default A and C include only K0 = 0. The table for default B includes both K0 = 0 and 1.

For FR1 only multiplexing pattern 1 is supported (default A). FR2 supports multiplexing pattern 2 (default B).

PDSCH configuration in 38.331

Up to 16 K0 values between 0-32 can be configured via pdsch-TimeDomainAllocationList in SIB1 for the initial BWP for Paging/SI/RA in PDSCH-ConfigCommon IE:

pdsch-TimeDomainAllocationList  PDSCH-TimeDomainResourceAllocationList  OPTIONAL,   -- Need R

PDSCH-TimeDomainResourceAllocationList ::=  SEQUENCE (SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation

PDSCH-TimeDomainResourceAllocation ::=   SEQUENCE {

k0                   INTEGER(0..32)                     OPTIONAL,   -- Need S

…

maxNrofDL-Allocations INTEGER ::= 16  -- Maximum number of PDSCH time domain resource allocations

UE capability in 38.306

There are two (IOT) UE capabilities defined in 38.306 (requirements 8-30/8-30a in 38.822):

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| dl-SchedulingOffset-PDSCH-TypeA Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type A. | UE | Yes | Yes | Yes |
| dl-SchedulingOffset-PDSCH-TypeB Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type B. | UE | Yes | Yes | Yes |

UE capability signalling in 38.331

The UE capabilities above are signalling the PHY capabilities:

Phy-ParametersXDD-Diff ::=          SEQUENCE {

…

dl-SchedulingOffset-PDSCH-TypeA     ENUMERATED {supported}                      OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB     ENUMERATED {supported}                      OPTIONAL,

But these capabilities have not been added to the UERadioPagingInformation IE, i.e. cannot be signalled in the Paging message from CN to RAN.

# Discussion

The UE may not have IOT tested K0>0 for DL PDSCH, and therefore set support for the mandatory capabilities dl-SchedulingOffset-PDSCH-TypeA or dl-SchedulingOffset-PDSCH-TypeB to false. UE logs also show that (some) Rel-15 UEs do that:

Observation 1: The UE is required to support K0>0 for DL PDSCH, but the UE may not have IOT tested this, and logs show that (some) REL-15 UEs do not support dl-SchedulingOffset-PDSCH-TypeA or dl-SchedulingOffset-PDSCH-TypeB.

However the UE is assumed to support the default configurations specified in 38.214, and default configuration B includes both K0 = 0 and 1:

Observation 2: Rel-15 supports the default configurations

Observation 3: Default PDSCH time domain resource allocation B for Paging and System Information includes both K0 = 0 and 1.

Therefore it should be assumed that the UE supports K0 = 0 and 1 for Paging and System Information, even when the UE does not indicate support for dl-SchedulingOffset-PDSCH-TypeA or dl-SchedulingOffset-PDSCH-TypeB:

Proposal 1: RAN2 to confirm that Rel-15 UE supports K0 = 0 and 1 for Paging and System Information.

In case proposal 1 is agreeable, it can be discussed further if a clarification is needed (e.g. clarify that the UE supports the default configuration independent from the IOT capability signalling).

In case proposal 1 is not agreeable, RAN2 should discuss if legacy UE supports K0 values in the pdsch-TimeDomainAllocationList provided in pdsch-ConfigCommon in SIB1 that have not been IOT tested by the UE, but the UE is only paged with K0=0 in the Paging PDCCH. This would enable the NW to use cross-slot scheduling for UEs that have indicated to support it, while using legacy scheduling for UEs that did not indicate support. But then dl-SchedulingOffset-PDSCH-TypeA and dl-SchedulingOffset-PDSCH-TypeB should be added to the UERadioPagingInformation message.

# Summary

RAN2 is kindly asked to discuss K0 values support by Rel-15 UE for Paging:

Observation 1: The UE is required to support K0>0 for DL PDSCH, but the UE may not have IOT tested this, and logs show that (some) REL-15 UEs do not support dl-SchedulingOffset-PDSCH-TypeA or dl-SchedulingOffset-PDSCH-TypeB.

Observation 2: Rel-15 supports the default configurations

Observation 3: Default PDSCH time domain resource allocation B for Paging and System Information includes both K0 = 0 and 1.

Proposal 1: RAN2 to confirm that Rel-15 UE supports K0 = 0 and 1 for Paging and System Information.

# References

- R2-2009955, Paging enhancement to reduce unnecessary UE paging receptions, Ericsson, DISC, RAN2#112-e