3GPP TSG-RAN WG2 Meeting #112 Electronic	R2-2010058

Elbonia, 02 – 13 November 2020

Agenda item:	8.15.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On configuration and operation of SL DRX

WID/SID:	NR_SL_enh-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

In order to reduce power/energy consumption, one objective of NR sidelink enhancement WI [1] is to design SL DRX for power/energy constraint SL UE.

3. Sidelink DRX for broadcast, groupcast, and unicast [RAN2] - Define on- and off-durations in sidelink and specify the corresponding UE procedure - Specify mechanism aiming to align sidelink DRX wake-up time among the UEs communicating with each other - Specify mechanism aiming to align sidelink DRX wake-up time with Uu DRX wake-up time in an in-coverage UE

In this contribution, we provide our view on SL DRX design.

# 2	Inter UE DRX coordination

SL DRX is also under study of SA2 as power efficient solution for pedestrian UEs. Several solutions have been contributed to SA2 [2]. This contribution discusses access stratum issues related to the solutions studied in SA2 on support of SL DRX .

According to Rel17 NR sidelink enhancement WID [1], SL DRX should be applied to support all the cast types: broadcast, groupcast and unicast. SL broadcast and groupcast communication is operated in connectionless manner, which means SL broadcast or groupcast transmitting (Tx) UE may not know or even care about the presence of any particular receiving (Rx) UE in proximity. On the other hand, one SL Rx UE may need to listen to multiple SL broadcast and/or groupcast communications in proximity without knowing SL Tx UEs beforehand. Thus, it makes SL DRX parameter configuration (such as the length of DRX cycle and ON/OFF period) between each pair of SL Tx and Rx UEs rather impractical for SL groupcast and broadcast.

For connection-based SL unicast communication, the DRX configuration parameters may be agreed and configured to a pair of SL Tx and Rx UEs beforehand during SL connection establishment procedure. However, if one or both of SL Tx or Rx UE has also other simultaneous SL broadcast/groupcast/unicast communication services in addition to the SL unicast communication in question, uncoordinated SL DRX configurations for each SL communication service may lead to a superposition of the SL DRX on-durations from the uncoordinated SL DRX configurations, such that the SL Rx UE needs to be awake and monitor PC5 during all the SL DRX on-durations of the uncoordinated SL DRX configurations. The mis-aligned SL DRX configuration among multiple SL communications of one SL Rx UE will make SL DRX less effective in term of power saving.

Observation 1: SL DRX configuration coordination among multiple SL communications in proximity is beneficial from power saving perspective.

Several solutions in SA2 TR23.776 [2] proposed to use the (pre-) configured/provisioned SL DRX configurations to facilitate the coordination of SL DRX configuration not only between a pair of SL Tx UE and Rx UEs for one SL application/service but also among a number of SL UEs involved in multiple SL communications in proximity. It also proposed in SA2 solutions that the information on the selected/applied SL DRX configuration of one SL UE may be provided to other SL UEs in proximity over PC5 interface.

Proposal 1: RAN2 to follow SA2s solutions as baseline for agreements on DRX alignment i.e. the (pre-) configured/provisioned SL DRX configurations should be considered by SL UEs to determine and agree on SL DRX parameters between relevant SL UEs in order to facilitate the coordination of SL DRX configuration among relevant SL UEs in proximity.

Proposal 2: The information of selected/applied SL DRX configuration should be provided to other relevant SL UEs in proximity over PC5 interface.

If Proposal 1 can be agreed, to avoid extensive coordination between relevant SL UEs in proximity to determine and agree on SL DRX parameters such as DRX cycle and length of ON/OFF period in the DRX cycle, a limited set of DRX configurations may be (pre-)configured/provisioned to the SL UEs so that SL UE only needs to select and coordinate the selection of DRX parameters from the limited set of DRX configurations. Each DRX configuration in the DRX configuration set may be associated with one or more SL services. Thus, one DRX configuration may fit the traffic pattern of combinations of multiple SL services for different use cases. The limited set of DRX configurations may be configured using broadcasted or dedicated RRC signalling in access stratum (AS). However, assistance information provided from upper layer may be considered in AS layer to determine the set of DRX configurations.

Proposal 3: RAN2 is kindly asked to discuss whether DRX configurations should be determined entirely by the AS layer or based on assistance information provided by the upper layers.

# 3	Sidelink DRX for mode 2

NR SL Mode 2 is specified for both in-coverage (IC) and out-of-coverage (OoC) operations of SL communications, also for operations in exceptional situations. Hence, support of SL DRX for SL UEs in Mode 2 needs to be considered.

Proposal 4: SL DRX is supported for SL UE in Mode 2.

To avoid unnecessary resource fragmentation resulted from introducing exclusive resource pool(s) for Mode 2 transmissions of SL UEs with SL DRX, it is expected that SL UEs with SL DRX may share same resource pool(s) with other SL UEs without SL DRX for Mode 2 transmissions.

Proposal 5: Support of SL DRX for SL UE in Mode 2 can be over a shared resource pool which can be used for Mode 2 transmissions of SL UEs with SL DRX and SL UEs without SL DRX.

The current resource allocation for Mode 2 over the common resource pool is based on continuous sensing. The current resource allocation for NR SL Mode 2 over a shared resource pool is based on continuous sensing. That is, SL UE needs to perform continuous monitoring and receiving of at least 1st-stage SCIs transmitted on PSCCHs of the shared resource pool. The sensing history within the sensing window (e.g., 100ms or 1100ms as specified in TS 38.331) is used for the resource selection at an individual SL UE for transmitting in Mode 2 on the shared resource pool. However, when SL DRX is enabled for a SL UE in Mode 2, the SL UE is not supposed to monitor and receive SL, including SCIs transmitted on PSCCHs, during the OFF period of a SL DRX cycle configured for the SL UE. This causes a disruption to the sensing of the SL UE. That is, the sensing history of the SL UE may not be adequate for the current sensing-based Mode 2 resource selection at least at the beginning of the ON period of the SL DRX cycle when UE is waking up from the OFF period of the SL DRX cycle. Therefore, the current sensing-based Mode 2 resource allocation cannot be used by the SL UE, at least at the start of the ON period if the SL UE has SL data to transmit over SL.

Proposal 6: RAN2 to send a LS to RAN1 asking RAN1 to study enhancements needed for Mode 2 in order to support SL DRX for SL UE in Mode 2 over a shared resource pool which can be used for Mode 2 transmissions of SL UEs with SL DRX and SL UEs without SL DRX.

# 4	DRX performance considerations

In the NR Uu interface during Rel.16, a PDCCH-based power saving signal was defined with the goal to instruct the UE in RRC Connected to wake up at the next DRX On-Duration. This signal was defined by RAN1 as “DCI with CRC scrambled by PS-RNTI”, also called DCP, where PS stands for Power Saving and the used DCI format is 2_6. When a UE does not receive the DCP during the network-defined WUS occasion(s) or a received DCP does not contain a wake-up indication, the UE will assume there is no data and can skip monitoring the PDCCH during the next DRX-On duration. Thus, saving power can be achieved when no data is present. To minimize false alarms (leading to waking up a UE unnecessarily), the DCP signal is targeted to a UE specific identifier, the PS-RNTI.

The Uu interface’s DCP concept can also be extended to SL DRX to permit additional power savings, although further study is required on how to achieve such a solution. Specifically, in the Uu interface it is the network that sends the DCP, in a SL context the SL-DCP could in principle be sent from both the network and SL peers.

Observation 2: In NR Rel-16 Uu has specified a wake up mechanism as to avoid unnecessary DRX_ON period, when there is no traffic.

Proposal 7: RAN2 to study how to avoid unnecessary DRX_ON periods within SL DRX operation in the absence of sidelink traffic.

NR SL in release 16 has specified two resource allocation modes, i.e. mode 1 and mode 2. Mode 1 refers to SL resource allocation by network and mode 2 is used for a UE to select SL resource autonomously. In mode 1 resource selection, the resources are granted by the network and, therefore, network may ensure that the SL resources allocated to different UEs do not overlap in order to provide a good communication reliability. While in mode 2, the UE needs to autonomously select SL transmit resources, e.g. based on the outcome of its sensing procedure. Due to the UE-autonomous resource selection operation in mode 2, there is a risk that multiple sidelink UEs will select the same resource, which will introduce collisions among them. This is especially a problem for selecting a set of periodic resources, where such collisions may continue over multiple periods.

If SL DRX is applied, multiple SL TX UEs may need to transmit towards one SL RX UE in the same SL DRX on-duration. This allows the SL RX UE to only wake up in one SL DRX on-duration in order to receive the data from multiple SL TX UEs, and it helps the SL RX UE to maximize its power/energy saving. However, if the mentioned multiple SL TX UEs operate in mode 2, having them to transmit in the same time window may further increase their collision risk since they may select overlapped SL transmit resources. Thus, although aligning the transmissions from different SL TX UEs in the same SL DRX on-duration may be beneficial for power saving at the SL RX UE, it may decrease communication reliability, as it may increase the number of SL TX UEs choosing and using the same resource selection window, i.e. the SL DRX on-duration of the SL RX UE.

Observation 3: Applying SL DRX at a SL RX UE may increase the number of its peer SL TX UEs choosing the same resource selection window and, thus, it may increase the collision risk in NR SL mode 2.

Proposal 8: RAN2 is suggested to study the impact of SL DRX on communication reliability for NR SL mode 2.

# 5	Conclusion

During this contribution we have provided the following observations and proposals;

Observation 1: SL DRX configuration coordination among multiple SL communications in proximity is beneficial from power saving perspective.

Proposal 1: RAN2 to follow SA2s solutions as baseline for agreements on DRX alignment i.e. the (pre-) configured/provisioned SL DRX configurations should be considered by SL UEs to determine and agree on SL DRX parameters between relevant SL UEs in order to facilitate the coordination of SL DRX configuration among relevant SL UEs in proximity.

Proposal 2: The information of selected/applied SL DRX configuration should be provided to other relevant SL UEs in proximity over PC5 interface.

Proposal 3: RAN2 is kindly asked to discuss whether DRX configurations should be determined entirely by the AS layer or based on assistance information provided by the upper layers.

Proposal 4: SL DRX is supported for SL UE in Mode 2.

Proposal 5: Support of SL DRX for SL UE in Mode 2 can be over a shared resource pool which can be used for Mode 2 transmissions of SL UEs with SL DRX and SL UEs without SL DRX.

Proposal 6: RAN2 to send a LS to RAN1 asking RAN1 to study enhancements needed for Mode 2 in order to support SL DRX for SL UE in Mode 2 over a shared resource pool which can be used for Mode 2 transmissions of SL UEs with SL DRX and SL UEs without SL DRX.

Observation 2: In NR Rel-16 Uu has specified a wake up mechanism as to avoid unnecessary DRX_ON period, when there is no traffic.

Proposal 7: RAN2 to study how to avoid unnecessary DRX_ON periods within SL DRX operation in the absence of sidelink traffic.

Observation 3: Applying SL DRX at a SL RX UE may increase the number of its peer SL TX UEs choosing the same resource selection window and, thus, it may increase the collision risk in NR SL mode 2.

Proposal 8: RAN2 is suggested to study the impact of SL DRX on communication reliability for NR SL mode 2.

# 6	References

[1] RP-201516 WID revision: NR sidelink enhancement

[2] TR 23.776 Study on architecture enhancements for 3GPP support of advanced Vehicle-to-Everything (V2X) services; Phase 2 (release 17); V0.2.0