3GPP TSG-RAN WG2#112-e Meeting	Tdoc R2-2010082

# Electronic, 2nd- 13th November 2020

Agenda Item:		6.10.3

Souce:	Samsung

Title:	Logged MDT support for non-SIB4 frequencies (early measurments)

Document for:		Discussion and decision

# Introduction

This paper discusses support of logged MDT for non-SIB4 frequencies, for which UE may have results available based on early measurments. As the specification is inconsistent, we provided a CR to R2#111e aiming to clarify that this is not suppored in R16 [2]. The CR was not agreed, which we assume was mainly because it also included some other aspects. Hence, we like to revist the issue by paper that again proposes to defer the featue to R17.

# Discussion

At present, the specification is inconsistent regarding support of logged MDT for non-SIB4 freqeuncies, for which results may be available based on early measurements (e.g. according to SIB11).

- Field description indicates that InterFreqTargetInfo can only include SIB4 frequencies

- If the field is not configured, the UE will provide results for non-SIB4 frequencies according to the procedural specification

We think the UE behaviour should be consistent for the two cases. In the following table we are reviewing the changes we think that would be required when clarifying support of this feature.

| Support in R16 | Do not support in R16 |
| --- | --- |
| Keep procedural specification unchanged | Modify procedures to state that UE only performs logging for frequencies listed in SIB4 (at least when interFreqTargetList is absent) |
| Remove the restriction in field description of interFreqTargetList that UE only performs logging for frequencies listed in SIB4 | None |
| Discuss and conclude whether there is need for UE capability, or whether condiationally mandatory i.e. required for UE supporting early measurements and logged MDT | None |

Tab.1 Changes for options for support of logged MDT for non-SIB4 frequencies

We note that in R16 it is possible for network to configure UE to perform (early) measurements for non-SIB4 frequencies, either by SIB11 or by dedicated signaling. We think that introducing logging of these results is not very complex for the network i.e. would not require SN control of logged MDT (as will revisited in R17). This is because the MDT configuration merely controls the logging but does not affect the frequencies to measure. Hence, the detailed parameters that might benefit from SN control are outside the MDT configuration.

When supporting the feature in R16 a UE capability may be deemed necessary. Although we think support of the feature is desirable it seems not essential. In general, we should be restrictive when adding UE requirements for R16 at this late stage. Hence, we think it is preferable to defer support of the functinoality to R17.

Proposal      	Do not support logged MDT for non-SIB4 frequencies in R16 (Can be considered for R17). Review and agree the corresponding CR available in [3]

# Conclusion & recommendation

The paper discusses support of logged MDT for non-SIB4 frequencies, for which UE may have results available based on early measurments. The document includes the following proposal that RAN2 is requested to discuss and conclude:

Proposal      	Do not support logged MDT for non-SIB4 frequencies in R16 (Can be considered for R17). Review and agree the corresponding CR available in [3]

# References

[1] TS 38.331 Radio Resource Control

[2] R2-2007225	Clarification on MDT regarding area configuration and target frequencies

[2] R2-2010083 CR to 38.331 (R16) on Clarification on logged MDT for non-SIB4 frequencies