3GPP TSG-RAN WG2 Meeting #112 electronic                 	   R2-2010134

Online, 2nd - 13th November, 2020

Source: 	Huawei, HiSilicon

Title: 	Discussion on RAN3 LS on full slot format support

Agenda Item:	6.14.2

Document for:	Discussion and decision

# Introduction

RAN3 sent an LS (R3-205794/R2-2008729) on the slot format support issue in TDD UL/DL configuration after RAN3 #109-e meeting as follows.

1. Overall Description: As part of RIM/CLI work item, RAN3 defined “Intended TDD UL-DL Configuration NR” IE (section 9.2.2.40 of TS 38.423) with slot configuration list, which is exchanged over Xn/F1 interface. RAN3 found that slot formats 46-55 (Table 11.1.1-1 of TS 38.213) cannot be supported except the slot configuration can be used as half-slot configuration. RAN3 would like to confirm with RAN1 whether all the slot formats defined in TS 38.213 shall be supported in the intended TDD UL-DL configuration exchange. XnAP/F1AP specs also quoted tdd-UL-DL-ConfigurationCommon defined in TS 38.331 for the TDD UL-DL configuration of each cell. RAN3 would like to confirm with RAN2 whether all the slot formats defined in TS 38.213 are supported by tdd-UL-DL-ConfigurationCommon. If yes, has maxNrofSlots = 320 already taken half-slot into consideration? 2. Actions: To RAN1: ACTION: RAN3 would like to ask RAN1 whether all the slot formats defined in TS 38.213 shall be supported in the intended TDD UL-DL configuration exchange. To RAN2: ACTION: RAN3 would like to ask RAN2 whether all the slot formats defined in TS 38.213 are supported by tdd-UL-DL-ConfigurationCommon. If yes, has maxNrofSlots = 320 already taken half-slot into consideration?

In this contribution, we discuss and provide our understanding of the slot configuration regarding RAN3’s questions.

# Discussion

According to RAN2 [1] and RAN1 [2] specifications, there are four ways for configuring slot formats and the corresponding override priority is illustrated as follows.

- TDD-UL-DL-ConfigCommon

The IE TDD-UL-DL-ConfigCommon provides the cell specific Uplink/Downlink TDD configuration which indicates a number of consecutive full DL/UL slots and/or partial DL/UL slots (as highlighted in yellow below) while the remaining unindicated will be flexible symbols if any.

#### – TDD-UL-DL-ConfigCommon The IE TDD-UL-DL-ConfigCommon determines the cell specific Uplink/Downlink TDD configuration. TDD-UL-DL-ConfigCommon information element -- ASN1START -- TAG-TDD-UL-DL-CONFIGCOMMON-START TDD-UL-DL-ConfigCommon ::= SEQUENCE { referenceSubcarrierSpacing SubcarrierSpacing, pattern1 TDD-UL-DL-Pattern, pattern2 TDD-UL-DL-Pattern OPTIONAL, -- Need R ... } TDD-UL-DL-Pattern ::= SEQUENCE { dl-UL-TransmissionPeriodicity ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10}, nrofDownlinkSlots INTEGER (0..maxNrofSlots), nrofDownlinkSymbols INTEGER (0..maxNrofSymbols-1), nrofUplinkSlots INTEGER (0..maxNrofSlots), nrofUplinkSymbols INTEGER (0..maxNrofSymbols-1), ..., [[ dl-UL-TransmissionPeriodicity-v1530 ENUMERATED {ms3, ms4} OPTIONAL -- Need R ]] } -- TAG-TDD-UL-DL-CONFIGCOMMON-STOP -- ASN1STOP TDD-UL-DL-ConfigCommon field descriptions. referenceSubcarrierSpacing Reference SCS used to determine the time domain boundaries in the UL-DL pattern which must be common across all subcarrier specific carriers, i.e., independent of the actual subcarrier spacing using for data transmission. Only the values 15, 30 or 60 kHz (FR1), and 60 or 120 kHz (FR2) are applicable. The network configures a not larger than any SCS of configured BWPs for the serving cell. The network or SL-PreconfigGeneral configures a not larger than the SCS of (pre-)configured SL BWP.See TS 38.213 [13], clause 11.1.. TDD-UL-DL-Pattern field descriptions. dl-UL-TransmissionPeriodicity Periodicity of the DL-UL pattern, see TS 38.213 [13], clause 11.1. If the dl-UL-TransmissionPeriodicity-v1530 is signalled, UE shall ignore the dl-UL-TransmissionPeriodicity (without suffix).. nrofDownlinkSlots Number of consecutive full DL slots at the beginning of each DL-UL pattern, see TS 38.213 [13], clause 11.1. In this release, the maximum value for this field is 80.. nrofDownlinkSymbols Number of consecutive DL symbols in the beginning of the slot following the last full DL slot (as derived from nrofDownlinkSlots). The value 0 indicates that there is no partial-downlink slot. (see TS 38.213 [13], clause 11.1).. nrofUplinkSlots Number of consecutive full UL slots at the end of each DL-UL pattern, see TS 38.213 [13], clause 11.1. In this release, the maximum value for this field is 80.. nrofUplinkSymbols Number of consecutive UL symbols in the end of the slot preceding the first full UL slot (as derived from nrofUplinkSlots). The value 0 indicates that there is no partial-uplink slot. (see TS 38.213 [13], clause 11.1)..

As for the maximum number of slots that can be configured in TDD-UL-DL-ConfigCommon, although the maxNrofSlots is 320, the actual configuration in the current release cannot reach this maximum value. According to the IE structure and configuration constrain (as highlighted in green above), the maximum applicable value of the referenceSubcarrierSpacing which should be no larger than any SCS of configured BWPs is 120 kHz while the maximum periodicity is 10 ms, which means that the maximum number of slots included in one periodicity is 10×23=80 (according to the formula  in TS 38.331, clause 11.1). Hence, it is specified in the above field descriptions that the maximum number of full UL slots and full DL slots is 80 respectively in the current release.

- TDD-UL-DL-ConfigDedicated

The IE TDD-UL-DL-ConfigDedicated provides the UE specific Uplink/Downlink TDD configuration which can override the symbols indicated as flexible in the common configuration (i.e., TDD-UL-DL-ConfigCommon).

#### – TDD-UL-DL-ConfigDedicated The IE TDD-UL-DL-ConfigDedicated determines the UE-specific Uplink/Downlink TDD configuration. TDD-UL-DL-ConfigDedicated information element -- ASN1START -- TAG-TDD-UL-DL-CONFIGDEDICATED-START TDD-UL-DL-ConfigDedicated ::= SEQUENCE { slotSpecificConfigurationsToAddModList SEQUENCE (SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotConfig OPTIONAL, -- Need N slotSpecificConfigurationsToReleaseList SEQUENCE (SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotIndex OPTIONAL, -- Need N ... } [text omitted here] TDD-UL-DL-SlotConfig ::= SEQUENCE { slotIndex TDD-UL-DL-SlotIndex, symbols CHOICE { allDownlink NULL, allUplink NULL, explicit SEQUENCE { nrofDownlinkSymbols INTEGER (1..maxNrofSymbols-1) OPTIONAL, -- Need S nrofUplinkSymbols INTEGER (1..maxNrofSymbols-1) OPTIONAL -- Need S } } } [text omitted here] TDD-UL-DL-SlotIndex ::= INTEGER (0..maxNrofSlots-1) -- TAG-TDD-UL-DL-CONFIGDEDICATED-STOP -- ASN1STOP TDD-UL-DL-SlotConfig field descriptions. nrofDownlinkSymbols Number of consecutive DL symbols in the beginning of the slot identified by slotIndex. If the field is absent the UE assumes that there are no leading DL symbols. (see TS 38.213 [13], clause 11.1).. nrofUplinkSymbols Number of consecutive UL symbols in the end of the slot identified by slotIndex. If the field is absent the UE assumes that there are no trailing UL symbols. (see TS 38.213 [13], clause 11.1).. slotIndex Identifies a slot within a slot configuration period given in tdd-UL-DL-configurationCommon, see TS 38.213 [13], clause 11.1.. symbols The direction (downlink or uplink) for the symbols in this slot. Value allDownlink indicates that all symbols in this slot are used for downlink; value allUplink indicates that all symbols in this slot are used for uplink; value explicit indicates explicitly how many symbols in the beginning and end of this slot are allocated to downlink and uplink, respectively..

- Slot Format Indicator (SFI)

On the basis of the common and dedicated TDD UL/DL configuration provided via RRC messages, a specific SFI-DCI, i.e. DCI format 2_0, can be used for redefining the direction of symbols which are configured as flexible according to the RRC configurations. More specifically, slot formats defined in the Table 11.1.1-1 of TS 38.213 are identified by the slot format indicator (SFI) in DCI format 2_0 and the UE can determine the symbol allocation within each indicated slot based on the received SFI information.

- Scheduling DCI

Apart from the above three approaches, the scheduling DCI (e.g., DCI format 1_0, DCI format 1_1, DCI format 0_0, DCI format 0_1, etc.) can also override symbols that are determined as flexible after taking into account all the possible common RRC configuration, dedicated RRC configuration and the DCI-based SFI.

Fig.1 An example for slot configuration

Fig. 1 presents an example for the above four ways. Flexible symbols configured via TDD-UL-DL-ConfigCommon can be subsequently overridden by tdd-UL-DL-ConfigurationDedicated, DCI format 2_0 or scheduling DCI according to the rule in TS 38.213. The IE tdd-UL-DL-ConfigurationCommon does not directly configure the slot formats in Table 11.1.1.-1 of TS 38.213, but provides a cell specific configuration where flexible symbols can be redefined to support the slot formats in TS 38.213 instead.

Therefore, regarding RAN3’s questions to RAN2, we have the following observations.

Observation 1: The IE tdd-UL-DL-ConfigurationCommon provides a cell-specific TDD UL/DL configuration where symbols indicated as flexible can be overridden by tdd-UL-DL-ConfigurationDedicated, DCI format 2_0 or scheduling DCI according to the rule in TS 38.213.

Observation 2: Slot formats defined in Table 11.1.1-1 of TS 38.213 are identified by slot format indicators (SFI) in DCI format 2_0 which can be used to indicate the symbol allocation for flexible symbols. The IE tdd-UL-DL-ConfigurationCommon does not directly configure the slot formats in TS 38.213 and the half-slot configuration is not considered in tdd-UL-DL-ConfigurationCommon.

Observation 3: The maximum number of slots that can be configured in tdd-UL-DL-ConfigurationCommon is 80 in the current release, which does not reach the maxNrofSlots (i.e. 320).

# Conclusion

In this paper we discuss the slot format support issue raised by RAN3. Observations are summarized as below.

Observation 1: The IE tdd-UL-DL-ConfigurationCommon provides a cell-specific TDD UL/DL configuration where symbols indicated as flexible can be overridden by tdd-UL-DL-ConfigurationDedicated, DCI format 2_0 or scheduling DCI according to the rule in TS 38.213.

Observation 2: Slot formats defined in Table 11.1.1-1 of TS 38.213 are identified by slot format indicators (SFI) in DCI format 2_0 which can be used to indicate the symbol allocation for flexible symbols. The IE tdd-UL-DL-ConfigurationCommon does not directly configure the slot formats in TS 38.213 and the half-slot configuration is not considered in tdd-UL-DL-ConfigurationCommon.

Observation 3: The maximum number of slots that can be configured in tdd-UL-DL-ConfigurationCommon is 80 in the current release, which does not reach the maxNrofSlots (i.e. 320).

# References

- 3GPP TS 38.331, V16.2.0, Radio Resource Control (RRC) protocol specification.

- 3GPP TS 38.213, V16.3.0, Physical layer procedures for control.