3GPP TSG-RAN WG2 Meeting #112 electronic	 R2-2010142

Online, November 2nd – 13th, 2020

Agenda item:	8.15.2 SL DRX for broadcast groupcast and unicast

Source:	Convida Wireless

Title:	Sidelink and Uu DRX

Document for:	Discussion & Decision

# Introduction

As part of the Rel-16 NR sidelink work item, the interaction of the sidelink on the NR Uu DRX was briefly discussed. During these discussions, a proposal regarding whether the MAC entity should monitor the PDCCH for the sidelink related RNTIs was presented. The proposal was not adopted for Rel-16.

As the Rel.17 NR_SL_enh WID [1] has as an objective to study power savings for sidelink UEs, it will touch on the Uu DRX operation.

3. Sidelink DRX for broadcast, groupcast, and unicast [RAN2]

- Define on- and off-durations in sidelink and specify the corresponding UE procedure

- Specify mechanism aiming to align sidelink DRX wake-up time among the UEs communicating with each other

- Specify mechanism aiming to align sidelink DRX wake-up time with Uu DRX wake-up time in an in-coverage UE

In this contribution, we propose to have the MAC entity monitor the PDCCH for the sidelink related RNTIs, if the Uu DRX is configured.

# Discussion

In LTE DRX, during the DRX cycle On Duration, the UE monitors the PDCCH for the following RNTI:  C-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, Semi-Persistent Scheduling C-RNTI (if configured), UL Semi-Persistent Scheduling V-RNTI (if configured), eIMTA-RNTI (if configured), SL-RNTI (if configured), SL-V-RNTI (if configured), CC-RNTI (if configured), SRS-TPC-RNTI (if configured), and AUL C-RNTI (if configured). As the MAC entity is monitoring for the SL related RNTIs, the Uu DRX operation is impacted by the sidelink operation.

In [2], an email discussion targeting MAC issues related to sidelink operation, companies proposed that if DRX is configured for NR, the MAC entity may monitor the PDCCH for the MAC entity's SL-RNTI, SLCS-RNTI and SL Semi-Persistent Scheduling V-RNTI. During the email discussion and online meeting, the main arguments against the proposal were: lack of time in Rel.16, NR Uu  DRX likely not needed for V2X UEs, and issue can be studied in Rel. 17. As a compromise, the agreement in Rel. 16 was:

UE does not expect DRX configuration if SL mode1 is configured in Rel-16.

The Rel-17 sidelink enhancement work clearly targets UEs that require power saving, and as a result, they will likely have a DRX configuration.

In addition, as there were no strong technical arguments against the original proposal, and there will be a need for such a proposal for sidelink UEs (other than vehicular), we propose that when in RRC_CONNECTED, if DRX is configured, the MAC entity may monitor the PDCCH for the MAC entity's SL-RNTI, SLCS-RNTI and SL Semi-Persistent Scheduling V-RNTI.

Proposal 1: When in RRC_CONNECTED, if DRX is configured, the MAC entity may monitor the PDCCH for the MAC entity's SL-RNTI, SLCS-RNTI and SL Semi-Persistent Scheduling V-RNTI.

# Conclusion

In this contribution, we propose:

Proposal 1: When in RRC_CONNECTED, if DRX is configured, the MAC entity may monitor the PDCCH for the MAC entity's SL-RNTI, SLCS-RNTI and SL Semi-Persistent Scheduling V-RNTI.

# References

- RP-201516, NR sidelink enhancement

- R2-2005720, Report of [Post109bis-e][957][V2X]: MAC issues, LG Electronics