3GPP TSG-RAN2 Meeting # 112 electronic	R2-2010201

# Online, November 2nd –13th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4500 | rev | - | Current version: | 16.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Inter-RAT SON for 36.331 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2020-10-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the MRO definition in stage 2, there are intra-system MRO and inter-system MRO cases. For intra-system MRO, it involves mobility between NR and eLTE. For inter-system MRO, in involves mobility between legacy LTE and NG-RAN(NR and eLTE). - For intra-system MRO, there are intra-system too late handover, intra-system too early handover and intra-system handover to wrong cell, including both HOF and RLFcases. - For inter-system MRO, there are only the RLF cases, one is too late inter-system handover from NG-RAN(NR and eLTE) to legacy LTE, and the other is too early inter-system handover from legacy LTE to NG-RAN(NR and eLTE) with RLF case. The description for the MRO cases to collect RLF Report is unclear. For handover to E-UTRA from NR, the RRCReconfiguration message including the reconfigurationWithSync should be embedded in E-UTRA RRC message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the HOF case occured between E-UTRA/EPC and E-UTRA/EPC, between E-UTRA/5GC and E-UTRA/5GC and from E-UTRA/5GC to NR. Clarify the RLF case occured from E-UTRA/EPC to E-UTRA, between E-UTRA/5GC and E-UTRA/5GC and from NR to E-UTRA/5GC. Clarify that the RRCConnectionReconfiguration message including the mobilityControlInfo is embedded in NR RRC message upon handover to E-UTRA from NR. Impact Analysis Impacted 5G architecture options: SA Impacted functionality RLF report collection. Inter-operability If the network is implemented according to the CR while the UE is not, the UE may collect the RLF Report in case of unsupported MRO cases. If the UE is implemented according to the CR while the network is not, there is no impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE will collect RLF report in case of unsupported MRO cases. And the network will not rightly decode it. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.6, 5.3.11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the change

#### 5.3.5.6	T304 expiry (handover failure)

If T304 expires (handover failure), the UE shall:

NOTE 1:	Following T304 expiry any dedicated preamble, if provided within the rach-ConfigDedicated, is not available for use by the UE anymore.

1>	if no DAPS bearer is configured; or

1>	if any DAPS bearer is configured and radio link failure has been detected for the source MCG in accordance with 5.3.11.3:

2>	if attemptCondReconf is not configured:

3>	revert back to the configuration used in the source PCell, excluding the configuration configured by the physicalConfigDedicated, the mac-MainConfig and the sps-Config;

2>	else:

3>	revert back to the configuration used in the source PCell;

NOTE 1a:	In the context above, "the configuration" includes state variables and parameters of each radio bearer. PDCP entities associtated with RLC UM and SRB bearers are reset after the successful RRC connection re-establishment procedure according to clause 5.2 in TS 36.323 [8]. In the above, "the configuration" includes the RB configuration using NR PDCP, if configured (i.e. by nr-RadioBearerConfig1 and nr-RadioBearerConfig2).

2>	store the following handover failure information in VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the source PCell based on measurements collected up to the moment the UE detected handover failure and in accordance with the following;

4>	if the UE includes rsrqResult, include the lastServCellRSRQ-Type;

3>	set the measResultNeighCells to include the best measured cells, other than the source PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected handover failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE includes rsrqResult, include the rsrq-Type;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

4>	if the UE was configured to perform measurement reporting, not related to NR sidelink communication, for one or more neighbouring NR frequencies, include the measResultListNR;

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

3>	if available, set the logMeasResultListWLAN to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

3>	if available, set the logMeasResultListBT to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	if last RRCConnectionReconfiguration message including mobilityControlInfo concerned a failed intra-RAT handover (E-UTRA/EPC to E-UTRA/EPC, or E-UTRA/5GC to E-UTRA/5GC):

4>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

3>	else if last MobilityFromEUTRACommand concerned a failed inter-RAT handover from E-UTRA/5GC to NR and if the UE supports Radio Link Failure Report for Inter-RAT MRO (EUTRA/5GC to NR):

4>	set the failedNR-PCellId to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

3>	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

3>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to 'hof';

3>	set the c-RNTI to the C-RNTI used in the source PCell;

2>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the RRC connection reconfiguration procedure ends;

1>	else (any DAPS bearer is configured and radio link failure has not been detected for the source MCG):

2>	release the MAC entity for the target PCell;

2>	for each DAPS bearer:

3>	re-establish the RLC entity for the target PCell;

3>	release the RLC entity or entities and the associated DTCH logical channel for the target PCell;

3>	reconfigure the PDCP entity to release DAPS as specified in TS 36.323 [8];

2>	for each non-DAPS bearer:

3>	revert back to the configuration used for the DRB in the source PCell, including PDCP and RLC states and the security configuration;

2>	for each SRB:

3>	discard any PDCP SDUs along with the PDCP data PDUs for the source PCell;

3>	re-establish the RLC entity for the source PCell;

3>	release the PDCP entity for the target PCell;

3>	release the RLC entity and the associated DCCH logical channel for the target PCell;

2>	resume the SRBs for the source PCell;

2>	initiate the failure information procedure as specified in 5.6.21 to report a DAPS HO failure.

The UE may discard the handover failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the failure is detected, upon power off or upon detach.

NOTE 3:	E-UTRAN may retrieve the handover failure information using the UE information procedure with rlf-ReportReq set to true, as specified in 5.6.5.3.

Start of next change

#### 5.3.11.3	Detection of radio link failure

The UE shall:

1>	in case any DAPS bearer is configured, only the target PCell is considered in the following;

1>	upon T310 expiry; or

1>	upon T312 expiry; or

1>	upon random access problem indication from MCG MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from MCG RLC, which is allowed to be send on PCell, that the maximum number of retransmissions has been reached for an SRB or DRB:

2>	consider radio link failure to be detected for the MCG i.e. RLF;

2>	discard any segments of segmented RRC messages received;

2>	if the UE is configured with (NG)EN-DC; and

2>	if T316 is configured; and

2>	if SCG transmission is not suspended; and

2>	if NR PSCell change is not ongoing (i.e. T304 for the NR PSCell is not running as specified in TS 38.331 [82], clause 5.3.5.5.2, in (NG)EN-DC):

3>	initiate the MCG failure information procedure as specified in 5.6.26 to report MCG radio link failure;

2>	else:

3>	store the following radio link failure information in the VarRLF-Report (VarRLF-Report-NB in NB-IoT) by setting its fields as follows:

4>	clear the information included in VarRLF-Report (VarRLF-Report-NB in NB-IoT), if any;

4>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

4>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio link failure;

4>	except for NB-IoT, set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio link failure, and set its fields as follows;

5>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

5>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

5>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

5>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

5>	if the UE was configured to perform measurement reporting, not related to NR sidelink communication, for one or more neighbouring NR frequencies, include the measResultListNR;

5>	for each neighbour cell included, include the optional fields that are available;

NOTE 1:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

4>	except for NB-IoT, if available, set the logMeasResultListWLAN to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

4>	except for NB-IoT, if available, set the logMeasResultListBT to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

4>	if detailed location information is available, set the content of the locationInfo as follows:

5>	include the locationCoordinates;

5>	include the horizontalVelocity, if available;

4>	set the failedPCellId to the global cell identity, if available, and otherwise , except for NB-IoT, to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

4>	except for NB-IoT, set the tac-FailedPCell to the tracking area code, if available, of the PCell where radio link failure is detected;

4>	except for NB-IoT, if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure:

5>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned an intra E-UTRA handover (E-UTRA/EPC to E-UTRA, or E-UTRA/5GC to E-UTRA/5GC):

6>	include the previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

6>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

5>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned a handover to E-UTRA from UTRA and if the UE supports Radio Link Failure Report for Inter-RAT MRO:

6>	include the previousUTRA-CellId and set it to the physical cell identity, the carrier frequency and the global cell identity, if available, of the UTRA Cell in which the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

6>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

5>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo embedded in NR RRC message MobilityFromNRCommand message as specified in TS 38.331 [82] clause 5.4.3.3 concerned a handover to E-UTRA/5GC from NR and if the UE supports Radio Link Failure Report for Inter-RAT MRO:

6>	include the previousNR-PCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received embedded in NR RRC message MobilityFromNRCommand message as specified in TS 38.331 [82] clause 5.4.3.3;

6>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo embedded in NR RRC message MobilityFromNRCommand message as specified in TS 38.331 [82] clause 5.4.3.3.

4>	except for NB-IoT, if the UE supports QCI1 indication in Radio Link Failure Report and has a DRB for which QCI is 1:

5>	include the drb-EstablishedWithQCI-1;

4>	except for NB-IoT, set the connectionFailureType to rlf;

4>	except for NB-IoT, set the c-RNTI to the C-RNTI used in the PCell;

4>	except for NB-IoT, set the rlf-Cause to the trigger for detecting radio link failure;

3>	if AS security has not been activated:

4>	if the UE is a NB-IoT UE:

5>	if the UE is connected to EPC and the UE supports RRC connection re-establishment for the Control Plane CIoT EPS optimisation; or

5>	if the UE is connected to 5GC and the UE supports RRC connection re-establishment for the Control Plane CIoT 5GS optimisation:

6>	initiate the RRC connection re-establishment procedure as specified in 5.3.7;

5>	else:

6>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

4>	else:

5>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

3>	else:

4>	initiate the connection re-establishment procedure as specified in 5.3.7;

In case of DC or NE-DC, the UE shall:

1>	upon T313 expiry; or

1>	upon random access problem indication from SCG MAC; or

1>	upon indication from SCG RLC, which is allowed to be sent on PSCell, that the maximum number of retransmissions has been reached for an SCG, for a split DRB or for a split SRB:

2>	consider radio link failure to be detected for the SCG i.e. SCG-RLF;

2>	if the UE is configured with DC; or

2>	if the UE is configured with NE-DC and MCG transmission is not suspended:

3>	initiate the SCG failure information procedure as specified in 5.6.13 to report SCG radio link failure;

2>	else:

3>	initiate the connection re-establishment procedure as specified in TS 38.331 [82], clause 5.3.7.

In case of CA PDCP duplication, the UE shall:

1>	upon indication from an RLC entity, which is restricted to be sent on SCell only, that the maximum number of retransmissions has been reached:

2>	initiate the failure information procedure as specified in 5.6.21 to report RLC failure of type duplication;

If any DAPS bearer is configured and T304 is running, the UE shall:

1>	upon T310 expiry; or

1>	upon T312 expiry; or

1>	upon random access problem indication from source MCG MAC; or

1>	upon indication from source MCG RLC, which is allowed to be sent on source PCell, that the maximum number of retransmissions has been reached for an DRB:

2>	consider radio link failure to be detected for the source MCG;

2>	suspend the transmission of all DRBs in the source MCG;

2>	reset MAC for the source MCG;

2>	release the source connection;

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report (VarRLF-Report-NB in NB-IoT), 48 hours after the radio link failure is detected, upon power off or upon detach, and for NB-IoT, upon entering another RAT.

End of the change