3GPP TSG-RAN WG2 Meeting #112- e  		        			    	  R2-2010213

Electronic, 2nd Nov. – 13th Nov. 2020

Source: 	vivo

Title:  	Discussion on IIOT QoS impacts in RAN

Agenda Item:	8.5.4

Document for:	Discussion and Decision

Introduction

According to the IIOT/URLLC WID [1] as quoted below, one major objective to RAN enhancements based on new QoS related parameters.

- RAN enhancements based on new QoS related parameters if any, e.g. survival time, burst spread, decided in SA2. [RAN2, RAN3]

In this contribution, we will focus on the survival time parameter and give our understanding of this concept. In addition, we will try to analyze the possible RAN impact and give our proposals to support this new QoS parameter.

Discussion

Concept of survival time

According to the description in TS 22.104, the survival time is defined as below:

Survival time

The maximum survival time indicates the time period the communication service may not meet the application's requirement before the communication service is deemed to be in an unavailable state.

NOTE 1:	The survival time indicates to the communication service the time available to recover from failure. This parameter is thus tightly related to maintainability [7].

Obviously, survival time is related to ‘Communication service availability’ that is one of the requirements. In TS 22.104, the definition of Communication service availability is given as:

communication service availability

The percentage value of the amount of time the end-to-end communication service is delivered according to an agreed QoS, divided by the amount of time the system is expected to deliver the end-to-end service according to the specification in a specific area.

NOTE 2:	The end point in "end-to-end" is assumed to be the communication service interface.

NOTE 3:	The communication service is considered unavailable if it does not meet the pertinent QoS requirements. If availability is one of these requirements, the following rule applies: the system is considered unavailable if an expected message is not received within a specified time, which, at minimum, is the sum of maximum allowed end-to-end latency and survival time.

An example of the relationship between survival time and communication service availability is presented in the Annex C.3 of TS 22.104 as below. Based the description of specification, as far as timely received messages are correct, the logical communication link status is up state (or alternatively “available”). If a message loss or an incorrectly or untimely received message is detected the logical communication link status is down state (or alternatively “unavailable”).

The logical communication link status changes to down state if it no longer can support end-to-end transmission of the source device's messages to the target device in agreement with the negotiated communication requirements. As shown in the figure, the application status is treated as failure when an application message cannot be received at the application of the target device. Thus, the mentioned end-to-end transmission refers to end-to-end transmission failure of an application message. The survival time will start when an expected message is not received within a specific time (i.e. end-to-end latency of an application message).

Fig.1.  Relation between logical communication link, communication service and application statuses (example with lost messages)

Observation 1: Survival time starts when an expected message is not received within a specific time (i.e. end-to-end latency of an application message).

When the logical communication link status changes to down state, the communication service and the application will wait a preset time (i.e. survival time) before they transition to unavailable state. During the survival time, the communication service status and the application status still keep up state.

Observation 2: Both the communication service and the Application will change to the unavailable state when the survival time has been exceeded.

As specified in TS 22.104, the performance requirement of communication service availability is up to 99.999999%. In order to satisfy the requirement of communication service availability, the number of times that the communication service transition to unavailable state should be reduced. It means that a packet should be guaranteed to receive successfully at the receiver before the survival time is exceeded. From the view of RAN, RAN should schedule packets with higher priority if the lost of these packets will lead the application of target device entering unavailable state. In order to realizing this kind of differentiated scheduling, RAN should know that the survival time is going to run out.

Observation 3: It is benefit for RAN to know that the survival time of a packet is going to run out so that RAN can scheduling the packets with high priority to avoid the application of target device entering unavailable state.

RAN impact on the support of survival time

For periodic traffic flow, survival time can be equal to one or multiple transfer interval (which can refer to TS 22.104). If survival time equals to one transfer interval, the application cannot tolerant that any two consecutive messages are lost. The following figures gives an example to show the transition of Application status when survive time is larger than one transfer interval and less than two transfer interval.

Figure 4.3-1: survival time vs. message loss

As shown in the figure, when a packet is detected as lost, the survival time starts. If a packet is received during the survival time, the application still keep available status. But if two consecutive messages are detected as lost, the application change to unavailable status. Thus, RAN shall guarantee that it will not occur that any two consecutive packet are lost.

Proposal 1: The error-free transmission of the consecutive two packets should be considered.

Similarly, if survival time equals to multiple transfer interval, RAN shall guarantee that it will not happen that more than two consecutive packet are lost. An example is shown with the assumption that the survival time equals to 3*transfer interval. The survival time will start when the target device detects that a expected packet is missing. During the survival time, the application of target device will enter unavailable status if the target device cannot receive packets successful. From the view of RAN, it is benefit for gNB to know the maximum number of consecutive packets that are tolerant to be lost in RAN. For example, the application of target device will enter unavailable status if three consecutive packets are lost. Thus, the maximum number of consecutive packets that are tolerant to be lost in RAN is two. When gNB know that two consecutive packet has been lost, RAN can adjust the scheduling policy, e.g. improve the priority of the corresponding radio bearer.

Proposal 2: The maximum number of consecutive packets that are tolerant to be lost in RAN should be provided to gNB, at least for periodic traffic.

For DL, NW can detect that one or more that two consecutive packet are not received by UE based on the feedback (e.g. RLC SR). Then, gNB can schedule the packets for which their survival time is going to expiry. Thus, there is no spec impact for DL data transmission with survival time requirement.

Proposal 3: For DL, there is no spec impact to support survival time.

For UL, gNB can detects that one or more that two consecutive packet by itself. Thus, gNB can adjust scheduling policy to guarantee the packets can be scheduled before the time run out. One possible way to increase reliability during survival from RAN2 perspective is to adjust LCP of the corresponding radio bearer to make it obtain UL grant with high priority.

Similar with gNB, UE can also detect that one or more that two consecutive packet are not received by UE based on the feedback (e.g. RLC SR). Thus, UE informs the gNB that the survival time is running out to assistant gNB adjusting LCP. Another possible solution is that UE automatically adjusts LCP before the survival time runs out, e.g., based on the predefined rules.

From our perspective, all the solutions mentioned above can be used to increase the reliability when entering survival time. It kindly requests RAN2 to consider them as candidate solutions.

Proposal 4: RAN2 should consider UE based and NW based LCP adjustment as candidate solutions for increasing the reliability when entering survival time.

Conclusions

Based on the analysis given above, we have the following Observations and Proposals:

Observation 1: Survival time starts when an expected message is not received within a specific time (i.e. end-to-end delay).

Observation 2: Both the communication service and the Application will change to the unavailable state when the survival time has been exceeded.

Observation 3: It is benefit for RAN to know that the survival time of a packet is going to run out so that RAN can scheduling the packets with high priority to avoid the application of target device entering unavailable state.

Proposal 1: The error-free transmission of the consecutive two packets should be considered.

Proposal 2: The maximum number of consecutive packets that are tolerant to be lost in RAN should be provided to gNB, at least for periodic traffic.

Proposal 3: For DL, there is no spec impact to support survival time.

Proposal 4: RAN2 should consider UE based and NW based LCP adjustment as candidate solutions for increasing the reliability when entering survival time.

Reference

- RP-201310, Nokia, “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR”.

- 3GPP TR 22.104, “Service requirements for cyber-physical control applications in vertical domains”

- 3GPP TR 22.832, “Study on enhancements for cyber-physical control applications in vertical domains”.