3GPP TSG-RAN WG2 Meeting #112 electronic	R2-2010313

2 – 13 November 2020

Agenda Item:	6.4.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on Sidelink process association

Document for:	Discussion and Decision

- Introduction

In this contribution, we would like to discuss left issues about Sidelink process association that need to be corrected in the specification.

- Discussion

Sidelink process association for a selected sidelink grant

Based on the agreements in RAN2#110 e-meeting as listed below, one mode2 resource reservation process will associate with only one Sidelink process, which is inherited from LTE V2X [1].

Agreements on MAC (Note RRC may also need to be updated according to agreements):

29:	For transmissions of multiple MAC PDUs in NR sidelink communication, the maximum number of transmitting Sidelink processes associated with each Sidelink HARQ Entity is ‘4’. (Wording can be further discuseed in CR implementation, but we are not going to change the modeling of mode2 booking process, which is inherited from LTE V2X)

In current 5.22.1.1 of 38.321 [2], the MAC entity performs mode2 resource reservation process for Sidelink process to obtain the selected sidelink grant, that is, the association between the selected sidelink grant and Sidelink process is determined by the MAC entity, which is similar to the operation in LTE V2X.

#### 5.22.1.1 SL Grant reception and SCI transmission …… If the MAC entity has been configured with Sidelink resource allocation mode 2 to transmit using pool(s) of resources in a carrier as indicated in TS 38.331 [5] or TS 36.331 [21] based on sensing or random selection, the MAC entity shall for each Sidelink process: …… 1> if the MAC entity has selected to create a selected sidelink grant corresponding to transmissions of multiple MAC PDUs, and SL data is available in a logical channel: 2> if the MAC entity has not selected a pool of resources allowed for the logical channel: 3> if sl-HARQ-FeedbackEnabled is set to enabled for the logical channel: 4> select any pool of resources configured with PSFCH resources among the pools of resources; 3> else: 4> select any pool of resources among the pools of resources; 2> perform the TX resource (re-)selection check on the selected pool of resources as specified in clause 5.22.1.2; ……

Observation 1: For the selected sidelink grant, the association between the selected sidelink grant and Sidelink process is determined by the MAC entity, which is similar to the operation in LTE V2X.

Duplicated Sidelink process association for a selected sidelink grant

In current specification, it is specified that for each sidelink grant, no matter whether it is configured sidelink grant, dynamic sidelink grant or selected sidelink grant, if the MAC entity determines the sidelink grant is used for initial transmission, the Sidelink HARQ entity needs to associate the sidelink grant to a Sidelink process. Then the Sidelink HARQ entity can instruct the associated Sidelink process to perform transmission.

##### 5.22.1.3.1 Sidelink HARQ Entity …… For each sidelink grant, the Sidelink HARQ Entity shall: 1> if the MAC entity determines that the sidelink grant is used for initial transmission as specified in clause 5.22.1.1; or 1> if the sidelink grant is a configured sidelink grant and no MAC PDU has been obtained in a sl-PeriodCG of the configured sidelink grant: 2> (re-)associate a Sidelink process to this grant, and for the associated Sidelink process: 3> obtain the MAC PDU to transmit from the Multiplexing and assembly entity, if any; 3> if a MAC PDU to transmit has been obtained: …… 1> else (i.e. retransmission): 2> if the HARQ Process ID corresponding to the sidelink grant received on PDCCH is associated to a Sidelink process of which HARQ buffer is empty; or 2> if the HARQ Process ID corresponding to the sidelink grant received on PDCCH is not associated to any Sidelink process: 3> ignore the sidelink grant. 2> else: 3> identify the Sidelink process associated with this grant, and for the associated Sidelink process: 4> deliver the sidelink grant of the MAC PDU to the associated Sidelink process; 4> instruct the associated Sidelink process to trigger a retransmission.

Observation 2: For each sidelink grant for initial transmission including configured sidelnk grant, dynamic sidelink grant and selected sidelink grant, the Sidelink HARQ entity will associate a Sidelink process to this grant.

In this case, duplicated Sidelink process association will happen as during resource reservation procedure the MAC entity has already associated a Sidelink process for a selected sidelink grant, however the Sidelink HARQ entity will also associate a Sidelink process for a selected sidelink grant if this grant is used for initial transmission. Based on the analysis, the following problem may occur:

- Problem: The Sidelink HARQ entity cannot perform retransmission for the Sidelink process determined by the Sidelink HARQ entity, because the selected sidelink grant for retransmission is only associated with the Sidelink process determined by the MAC entity.

Taking the following Fig.1 as an example:

- Step 1: Firstly, the MAC entity performs mode 2 resource reservation process for Sidelink process 1 and delivers the selected sidelink grant to the Sidelink HARQ entity, where the selected Sidelink grants for initial transmission and retransmission are associated with the Sidelink process 1.

- Step 2: Secondly, the Sidelink HARQ entity associates one of the selected sidelink grants for initial transmission (e.g. grant 2) to Sidelink process 2, and the MAC PDU2 is generated and delivered to Sidelink process 2. Then, Sidelink process 2 will perform new transmission of MAC PDU2.

In this case, according to the green highlighted description, as the selected sidelink grant for retransmission is associated with sidelink process 1, the sidelink HARQ entity is not able to perform retransmission for the sidelink process 2, which is not a desired behaviour.

And, for the transmissions on the selected sidelink grant for retransmission after grant 2, as the identified sidelink process is process 1, the UE will trigger a retransmission of the packet in the HARQ buffer corresponding to sidelink process 1, there are two cases:

- Case 1: If the HARQ buffer of Sidelink process 1 is not empty (e.g. there is MAC PDU1 in the HARQ buffer of Sidelink process 1 as the transmission of Sidelink process 1 is not yet successful), the Sidelink HARQ entity will perform retransmission of Sidelink process 1 on the selected sidelink grant for retransmission after grant 2.

- Case 2: If the HARQ buffer of Sidelink process 1 is empty (e.g. Sidelink process 1 flushes the corresponding HARQ buffer as the transmission of Sidelink process 1 is successful), the Sidelink HARQ entity does not perform transmission on the selected sidelink grant for retransmission after grant 2.

Fig.1 Illustration of transmissions for the selected sidelink grant

Observation 3: In current specification, the Sidelink HARQ entity may associate the selected sidelink grant to another Sidelink process which is different from the Sidleink process determined by the MAC entity, which causes that the Sidelink HARQ entity cannot perform retransmission for the Sidelink process determined by the Sidelink HARQ entity.

Therefore, we propose two options to solve the duplicated sidelink process assocaiton issue for a selected sidelink grant as listed below and RAN2 is suggested to discuss and choose one option.

Proposal 1: RAN2 discuss and determine the solution for the duplication association by taking the following two options into consideration:

- Option1: Clarify that the Sidelink HARQ entity only needs to associate a Sidleink process for the configured sidelink grant and the dynamic sidelink grant.

- Option2: Add a Note that the Sidelink HARQ entity will associate the selected sidelink grant to the Sidelink process determined by the MAC entity.

Text proposals based on the two options are given in Annex A. RAN2 is suggested to adopt one of them.

- Conclusion

In this contribution, we discuss the remaining issues of Sidelink process association and achieve the following observations and proposals.

Observation 1: For the selected sidelink grant, the association between the selected sidelink grant and Sidelink process is determined by the MAC entity, which is similar to the operation in LTE V2X.

Observation 2: For each sidelink grant for initial transmission including configured sidelnk grant, dynamic sidelink grant and selected sidelink grant, the Sidelink HARQ entity will associate a Sidelink process to this grant.

Observation 3: In current specification, the Sidelink HARQ entity may associate the selected sidelink grant to another Sidelink process which is different from the Sidleink process determined by the MAC entity, which causes that the Sidelink HARQ entity cannot perform retransmission for the Sidelink process determined by the Sidelink HARQ entity.

Proposal 1: RAN2 discuss and determine the solution for the duplication association by taking the following two options into consideration:

- Option1: Clarify that the Sidelink HARQ entity only needs to associate a Sidleink process for the configured sidelink grant and the dynamic sidelink grant.

- Option2: Add a Note that the Sidelink HARQ entity will associate the selected sidelink grant to the Sidelink process determined by the MAC entity.

- Reference

- Draft_RAN2_110-e_Meeting_Report_v1.

- 3GPP TS 38.321 V16.2.1, NR; Medium Access Control (MAC) protocol specification (Release 16).

Annex A	 Text proposals on TS 38.321

START OF TP-option 1

##### 5.22.1.3.1	Sidelink HARQ Entity

The MAC entity includes at most one Sidelink HARQ entity for transmission on SL-SCH, which maintains a number of parallel Sidelink processes.

The maximum number of transmitting Sidelink processes associated with the Sidelink HARQ Entity is 16. A sidelink process may be configured for transmissions of multiple MAC PDUs. For transmissions of multiple MAC PDUs with Sidelink resource allocation mode 2, the maximum number of transmitting Sidelink processes associated with the Sidelink HARQ Entity is 4.

A delivered sidelink grant and its associated Sidelink transmission information are associated with a Sidelink process. Each Sidelink process supports one TB.

For each sidelink grant, the Sidelink HARQ Entity shall:

1>	if the MAC entity determines that the sidelink grant is used for initial transmission as specified in clause 5.22.1.1; or

1>	if the sidelink grant is a configured sidelink grant and no MAC PDU has been obtained in a sl-PeriodCG of the configured sidelink grant:

2>	if the sidelink grant is a configured sidelink grant or the dynamic sidelink grant;

3>	 (re-)associate a Sidelink process to this grant;

2>	for the associated Sidelink process:

3>	obtain the MAC PDU to transmit from the Multiplexing and assembly entity, if any;

3>	if a MAC PDU to transmit has been obtained:

4>	if a HARQ Process ID has been set for the sidelink grant:

5>	(re-)associate the HARQ Process ID corresponding to the sidelink grant to the Sidelink process;

NOTE 1:	There is one-to-one mapping between a HARQ Process ID and a Sidelink process in the MAC entity configured with Sidelink resource allocation mode 1.

4>	determines Sidelink transmission information of the TB for the source and destination pair of the MAC PDU as follows:

5>	set the Source Layer-1 ID to the 8 LSB of the Source Layer-2 ID of the MAC PDU;

5>	set the Destination Layer-1 ID to the 16 LSB of the Destination Layer-2 ID of the MAC PDU;

5>	consider the NDI to have been toggled and set the NDI to the toggled value;

NOTE 2:	The initial value of the NDI set to the very first transmission for the associated Sidelink process is left to UE implementation.

5>	(re-)associate the Sidelink process to a Sidelink process ID;

NOTE 3:	How UE determine Sidelink process ID in SCI is left to UE implementation for NR sidelink.

5>	set the cast type indicator to one of broadcast, groupcast and unicast as indicated by upper layers;

5>	if HARQ feedback has been enabled for the MAC PDU according to clause 5.22.1.4.2;

6>	set the HARQ feedback enabled/disabled indicator to enabled.

5>	else:

6>	set the HARQ feedback enabled/disabled indicator to disabled.

5>	set the priority to the value of the highest priority of the logical channel(s), if any, and a MAC CE, if included, in the MAC PDU;

5>	if HARQ feedback is enabled for groupcast:

6>	if both a group size and a member ID are provided by upper layers and the group size is not greater than the number of candidate PSFCH resources associated with this sidelink grant:

7>	select either positive-negative acknowledgement or negative-only acknowledgement.

NOTE 4:	Selection of positive-negative acknowledgement or negative-only acknowledgement is up to UE implementation.

6>	else:

7>	select negative-only acknowledgement.

6>	if negative-only acknowledgement is selected, UE's location information is available, and sl-TransRange has been configured for a logical channel in the MAC PDU, and Zone_id is determined as specified in TS 38.331 [5]:

7>	set the communication range requirement to the value of the longest communication range of the logical channel(s) in the MAC PDU;

7>	set Zone_id to the value of the determined Zone_id.

4>	deliver the MAC PDU, the sideink grant and the Sidelink transmission information of the TB to the associated Sidelink process;

4>	instruct the associated Sidelink process to trigger a new transmission.

3>	else:

4>	flush the HARQ buffer of the associated Sidelink process.

1>	else (i.e. retransmission):

2>	if the HARQ Process ID corresponding to the sidelink grant received on PDCCH is associated to a Sidelink process of which HARQ buffer is empty; or

2>	if the HARQ Process ID corresponding to the sidelink grant received on PDCCH is not associated to any Sidelink process:

3>	ignore the sidelink grant.

2>	else:

3>	identify the Sidelink process associated with this grant, and for the associated Sidelink process:

4>	deliver the sidelink grant of the MAC PDU to the associated Sidelink process;

4>	instruct the associated Sidelink process to trigger a retransmission.

END OF TP-option1

START OF TP-option2

##### 5.22.1.3.1	Sidelink HARQ Entity

The MAC entity includes at most one Sidelink HARQ entity for transmission on SL-SCH, which maintains a number of parallel Sidelink processes.

The maximum number of transmitting Sidelink processes associated with the Sidelink HARQ Entity is 16. A sidelink process may be configured for transmissions of multiple MAC PDUs. For transmissions of multiple MAC PDUs with Sidelink resource allocation mode 2, the maximum number of transmitting Sidelink processes associated with the Sidelink HARQ Entity is 4.

A delivered sidelink grant and its associated Sidelink transmission information are associated with a Sidelink process. Each Sidelink process supports one TB.

For each sidelink grant, the Sidelink HARQ Entity shall:

1>	if the MAC entity determines that the sidelink grant is used for initial transmission as specified in clause 5.22.1.1; or

1>	if the sidelink grant is a configured sidelink grant and no MAC PDU has been obtained in a sl-PeriodCG of the configured sidelink grant:

2>	(re-)associate a Sidelink process to this grant, and for the associated Sidelink process:

NOTE 1:	The Sidelink HARQ entity will associate the selected sidelink grant to the Sidelink process determined by the MAC entity.

3>	obtain the MAC PDU to transmit from the Multiplexing and assembly entity, if any;

3>	if a MAC PDU to transmit has been obtained:

4>	if a HARQ Process ID has been set for the sidelink grant:

5>	(re-)associate the HARQ Process ID corresponding to the sidelink grant to the Sidelink process;

NOTE 2:	There is one-to-one mapping between a HARQ Process ID and a Sidelink process in the MAC entity configured with Sidelink resource allocation mode 1.

4>	determines Sidelink transmission information of the TB for the source and destination pair of the MAC PDU as follows:

5>	set the Source Layer-1 ID to the 8 LSB of the Source Layer-2 ID of the MAC PDU;

5>	set the Destination Layer-1 ID to the 16 LSB of the Destination Layer-2 ID of the MAC PDU;

5>	consider the NDI to have been toggled and set the NDI to the toggled value;

NOTE 3:	The initial value of the NDI set to the very first transmission for the associated Sidelink process is left to UE implementation.

5>	(re-)associate the Sidelink process to a Sidelink process ID;

NOTE 4:	How UE determine Sidelink process ID in SCI is left to UE implementation for NR sidelink.

5>	set the cast type indicator to one of broadcast, groupcast and unicast as indicated by upper layers;

5>	if HARQ feedback has been enabled for the MAC PDU according to clause 5.22.1.4.2;

6>	set the HARQ feedback enabled/disabled indicator to enabled.

5>	else:

6>	set the HARQ feedback enabled/disabled indicator to disabled.

5>	set the priority to the value of the highest priority of the logical channel(s), if any, and a MAC CE, if included, in the MAC PDU;

5>	if HARQ feedback is enabled for groupcast:

6>	if both a group size and a member ID are provided by upper layers and the group size is not greater than the number of candidate PSFCH resources associated with this sidelink grant:

7>	select either positive-negative acknowledgement or negative-only acknowledgement.

NOTE 5:	Selection of positive-negative acknowledgement or negative-only acknowledgement is up to UE implementation.

6>	else:

7>	select negative-only acknowledgement.

6>	if negative-only acknowledgement is selected, UE's location information is available, and sl-TransRange has been configured for a logical channel in the MAC PDU, and Zone_id is determined as specified in TS 38.331 [5]:

7>	set the communication range requirement to the value of the longest communication range of the logical channel(s) in the MAC PDU;

7>	set Zone_id to the value of the determined Zone_id.

4>	deliver the MAC PDU, the sideink grant and the Sidelink transmission information of the TB to the associated Sidelink process;

4>	instruct the associated Sidelink process to trigger a new transmission.

3>	else:

4>	flush the HARQ buffer of the associated Sidelink process.

1>	else (i.e. retransmission):

2>	if the HARQ Process ID corresponding to the sidelink grant received on PDCCH is associated to a Sidelink process of which HARQ buffer is empty; or

2>	if the HARQ Process ID corresponding to the sidelink grant received on PDCCH is not associated to any Sidelink process:

3>	ignore the sidelink grant.

2>	else:

3>	identify the Sidelink process associated with this grant, and for the associated Sidelink process:

4>	deliver the sidelink grant of the MAC PDU to the associated Sidelink process;

4>	instruct the associated Sidelink process to trigger a retransmission.

END OF TP-option2