3GPP TSG-RAN WG2 Meeting #112-e	R2-2010342

# Electronic Meeting, 2 – 13 Nov 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1797 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding UE capability for beam level early measurement reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2020-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN4 agreed to introduce beam level NR early measurement reporting in case of (NG)EN-DC and send LS in R4-2011929 to require RAN2 to capture the corresponding UE capability. 6-2a; Support of beam level Early Measurement Reporting; Supporting of NR beam level measurement and reporting when in LTE Idle/Inactive mode for Early Measurement Reporting at connection setup for FR1; endc-IdleInactiveMeasFR1-r16; Yes; N/A; Network cannot configure beam-level reporting to UE for EMR; Per UE. 6-2b; Support of beam level Early Measurement Reporting; Supporting of NR beam level measurement and reporting when in LTE Idle/Inactive mode for Early Measurement Reporting at connection setup for FR2; endc-IdleInactiveMeasFR2-r16; Yes; N/A; Network cannot configure beam-level reporting to UE for EMR; Per UE. Note 1: 6-2a and 6-2b are LTE features. |
| Summary of change: |  | In 4.3.6 Measurement parameters: - add paramenter 4.3.6.x1 endc-IdleInactiveMeasBeamReportFR1-r16; - add paramenter 4.3.6.x2 endc-IdleInactiveMeasBeamReportFR2-r16. Impact analysis Impacted 5G architecture options: (NG)EN-DC Impacted functionality: Early measurement reporting Inter-operability: There is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE can not report the support of beam level NR early measurement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR TS36.331 CR 4510 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

4.3.6	Measurement parameters

<Unchanged part>

#### 4.3.6.x1	endc-IdleInactiveMeasBeamReportFR1-r16

This field defines whether the UE supports performing eNB-configured beam level measurements for configured NR FR1 carrier(s) in RRC_IDLE and in RRC_INACTIVE (if the UE also indicates support of inactiveState-r15), including reporting them when requested by the eNB while resuming from RRC_IDLE/RRC_INACTIVE or in RRC_CONNECTED, as specified in TS 36.331 [5]. A UE supports this feature shall also support endc-IdleInactiveMeasFR1-r16.

#### 4.3.6.x2	endc-IdleInactiveMeasBeamReportFR2-r16

This field defines whether the UE supports performing eNB-configured beam level measurements for configured NR FR2 carrier(s) in RRC_IDLE and in RRC_INACTIVE (if the UE also indicates support of inactiveState-r15), including reporting them when requested by the eNB while resuming from RRC_IDLE/RRC_INACTIVE or in RRC_CONNECTED, as specified in TS 36.331 [5]. A UE supports this feature shall also support endc-IdleInactiveMeasFR2-r16.

END OF CHANGE