3GPP TSG-RAN WG2 #112e electronic                                    R2-2010370

Online: 2nd – 13th, November, 2020

Agenda Item:	8.10.3.2

Source:	CMCC

Title:	Discussion of UE location information assistant for cell selection and reselection in NTN

Document for:	Discussion, Decision

Introduction

In the last RAN2 meeting, there were some discussions on cell selection/reselection in email [1] and online. Agreements are as follows:

Agreements:

- Cell selection / reselection in NR is the baseline in NTN idle mode procedure.

- Satellite/HAPS ephemeris based cell selection and reselection should be defined for NTN (FFS what the term satellite/HAPS ephemeris actually means). FFS when this ephemeris based cell selection / reselection can be used. FFS whether UE location (and/or other information) based cell selection and reselection should be introduced for NTN

- The satellite ephemeris should be provided to UE, at least for Satellite/HAPS ephemeris based cell selection and reselection (FFS what the term satellite/HAPS ephemeris actually means).

Agreements via email - from offline 106:

- The existing cell reselection priority configuration can be taken as a baseline in NTN. FFS on any further enhancement.

In this contribution, we would like to provide some considerations on the UE location information assistant for cell selection and reselection in NTN.

Discussion

In the current spec. of R15, the specific procedure of UE mobility in Idle mode (i.e. cell selection and reselection) is provided in TS38.304 chapter 5. And the implementation of cell selection needs to fulfill S criterion with Srxlev > 0 AND Squal > 0, where Srxlev means cell selection RX level value (dB) and Squal is cell selection quality value (dB). While in NTN, the cell signal strength and quality measured by the UE may have no obvious difference due to the long distance and wide coverage.

Observation 1: In NTN, only with S criterion for cell selection may not help the UE to choose a better cell to camp on.

Moreover, cell reselection priorities of different frequencies are obtained through the network side. Common priority and corresponding parameters are provided in system information，while dedicated priority information could be acquired from the RRC release message. Hence we can see that cell reselection procedure is restricted by the network side.

Observation 2 Since UE's cell reselection behavior is based on cell reselection parameters configured by NW, large amount of dedicated signalling to indicate UE-specific priority will increase network overhead.

However, if the UE location information and satellite ephemeris are always available, the situation will be different. For cell selection, with the UE location information and satellite ephemeris under the condition of satisfying S criterion, the UE could choose a better cell to camp on. For cell reselection, with the UE location information and satellite ephemeris, UE could reduce measurement overhead, that is, some cells may not need to measure, which is particularly beneficial for LEO scenarios.

Observation 3: For both cell selection and reselection, it is very meaningful if the UE location information and satellite ephemeris are available at any time.

Some companies believe that utilizing UE location information in cell selection/reselection may cause serious energy consumption problems due to the need for GNSS to continuously track the UE location. And Near-far effect that a small difference in signal strength between two beams in a region of overlap in NTN systems could be solved via appropriate measurement thresholds configurations. However, compared to the coverage of the terrestrial network, the satellite coverage is very large, especially GEO scenarios. Moreover, the LEO scenarios with steerable beams could also ensure that the coverage does not change for a period of time. With the large coverage, the position of UE may not quickly change to near the edge of the cell, and the signal quality of current serving satellite remains good for a period of time, then GNSS continuous tracking UE location may be not required. At the other hand, because of the too small difference in signal strength changes in NTN cell, just adjusting the measurement threshold may not find a suitable cell to camp on. Therefore, UE location information is still beneficial for cell selection/reselection for UEs in NTN deployments. Moreover, in order to decrease energy consumption of the UEs, we could consider using GNSS to track the location of the UEs periodically instead of continuously tracking. And we could consider combining TA value, neighboring PCI and other information to design the conditions for starting GNSS to save electricity.

Observation 4: With the large coverage of satellite, the position of UE may not quickly change to near the edge of the cell, and the signal quality of current serving satellite remains good for a period of time, then GNSS continuous tracking UE location may be not required.

Proposal 1: For cell selection and reselection, in order to decrease energy consumption of the UEs, we could consider using GNSS to track the location of the UEs periodically instead of continuously tracking, with combining TA value, neighboring PCI and other information to design the conditions for starting GNSS to save electricity.

Considering that the ephemeris information size may be too large for network with many serving satellites. Moreover, the capacity of UE uSIM is usually 128kb which may be not sufficient for the pre-provisioning solution. Therefore, a method should be designed that can provide valid ephemeris information without excessive size.

Proposal 2: We suggest that RAN2 design a solution that can provide valid ephemeris information without excessive size.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on the cell selection and reselection for NTN and have the following observations and proposals:

Observation 1: In NTN, only with S criterion for cell selection may not help the UE to choose a better cell to camp on.

Observation 2 Since UE's cell reselection behavior is based on cell reselection parameters configured by NW, large amount of dedicated signalling to indicate UE-specific priority will increase network overhead.

Observation 3: For both cell selection and reselection, it is very meaningful if the UE location information and satellite ephemeris are available at any time.

Observation 4: With the large coverage of satellite, the position of UE may not quickly change to near the edge of the cell, and the signal quality of current serving satellite remains good for a period of time, then GNSS continuous tracking UE location may be not required.

Proposal 1: For cell selection and reselection, in order to decrease energy consumption of the UEs, we could consider using GNSS to track the location of the UEs periodically instead of continuously tracking, with combining TA value, neighboring PCI and other information to design the conditions for starting GNSS to save electricity.

Proposal 2: We suggest that RAN2 design a solution that can provide valid ephemeris information without excessive size.

Reference

[1] R2-2008187_Summary of [AT111-e][106][NTN] Idle mode issues (ZTE)