3GPP TSG-RAN WG2 Meeting #112e                  	R2-2010416

E-Meeting, 2 – 13 November, 2020

Source:	Xiaomi

Title:	Discussion of the paging cause support for MUSIM

Agenda Item:	8.3.4

Document for:	Discussion and Decision

# Introduction

During the RAN#86 meeting, MUSIM SID was approved in RP-193263 [1] and further updated in RP-201309 [2] described with the following objectives:

- Specify, if necessary, enhancement(s) to address the collision due to reception of paging when the UE is in IDLE/INACTIVE mode in both the networks associated with respective SIMs [RAN2] - RAT Concurrency: Network A can be NR. Network B can either be LTE or NR. - Applicable UE architecture: Single-Rx/Single-Tx. - Specify mechanism for UE to notify Network A of its switch from Network A (for MUSIM purpose) [RAN2]: - RAT Concurrency: Network A is NR. Network B can either be LTE or NR. - Applicable UE architecture: Single-Rx/Single-Tx, Dual-Rx/Single-Tx - Unless SA2 find an alternative solution or decides otherwise, specify mechanism for an incoming page to indicate to the UE whether the service is voLTE/VoNR. [ RAN2] - RAT Concurrency: Network A is either LTE or NR. Network B is either LTE or NR. - Applicable UE architecture: Single-Rx/Dual-Rx/Single-Tx

In this paper, we discuss the problem indicated in the objective 3 and make proposals accordingly.

# Discussion

According to SA2 study about MUSIM in SP-200091 [4], they identify several key issues and also propose solutions to each key issue in TR 23.761 [3] as listed in Table 1 below. We think the key issue 1, i.e. handling of Mobile Terminated service with Multi-USIM device, is closely related to objective 3 in [2] and several solutions with paging cause are proposed including solution 1, 9 and 24.

Table 1: Mapping of Solutions to Key Issues from TR 23.761 [3]

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| 1 | X |  |  |  |
| 2 | X |  |  |  |
| 3 | X |  |  |  |
| 4 |  |  | X |  |
| 5 |  |  | X |  |
| 6 |  |  | X |  |
| 7 | X | X |  |  |
| 8 | X |  |  |  |
| 9 | X |  |  |  |
| 10 | X |  |  |  |
| 11 | X |  |  |  |
| 12 | X | X |  |  |
| 13 | X |  |  |  |
| 14 |  | X |  |  |
| 15 |  | X |  |  |
| 16 |  | X |  |  |
| 17 |  | X | X |  |
| 18 |  | X |  |  |
| 19 |  | X |  |  |
| 20 |  | X |  |  |
| 21 |  | X |  |  |
| 22 |  |  | X |  |
| 23 | X |  | X |  |
| 24 | X | X |  |  |
| 25 | X |  | X |  |
| 26 | X | X |  |  |
| 27 |  | X | X |  |

In the LS [5] from SA2, RAN2 is asked by SA2 to confirm the feasibility and overhead of sending a Paging Cause in [Uu] Paging message for EPS and for 5GS.

According to the RAN2 MUSIM email discussion [6], 19 out of 26 participating companies believe that it is feasible to have paging cause on Uu for EPS and 5GS from RAN2 point of view.

In the SA2#141e meeting, a show of hands for introduing paging cause was held. The results are:

Option 1: No paging cause				8 supporters

Option 2: Multiple paging causes			19 supporters

Option 3: One paging cause			19 supporters

Option 4: Postpone decision to SA2#142E		10 supporters

It is quite clear that introducing paging cause also has majority support in SA2.

Proposal 1: Considering to specify paging cause for MUSIM in RAN2.

# Conclusion

This paper discusses the problem indicated in the objective 3 and proposes:

Proposal 1: Considering to specify paging cause for MUSIM in RAN2.

# References

[1] PR-193263 Support for Multi-SIM devices in Rel-17.

[2] RP-201309 Revision on WID Support for Multi-SIM devices.

[3] TR 23.761 V1.0.0 Study on system enablers for devices having multiple Universal Subscriber Identity Modules (USIM).

[4] SP-200091 Revised SID: Study on system enablers for multi-SIM devices.

[5] S2-2006037, LS on System support for Multi-USIM devices.

[6] R2-2009325, Multi-SIM summary.