3GPP TSG-RAN WG2 Meeting #112 Electronic	R2-2010445

2-13 November 2020

Agenda Item:	8.3.2

Source:	Charter Communications

Title:	Considerations for Paging Collision for Multi-SIM UEs

Document for:	Discussion, Decision

# Introduction

At RAN#86 meeting, a new work Item on multi-SIM devices [1] was approved with the following objectives:

- Specify, if necessary, enhancement(s) to address the collision due to reception of paging when the UE is in IDLE/INACTIVE in both the networks associated with respective SIMs [RAN2] - RAT Concurrency: Network A can be NR. Network B can either be LTE or NR. - Applicable UE architecture: Single-Rx/Single-Tx. - Mechanism for UE to notify Network A of its switch from Network A (for MUSIM purpose): - RAT Concurrency: Network A is NR. Network B can either be LTE or NR. - Applicable UE architecture: Single-Rx/Single-Tx, Dual-Rx/Single-Tx - Unless SA2 find an alternative solution or decides otherwise, specify mechanism for an incoming page to indicate to the UE whether the service is voLTE/VoNR [RAN2] - RAT Concurrency: Network A is either LTE or NR. Network B is either LTE or NR. - Applicable UE architecture: Single-Rx/Dual-Rx/Single-Tx UE SIMs may belong to same or different operators. USIM can be a physical SIM or eSIM. Coordination with relevant WGs, such as SA2, should be considered where relevant. Specification change should focus on NR side for objective 1.

The WI work would also address potential RAN impacts due to Multi-SIM devices resulting from SA2 SI outcome on Multi-SIM devices [2]. This contribution describes the potential solutions for above objectives.

# Paging Collision

UEs that are not in RRC connected state are required to periodically check, during a paging occasion (PO) of a paging frame (PF), if they are paged. The PF and PO that a UE expects to find a page, if any, are calculated based on these formulas:

SFN for the PF is determined by: (SFN + PF_offset) mod T = (T div N)*(UE_ID mod N)

Index (i_s), indicating the index of the PO is determined by: i_s = floor (UE_ID/N) mod Ns

Where UE_ID is:

UE_ID: IMSI mod 1024			(in the EPS)

UE_ID: 5G-TMSI mod 1024		(in the 5GS)

For a multi-SIM UE, the PF/PO associated with Network A are independently determined versus that of Network B. Therefore, there is a chance that the POs of the two networks that a multi-SIM UE is required to monitor might fully or partially overlap, making it difficult for the UE to check the POs of both networks. Objective 1 of the WID [1] considers the case where a multi-SIM UE has paging occasions about the same times such that the UE would be able to check the paging only on one network, but not both.

Above situation is defined as a paging collision case. For a multi-SIM UE, paging collision may appear or disappear between the same two networks while a UE goes through cell selection and reselection, due to updated RAN-related parameters from one cell to another.

## Possible solutions from SA2

In SA2 study for operation and support of multi-SIM devices, SA2 has presented questions about the following solutions for paging collision [3].

- Option 1: UE -requested 5G-GUTI reassignment for one USIM using the Mobility Registration Update (MRU). However, it should be noted the 5G-GUTI is systematically reassigned by the network during the Mobility Registration Update procedure (as of Rel-15) requires. Proposed for 5GS only. - Option 2: Changes related to the UE_ID (UE Identity Index) that is used for calculation of PF/PO only: - Option 2a Calculation of PF/PO by using an Alt UE_ID I. The UE ID sent in the paging message is not impacted by this Alt ID that is only used for PO/PF calculations Proposed for both EPS and 5GS. - Option 2b Calculation of PF/PO by using a UE_ID which is derived from IMSI+offset value. The offset value is negotiated between UE and MME. Proposed for EPS only. - Option 2c Calculation of PF/PO based on MUSIM Assistance Information which can carry either a paging policy selector in RAN or an Alternative ID (like in the solution above) or a pattern of availability (e.g. specific SFN Slots/ DRX cycles). - Option 3 Repeating paging in the RAN on consecutive POs for MUSIM devices. - Option 4 UE Implementation-based solution to address overlapping POs (like today) - Option 5 Access Stratum-based solution with scheduling gap.

## Implementation-based solutions

Solutions based on UE implementation may address a paging collision scenario to some extent. However, for a single-RX multi-SIM UE, there is no implementation that can resolve a paging collision scenario without missing some pages on either of the networks, hence scarifying network resources. Therefore, we propose that RAN2 should avoid relying on implementation-based solutions (Option 4) as the primary solution to address paging collision, and should focus on standardized solution(s) to resolve paging collision.

Proposal 1: RAN2 should avoid relying on implementation-based solutions (Option 4) as the primary solution to address paging collision, and should focus on standardized solution(s).

## Solutions based on reassigned or modified UE identity

Evaluating above formulas, there could be multiple solutions to avoid paging collision. For instance, if a paging collision is detected by a multi-SIM UE, the UE may request a new UE_ID, 5G-TMSI, in hope of the new ID resolves the collision. In fact, using the Mobility Registration Update (MRU) a UE obtains a new 5G-GUTI (and 5G-TMSI), which helps such a solution. Note that the 10 LSBs of the total 32 bits of the 5G-TMSI are used in the PF/PO formula. Therefore, a solution could be reallocation of a new 5G-TMSI, upon indication from the UE of paging collision (Option 1). This could possibly shift the UE’s PO, if the new 5G-TMSI has a favorable 10 LSBs, in one of the networks and resolve the collision.

While this solution seems to be a simple fix and requires a limited RAN2 spec change, it has the following drawbacks:

- Even if a new UE_ID resolves paging collision among the two networks, it is not guaranteed that it resolves a potential paging collision among all cells within a TA. Hence, after a cell reselection, for either of the networks, paging collision may happen.

- The new 5G-TMSI may result in no difference in PO (e.g. if the new 5G-TMSI has the same 10 LSBs as the old 5G-TMSI).

- A request to change UE_ID would likely trigger additional exchanges in core functions, hence a repeated 5G-GUTI reassignment is not favourable. Also depending on how long a 5G-GUTI reassignment takes, the reassignment limits the UE mobility until a new ID is assigned.

Observation 1: While reassignment of 5G-GUTI (Option1) is feasible, it has several drawbacks: it may not resolve a paging collision of the serving cell, the paging collision may persist among other cells in the TA, and it could possibly limit the UE mobility until a new ID is assigned.

Proposal 2: RAN2 to avoid solutions based on reassignment of 5G-GUTI (Option1) for resolving paging collision.

A similar solution is Option 2a that suggests to calculate PF/PO by using an alternative UE_ID. This solution requires UE assistance so that a favourable alternative UE_ID is chosen, otherwise whether the alternative UE_ID resolves the paging collision is up to chance. We believe this solution has similar disadvantages as listed above for Option 1.

Observation 2: Use of an alternative UE_ID for the calculation of PF/PO (Option 2a) may not resolve a paging collision of the serving cell and/or the paging collision may persist among other cells in the TA.

Proposal 3: RAN2 to avoid solutions based on alternative UE_ID (Option 2a) for resolving paging collision.

## RAN-based solutions

Given above discussion, we believe the paging collision issue is more efficient if is handled within RAN.

Due to dependence of the POs to parameters that are determined by RAN and core, the UE is in the best position to assess the possibility of paging collision. Each of the networks have partial knowledge about POs of the UE at the other network, and a network has to be informed about various parameters of the other network to determine if there is a paging collision for a given multi-SIM UE. Such exchange of information is unnecessary and inefficient, especially that has to be done for all multi-SIM UEs, regardless of an actual paging collision. Therefore, we suggest that RAN2 considers solutions that are triggered by the UE after it has determined that paging collision likely to happen. The signalling should preferably be NAS-level to avoid repeated signalling for multiple cells.

Proposal 4: RAN2 to consider solutions that are based on a trigger by a multi-SIM UE after it has determined that paging collision happens for the UE. The signalling should preferably be NAS-level to avoid repeated signalling for multiple cells.

In addition of identifying paging collision and triggering a solution, a multi-SIM UE can provide assistant information to the network. With such assistant info, the gNB may be able to resolve the collision more efficiently. For instance, Option 2c points to a pattern of availability that is provided by UE.

Among the solutions that RAN can perform to resolve paging collision are: providing pages in alternative POs, or repeating POs (Option 3) for the UE that experiences paging collision.

- The POs of a UE that experiences paging collision may be shifted to subsequent POs. Preferably this solution should avoid change of PO formula. Therefore, the UE should be informed of the shifted PO by means other than the PO formula.

- Additional PF/PO may be provided to UEs that have informed the network of paging collision. A gNB that supports this feature could advertise additional PF/PO in a new SIB. A multi-SIM UE that detects paging collision looks for the additional advertised PO in addition to the original PO.

- POs may be repeated so that if a multi-SIM UE misses a PO due to paging collision, the UE has the opportunity to check the additional POs. Note that such paging repetition was proposed and adopted in NR-U WI and may be used for multi-SIM UEs. However, PO repetition across all cells within a TA, and/or multiple repetition of POs is wasteful and should be avoided.

Proposal 5: RAN2 to consider solutions where the gNB resolves paging collision by providing additional PO or repetition of POs, considering efficiency and effectiveness trade-off.

# Conclusion

Proposal 1: RAN2 should avoid relying on implementation-based solutions (Option 4) as the primary solution to address paging collision, and should focus on standardized solution(s).

Observation 1: While reassignment of 5G-GUTI (Option1) is feasible, it has several drawbacks: it may not resolve a paging collision of the serving cell, the paging collision may persist among other cells in the TA, and it could possibly limit the UE mobility until a new ID is assigned.

Proposal 2: RAN2 to avoid solutions based on reassignment of 5G-GUTI (Option1) for resolving paging collision.

Observation 2: Use of an alternative UE_ID for the calculation of PF/PO (Option 2a) may not resolve a paging collision of the serving cell and/or the paging collision may persist among other cells in the TA.

Proposal 3: RAN2 to avoid solutions based on alternative UE_ID (Option 2a) for resolving paging collision.

Proposal 4: RAN2 to consider solutions that are based on a trigger by a multi-SIM UE after it has determined that paging collision happens for the UE. The signalling should preferably be NAS-level to avoid repeated signalling for multiple cells.

Proposal 5: RAN2 to consider solutions where the gNB resolves paging collision by providing additional PO or repetition of POs, considering efficiency and effectiveness trade-off.

# References

- RP-201309	Support for Multi-SIM devices for LTE/NR

- SP-190248	Revised SID: Study on system enablers for multi-SIM devices

- TR 23.761 	Study on system enablers for devices having MUSIM