3GPP TSG-RAN WG2 Meeting#112-e Meeting		                     R2-2010460

Online, November 2020

Source: 	MediaTek, Inc.

Title: 	Measurement before radio link failure

Agenda item:	9.1.2

Document for:     Discussion and Decision

# Introduction

In WID on Rel-17 enhancements for NB-IoT [1], one objective to reduce the time taken to RRC re-establishment is described as:

- Specify signaling for neighbor cell measurements and corresponding measurement triggering before RLF, to reduce the time taken to RRC re-establishment to another cell, without defining specific gaps. [NB-IoT] [RAN2, RAN4].

Measurement before radio link failure was discussed in RAN2#111-e with following agreements:

- Study current RLF procedure to understand the time taken to select the cell for access. Start and end points FFS.

- Support neighbour cell measurements in RRC_CONNECTED, at least for intra-frequency.

    - FFS inter-frequency

    - FFS whether measurements are done on the anchor carrier

    - FFS how neighbour cell measurement is triggered

    - FFS how to perform neighbour cell measurements

- Working assumption: Neighbour cell measurement results are not reported to the network in RRC_CONNECTED.

    - FFS whether and when other information can be sent

This contribution further discusses the above FFS items.

# Discussion

The problem of intra-frequency deployment is the intra-frequency interference. The signal power of intra-frequency neighbour cell A becomes the interference of intra-frequency neighbour cell B. Inter-frequency deployment can reduce interference and improve SINR, hence UE would have better performance with it. It is necessary to support neighbour cell measurement for inter-frequency before RLF.

Although the WID specified that no measurement gap would be defined, there are still plenty of opportunities for UE to perform inter-frequency neighbour cell measurement. There are a lot of sub-frames that UE is not monitoring or transmitting, such as:

- DL gap

- UL gap

- multiple sub-frames not monitored because of C-DRX

- multiple sub-frames between NDPCCH and NPDSCH/NPUSCH

- multiple sub-frames between USSs

- scheduled NPUSH sub-frames but collide with NPRACH

- NRACH sub-frames collide with different CE level

- The sub-frames after NPDCCH/NPDSCH early decoding

But there still problems to utilize these opportunities:

- Some opportunities rely on the specific configuration, like the DL gap, UL gap, C-DRX.

- The opportunity may be too short to perform inter-frequency neighbour cell measurement, as UE takes some time to switch frequency and detect cell.

- There may no inter-frequency neighbour cell’s NPSS/NSSS sub-frame during the short opportunity.

We can see that the inter-frequency neighbour cell measurement is feasible even when no measurement gap is configured.

RAN4 should define the detailed requirement of inter-frequency neighbour cell measurement for NB-IoT in RRC_CONNECTED mode.

Proposal 1: support inter-frequency neighbour cell measurement before RLF.

When UE perform neighbour cell measurement, UE will synchronize NPSS and NSSS on the anchor carrier to detect cell before measure NRS. The non-anchor carrier does not contain NPSS and NSSS, therefore the measurement should be done on the anchor carrier.

Proposal 2: Measurement should be done on the anchor carrier.

To reduce the time taken for RRC re-establishment as WID expected, the timing of triggering measurement should be before RRC re-establishment at least. The following 3 options can be considered:

Option 1: Same as early detection of physical layer problems, start measurement based on a certain threshold, before the detection of physical layer problems, i.e. out-of-sync indication.

Option 2: start measurement when received out-of-sync indication.

Option 3: start measurement when T310 started.

For intra-frequency measurement, which can be easily done together with normal serving cell NPDCCH/NPDSCH reception in RRC_CONNECTED mode, no need to be performed as early as option 1. For power consumption consideration, option 3 is suitable for intra-frequency measurement.

For inter-frequency measurement, which needs to seek available sub-frames, needs much more time than intra-frequency measurement. It should be started as early as possible, therefore option 1 is more suitable.

Proposal 3: If the inter-frequency measurement is not supported, start intra-frequency measurement when T310 started.

Proposal 4: If the inter-frequency measurement is supported, start intra-frequency and inter-frequency measurement based on a certain threshold, before receiving an out-of-sync indication.

Regarding the FSS, how to perform neighbour cell measurement, it is out of RAN2’s scope, it should be discussed in RAN4.

Proposal 5: Discuss how to perform neighbour cell measurement in RAN4.

The measurement result is not useful for the network, and because of the up-coming RLF, the radio condition is not good,  there is no need to send any information to the network.

Proposal 6: No information needs to be sent to the network.

# Conclusion

Proposal 1: support inter-frequency neighbour cell measurement before RLF.

Proposal 2: Measurement should be done on the anchor carrier.

Proposal 3: If the inter-frequency measurement is not supported, start intra-frequency measurement when T310 started.

Proposal 4: If the inter-frequency measurement is supported, start intra-frequency and inter-frequency measurement based on a certain threshold, before receiving an out-of-sync indication.

Proposal 5: Discuss how to perform neighbour cell measurement in RAN4.

Proposal 6: No information that needs to be sent to the network.

# Reference

RP-201064 WID: Additional enhancements for NB-IoT and LTE-MTC