# 3GPP TSG RAN WG2 Meeting #112-e						    R2-2010488

# E-Conference, 2nd – 13th Nov. 2020

# Title:	[DRAFT] Reply LS on Cell Configuration within TA/RA to Support Allowed NSSAI

# Release:	Release 17

# Work Item:	FS_NR_Slice, FS_eNS_Ph2

Source:                 Qualcomm Incorporated [to be RAN2]

To:                         SA2, RAN3, CT1

Cc:

Contact Person:

Name:	Peng Cheng

Tel. Number:

E-mail Address:	chengp@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN2 thanks SA2 for the LS on Cell Configuration within TA/RA to Support Allowed NSSAI. RAN2 agrees with SA2’s assumption that all S-NSSAIs in the Allowed NSSAI are supported within the TA and also in all TAs of the RA.

For the specific questions asked by SA2, RAN2 would like to provide the following feedback:

Q1: In Rel-15 and 16, is it expected that each cell in the tracking area supports the same S-NSSAI(s)? (or, said otherwise, do all cells advertising the same TAC support the same set of S-NSSAIs?).

Clause 16.3.1 of TS 38.300 states that “it is assumed that the slice availability does not change within the UE’s registration area”, and it follows that the same assumption applies to a tracking area. Therefore, RAN2 believes that a cell broadcasting TAI X shall normally be able to provide appropriate slice resources for all slices associated with TAI X. Whether this implies that the resources must be owned by that cell requires further discussion since no specific normative statement exists to that effect.

The same clause also states that “Admission or rejection of access to a slice may depend by factors such as support for the slice, availability of resources, support of the requested service by NG-RAN”. RAN2 thinks that, due to resource shortage, it is possible that a slice may not be available in a cell of TAI X even if declared in the list associated with TAI X by the respective RAN node.

Q2a: Can RAN WGs and CT1 explain if it can happen that a UE, e.g. due to local radio conditions, can only use a cell in the TA where not all S-NSSAIs are present in the Allowed NSSAI it received (and that the TA supports), and can RAN WGs and CT1 explain how it is handled today in rel-15/16?

Assuming that all RAN nodes involved are within the same tracking area, this could happen in rel15/16 due to e.g. resource shortage. Resource shortage can impact handover or context / session establishment as well as normal operation of a previously admitted PDU session. In such case, HO/redirection may be required when S-NSSAI associated with the arriving traffic is not available in current cell.

Q2b: If an S-NSSAI can be rejected depending on which cell the UE camps on even though it is supported in the TA, for the reason that it is not supported in the cell, is there in rel-15/16 a CT1 error code to handle this case (i.e. can a S-NSSAI be rejected, with a suitable cause code, depending on which cell of the TA the UE camps on, even though this S-NSSAI is known to be supported in the TA, for the reason that this S-NSSAI is actually not supported in the cell of the TA)? Is there any provisions in the RAN or CT1 specifications to handle this case?

From RAN2 point of view, except for the resource congestion scenario, the case described above should not normally happen in rel15/16, since the slice support is TA-homogenous, and involved nodes exchange information on slice support via configuration exchange procedures. If support does change, the configuration should be updated.

2. Actions:

To SA WG2, RAN WG3 and CT WG1.

ACTION: 	RAN2 kindly asks SA2, RAN3 and CT1 to take the above information into account.

3. Date of Next RAN2 Meetings:

TSG-RAN WG2 Meeting #112e	2020-11-02 to 2020-11-13	E-Meeting

TSG-RAN WG2 Meeting #113e	2021-03-01 to 2021-03-05	E-Meeting