3GPP TSG-RAN WG2 Meeting #112e	R2-2010510

# Electronic Meeting, Nov 2-13, 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4520 | rev | - | Current version: | 16.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RRC segmentation for handover and dual connectivity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-10-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RRC segmentation was introduced in Release 16 to handle sending of large UE Radio Configuration Information messages in UL and UE configuration messages in DL that would exceed the PDCP size limit. In case of handover, the target eNB generates the UE configuration message (RRCReconfiguration) that is sent to the source eNB and then to the UE. However, currently there is no information sent from the source eNB to the target eNB on whether RRC segmentation can be used or not. Correspondingly, for dual connectivity, the SN generates the SCG configuration that is sent to the MN and then to the UE, but there is no information provided from MN to SN on whether RRC segmentation can be used or not. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Information about RRC segmentation is added to the HandoverPreparationInformation message and to the CG-ConfigInfo message that the target eNB and the SN can use to determine if the configuration message may exceed the PDCP size limit. Impact Analysis Impacted 5G architecture options:(NG)EN-DC, NE-DC Impacted functionality: Handover preparation, SN addition Inter-operability: There are no interoperability issues foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Target eNBs and SN may generate configuration messages too large to fit within a single PDCP packet, which hence cannot be sent to the UE by the source and MN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.2, 10.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 10.2.2	Message definitions

*** Unchanged text not included ***

#### –	HandoverPreparationInformation

This message is used to transfer the E-UTRA RRC information used by the target eNB or target ng-eNB during handover preparation or UE context retrieval, e.g. in case of resume or re-establishment, including UE capability information.

Direction: source eNB/ source RAN to target eNB or target ng-eNB

HandoverPreparationInformation message

-- ASN1START

HandoverPreparationInformation ::=	SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE{

handoverPreparationInformation-r8	HandoverPreparationInformation-r8-IEs,

spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

HandoverPreparationInformation-r8-IEs ::= SEQUENCE {

ue-RadioAccessCapabilityInfo		UE-CapabilityRAT-ContainerList,

as-Config							AS-Config					OPTIONAL,		-- Cond HO

rrm-Config							RRM-Config					OPTIONAL,

as-Context							AS-Context				OPTIONAL,		-- Cond HO

nonCriticalExtension				HandoverPreparationInformation-v920-IEs		OPTIONAL

}

HandoverPreparationInformation-v920-IEs	::= SEQUENCE {

ue-ConfigRelease-r9					ENUMERATED {

rel9, rel10, rel11, rel12, v10j0, v11e0,

v1280, rel13, ..., rel14, rel15, rel16}		OPTIONAL,	-- Cond HO2

nonCriticalExtension				HandoverPreparationInformation-v9d0-IEs		OPTIONAL

}

HandoverPreparationInformation-v9d0-IEs	::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING (CONTAINING HandoverPreparationInformation-v9j0-IEs)	OPTIONAL,

nonCriticalExtension				HandoverPreparationInformation-v9e0-IEs			OPTIONAL

}

-- Late non-critical extensions:

HandoverPreparationInformation-v9j0-IEs ::= SEQUENCE {

-- Following field is only for pre REL-10 late non-critical extensions

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

nonCriticalExtension				HandoverPreparationInformation-v10j0-IEs		OPTIONAL

}

HandoverPreparationInformation-v10j0-IEs ::= SEQUENCE {

as-Config-v10j0						AS-Config-v10j0			OPTIONAL,

nonCriticalExtension				HandoverPreparationInformation-v10x0-IEs		OPTIONAL

}

HandoverPreparationInformation-v10x0-IEs ::= SEQUENCE {

-- Following field is only for late non-critical extensions from REL-10 to REL-12

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

nonCriticalExtension				HandoverPreparationInformation-v13c0-IEs		OPTIONAL

}

HandoverPreparationInformation-v13c0-IEs ::= SEQUENCE {

as-Config-v13c0						AS-Config-v13c0			OPTIONAL,

-- Following field is only for late non-critical extensions from REL-13

nonCriticalExtension				SEQUENCE {}					OPTIONAL

}

-- Regular non-critical extensions:

HandoverPreparationInformation-v9e0-IEs	::= SEQUENCE {

as-Config-v9e0						AS-Config-v9e0					OPTIONAL,	-- Cond HO2

nonCriticalExtension				HandoverPreparationInformation-v1130-IEs		OPTIONAL

}

HandoverPreparationInformation-v1130-IEs	::= SEQUENCE {

as-Context-v1130					AS-Context-v1130				OPTIONAL,	-- Cond HO2

nonCriticalExtension				HandoverPreparationInformation-v1250-IEs						OPTIONAL

}

HandoverPreparationInformation-v1250-IEs ::= SEQUENCE {

ue-SupportedEARFCN-r12				ARFCN-ValueEUTRA-r9				OPTIONAL,	-- Cond HO3

as-Config-v1250					AS-Config-v1250				OPTIONAL,	-- Cond HO2

nonCriticalExtension				HandoverPreparationInformation-v1320-IEs						OPTIONAL

}

HandoverPreparationInformation-v1320-IEs ::= SEQUENCE {

as-Config-v1320						AS-Config-v1320					OPTIONAL,	-- Cond HO2

as-Context-v1320					AS-Context-v1320				OPTIONAL,	-- Cond HO2

nonCriticalExtension				HandoverPreparationInformation-v1430-IEs						OPTIONAL

}

HandoverPreparationInformation-v1430-IEs ::= SEQUENCE {

as-Config-v1430					AS-Config-v1430						OPTIONAL,	-- Cond HO2

makeBeforeBreakReq-r14			ENUMERATED {true}				OPTIONAL,	-- Cond HO2

nonCriticalExtension			HandoverPreparationInformation-v1530-IEs			OPTIONAL

}

HandoverPreparationInformation-v1530-IEs ::= SEQUENCE {

ran-NotificationAreaInfo-r15		RAN-NotificationAreaInfo-r15			OPTIONAL,

nonCriticalExtension				HandoverPreparationInformation-v1540-IEs							OPTIONAL

}

HandoverPreparationInformation-v1540-IEs ::= SEQUENCE {

sourceRB-ConfigIntra5GC-r15		OCTET STRING						OPTIONAL,	--Cond HO4

nonCriticalExtension				HandoverPreparationInformation-v1610-IEs	OPTIONAL

}

HandoverPreparationInformation-v1610-IEs ::= SEQUENCE {

as-Context-v1610			AS-Context-v1610						OPTIONAL,	--Cond HO5

nonCriticalExtension		HandoverPreparationInformation-v1620-IEs	OPTIONAL

}

HandoverPreparationInformation-v1620-IEs ::= SEQUENCE {

as-Context-v1620			AS-Context-v1620						OPTIONAL, 	--Cond HO2

nonCriticalExtension		HandoverPreparationInformation-v16xy-IEs								OPTIONAL

}

HandoverPreparationInformation-v16xy-IEs ::= SEQUENCE {

as-Context-v16xy			AS-Context-v16xy						OPTIONAL,

nonCriticalExtension		SEQUENCE {}								OPTIONAL

}

-- ASN1STOP

HandoverPreparationInformation field descriptions

as-Config The radio resource configuration. Applicable in case of intra-E-UTRA handover. If the target receives an incomplete MeasConfig and/or RadioResourceConfigDedicated in the as-Config, the target eNB may decide to apply the full configuration option based on the ue-ConfigRelease.

as-Context Local E-UTRAN context required by the target eNB.

makeBeforeBreakReq To request the target eNB to add the makeBeforeBreak indication in the mobilityControlInfo in case of intra-frequency handover.

rrm-Config Local E-UTRAN context used depending on the target node's implementation, which is mainly used for the RRM purpose. May also be provided at inter-RAT handover from NR.

sourceRB-ConfigIntra5GC NR radio bearer config used at intra5GC handover, as defined by RadioBearerConfig IE in TS 38.331 [82].

ue-ConfigRelease Indicates the RRC protocol release or version applicable for the current UE configuration. This could be used by target eNB to decide if the full configuration approach should be used. If this field is not present, the target assumes that the current UE configuration is based on the release 8 version of RRC protocol. NOTE 1.

ue-RadioAccessCapabilityInfo For E-UTRA radio access capabilities, it is up to E-UTRA how the backward compatibility among supportedBandCombinationReduced, supportedBandCombination and supportedBandCombinationAdd is ensured. If supportedBandCombinationReduced and supportedBandCombination/supportedBandCombinationAdd are included into ueCapabilityRAT-Container, it can be assumed that the value of fields, requestedBands, reducedIntNonContCombRequested and requestedCCsXL are consistend with all supported band combination fields. NOTE 2

ue-SupportedEARFCN Includes UE supported EARFCN of the handover target E-UTRA cell if the target E-UTRA cell belongs to multiple frequency bands.

NOTE 1:	The source typically sets the ue-ConfigRelease to the release corresponding with the current dedicated radio configuration. The source may however also consider the common radio resource configuration e.g. in case interoperability problems would appear if the UE temporary continues extensions of this part of the configuration in a target PCell not supporting them.

NOTE 2:	The following table indicates per source RAT whether RAT capabilities are included or not.

| Source RAT | E-UTRA capabilites | UTRA capabilities | GERAN capabilities | MR DC capabilities | NR capabilities |
| --- | --- | --- | --- | --- | --- |
| UTRAN | Included | May be included, ignored by eNB if received | May be included | Excluded | Excluded |
| GERAN CS | Excluded | May be included, ignored by eNB if received | Included | Excluded | Excluded |
| GERAN PS | Excluded | May be included, ignored by eNB if received | Included | Excluded | Excluded |
| E-UTRAN | May be included if UE Radio Capability ID as specified in 23.502 [102] is used for the UE. Included otherwise. | May be included | May be included | May be included | May be included |
| NR | May be included if UE Radio Capability ID as specified in 23.502 [102] is used for the UE. Included otherwise. | Excluded | Excluded | May be included | May be included |

| Conditional presence | Explanation |
| --- | --- |
| HO | The field is mandatory present in case of handover within E-UTRA; otherwise the field is not present. |
| HO2 | The field is optional present in case of handover within E-UTRA; otherwise the field is not present. |
| HO3 | The field is optional present in case of handover from GERAN to E-UTRA, otherwise the field is not present. |
| HO4 | The field is mandatory present in case of handover within E-UTRA/5GC and optional present in case of handover from NR to E-UTRA/5GC; otherwise the field is not present. |
| HO5 | The field is optional present in case of handover within E-UTRA, or handover from NR to E-UTRA; otherwise the field is not present. |

*** Unchanged text not included ***

#### –	SCG-ConfigInfo

This message is used by MeNB to request the SeNB to perform certain actions e.g. to establish, modify or release an SCG, and it may include additional information e.g. to assist the SeNB with assigning the SCG configuration.

Direction: Master eNB to secondary eNB

SCG-ConfigInfo message

-- ASN1START

SCG-ConfigInfo-r12 ::=					SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE{

scg-ConfigInfo-r12					SCG-ConfigInfo-r12-IEs,

spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

SCG-ConfigInfo-r12-IEs ::=			SEQUENCE {

radioResourceConfigDedMCG-r12	RadioResourceConfigDedicated		OPTIONAL,

sCellToAddModListMCG-r12		SCellToAddModList-r10				OPTIONAL,

measGapConfig-r12				MeasGapConfig						OPTIONAL,

powerCoordinationInfo-r12		PowerCoordinationInfo-r12			OPTIONAL,

scg-RadioConfig-r12				SCG-ConfigPartSCG-r12				OPTIONAL,

eutra-CapabilityInfo-r12		OCTET STRING (CONTAINING UECapabilityInformation)	OPTIONAL,

scg-ConfigRestrictInfo-r12		SCG-ConfigRestrictInfo-r12			OPTIONAL,

mbmsInterestIndication-r12		OCTET STRING (CONTAINING

MBMSInterestIndication-r11)		OPTIONAL,

measResultServCellListSCG-r12	MeasResultServCellListSCG-r12		OPTIONAL,

drb-ToAddModListSCG-r12			DRB-InfoListSCG-r12					OPTIONAL,

drb-ToReleaseListSCG-r12		DRB-ToReleaseList					OPTIONAL,

sCellToAddModListSCG-r12		SCellToAddModListSCG-r12			OPTIONAL,

sCellToReleaseListSCG-r12		SCellToReleaseList-r10				OPTIONAL,

p-Max-r12							P-Max								OPTIONAL,

nonCriticalExtension			SCG-ConfigInfo-v1310-IEs			OPTIONAL

}

SCG-ConfigInfo-v1310-IEs ::=		SEQUENCE {

measResultSSTD-r13				MeasResultSSTD-r13					OPTIONAL,

sCellToAddModListMCG-Ext-r13		SCellToAddModListExt-r13			OPTIONAL,

measResultServCellListSCG-Ext-r13	MeasResultServCellListSCG-Ext-r13	OPTIONAL,

sCellToAddModListSCG-Ext-r13		SCellToAddModListSCG-Ext-r13			OPTIONAL,

sCellToReleaseListSCG-Ext-r13	SCellToReleaseListExt-r13			OPTIONAL,

nonCriticalExtension			SCG-ConfigInfo-v1330-IEs			OPTIONAL

}

SCG-ConfigInfo-v1330-IEs ::=		SEQUENCE {

measResultListRSSI-SCG-r13		MeasResultListRSSI-SCG-r13			OPTIONAL,

nonCriticalExtension			SCG-ConfigInfo-v1430-IEs							OPTIONAL

}

SCG-ConfigInfo-v1430-IEs ::=		SEQUENCE {

makeBeforeBreakSCG-Req-r14		ENUMERATED {true}					OPTIONAL,

measGapConfigPerCC-List		MeasGapConfigPerCC-List-r14			OPTIONAL,

nonCriticalExtension			SCG-ConfigInfo-v1530-IEs					OPTIONAL

}

SCG-ConfigInfo-v1530-IEs ::=		SEQUENCE {

drb-ToAddModListSCG-r15			DRB-InfoListSCG-r15					OPTIONAL,

drb-ToReleaseListSCG-r15		DRB-ToReleaseList-r15				OPTIONAL,

nonCriticalExtension			SCG-ConfigInfo-v16xy-IEs							OPTIONAL

}

SCG-ConfigInfo-v16xy-IEs ::=		SEQUENCE {

mn-rrcSegAllowed-r16			ENUMERATED {true}					OPTIONAL,

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

DRB-InfoListSCG-r12 ::=				SEQUENCE (SIZE (1..maxDRB)) OF DRB-InfoSCG-r12

DRB-InfoListSCG-r15 ::=				SEQUENCE (SIZE (1..maxDRB-r15)) OF DRB-InfoSCG-r12

DRB-InfoSCG-r12 ::=				SEQUENCE {

eps-BearerIdentity-r12			INTEGER (0..15)				OPTIONAL,	-- Cond DRB-Setup

drb-Identity-r12				DRB-Identity,

drb-Type-r12					ENUMERATED {split, scg}		OPTIONAL,	-- Cond DRB-Setup

...

}

SCellToAddModListSCG-r12 ::=	SEQUENCE (SIZE (1..maxSCell-r10)) OF Cell-ToAddMod-r12

SCellToAddModListSCG-Ext-r13 ::=	SEQUENCE (SIZE (1..maxSCell-r13)) OF Cell-ToAddMod-r12

Cell-ToAddMod-r12 ::=				SEQUENCE {

sCellIndex-r12						SCellIndex-r10,

cellIdentification-r12				SEQUENCE {

physCellId-r12						PhysCellId,

dl-CarrierFreq-r12					ARFCN-ValueEUTRA-r9

}																OPTIONAL,	-- Cond SCellAdd

measResultCellToAdd-r12				SEQUENCE {

rsrpResult-r12						RSRP-Range,

rsrqResult-r12						RSRQ-Range

}																OPTIONAL,	-- Cond SCellAdd2

...,

[[		sCellIndex-r13					SCellIndex-r13				OPTIONAL,

measResultCellToAdd-v1310			SEQUENCE {

rs-sinr-Result-r13					RS-SINR-Range-r13

}															OPTIONAL	-- Cond SCellAdd2

]]

}

MeasResultServCellListSCG-r12 ::=	SEQUENCE (SIZE (1..maxServCell-r10)) OF MeasResultServCellSCG-r12

MeasResultServCellListSCG-Ext-r13 ::=	SEQUENCE (SIZE (1..maxServCell-r13)) OF MeasResultServCellSCG-r12

MeasResultServCellSCG-r12 ::=			SEQUENCE {

servCellId-r12						ServCellIndex-r10,

measResultSCell-r12					SEQUENCE {

rsrpResultSCell-r12					RSRP-Range,

rsrqResultSCell-r12					RSRQ-Range

},

...,

[[		servCellId-r13						ServCellIndex-r13		OPTIONAL,

measResultSCell-v1310				SEQUENCE {

rs-sinr-ResultSCell-r13				RS-SINR-Range-r13

}															OPTIONAL

]]

}

MeasResultListRSSI-SCG-r13 ::=	SEQUENCE (SIZE (1..maxServCell-r13)) OF MeasResultRSSI-SCG-r13

MeasResultRSSI-SCG-r13 ::=			SEQUENCE {

servCellId-r13						ServCellIndex-r13,

measResultForRSSI-r13				MeasResultForRSSI-r13

}

SCG-ConfigRestrictInfo-r12 ::=		SEQUENCE {

maxSCH-TB-BitsDL-r12				INTEGER (1..100),

maxSCH-TB-BitsUL-r12				INTEGER (1..100)

}

-- ASN1STOP

SCG-ConfigInfo field descriptions

drb-ToAddModListSCG Includes DRBs the SeNB is requested to establish or modify (DRB type change).

drb-ToReleaseListSCG Includes DRBs the SeNB is requested to release.

makeBeforeBreakSCG-Req To request the target eNB to add the makeBeforeBreakSCG indication in the mobilityControlInfoSCG in case of intra-frequency SCG change.

maxSCH-TB-BitsXL Indicates the maximum DL-SCH/UL-SCH TB bits that may be scheduled in a TTI. Specified as a percentage of the value defined for the applicable UE category.

measGapConfig Includes the current measurement gap configuration.

measResultListRSSI-SCG Includes RSSI measurement results of SCG (serving) cells

measResultSSTD Includes measurement results of UE SFN and Subframe Timing Difference between the PCell and the PSCell.

measResultServCellListSCG Includes measurement results of SCG (serving) cells.

mn-rrcSegAllowed Indicates if MN supports sending segmented RRC messages in DL to the UE. If both MN and UE allow RRC segmentation, then SN may generate a RRCReconfiguration message exceeding the PDCP size limit, max 45 Kbytes.

radioResourceConfigDedMCG Includes the current dedicated MCG radio resource configuration.

sCellIndex If sCellIndex-r13 is present, sCellIndex-r12 shall be ignored.

sCellToAddModListMCG, sCellToAddModListMCG-Ext Includes the current MCG SCell configuration. Field sCellToAddModListMCG is used to add the first 4 SCells with sCellIndex-r10 while sCellToAddModListMCG-Ext is used to add the rest.

sCellToAddModListSCG, sCellToAddModListSCG-Ext Includes SCG cells the SeNB is requested to establish. Measurement results may be provided for these cells. Field sCellToAddModListSCG is used to add the first 4 SCells with sCellIndex-r12 while sCellToAddModListSCG-Ext is used to add the rest.

sCellToReleaseListSCG, sCellToReleaseListSCG-Ext Includes SCG cells the SeNB is requested to release.

scg-RadioConfig Includes the current dedicated SCG configuration.

scg-ConfigRestrictInfo Includes fields for which MeNB explictly indicates the restriction to be observed by SeNB.

servCellId If servCellId-r13 is present, servCellId-r12 shall be ignored.

p-Max Cell specific value i.e. as broadcast by PCell.

| Conditional presence | Explanation |
| --- | --- |
| DRB-Setup | The field is mandatory present in case DRB establishment is requested; otherwise the field is not present. |
| SCellAdd | The field is mandatory present in case SCG cell establishment is requested; otherwise the field is not present. |
| SCellAdd2 | The field is optional present in case SCG cell establishment is requested; otherwise the field is not present. |

*** Unchanged text not included ***

Next change

## 10.3	Inter-node RRC information element definitions

*** Unchanged text not included ***

#### –	AS-Context

The IE AS-Context is used to transfer local E-UTRAN context required by the target eNB.

AS-Context information element

-- ASN1START

AS-Context ::=							SEQUENCE {

reestablishmentInfo						ReestablishmentInfo			OPTIONAL	-- Cond HO

}

AS-Context-v1130 ::=					SEQUENCE {

idc-Indication-r11						OCTET STRING (CONTAINING

InDeviceCoexIndication-r11)	OPTIONAL,	-- Cond HO2

mbmsInterestIndication-r11				OCTET STRING (CONTAINING

MBMSInterestIndication-r11)	OPTIONAL,	-- Cond HO2

powerPrefIndication-r11					OCTET STRING (CONTAINING

UEAssistanceInformation-r11)	OPTIONAL,	-- Cond HO2

...,

[[	sidelinkUEInformation-r12				OCTET STRING (CONTAINING

SidelinkUEInformation-r12)	OPTIONAL	-- Cond HO2

]],

[[	sourceContextEN-DC-r15				OCTET STRING					OPTIONAL	-- Cond HO2

]],

[[	selectedbandCombinationInfoEN-DC-v1540		OCTET STRING			OPTIONAL	-- Cond HO2

]]

}

AS-Context-v1320 ::=					SEQUENCE {

wlanConnectionStatusReport-r13			OCTET STRING (CONTAINING

WLANConnectionStatusReport-r13)	OPTIONAL	-- Cond HO2

}

AS-Context-v1610 ::=					SEQUENCE {

sidelinkUEInformationNR-r16				OCTET STRING	OPTIONAL, -- Cond HO3

ueAssistanceInformationNR-r16			OCTET STRING	OPTIONAL, -- Cond HO3

configRestrictInfoDAPS-r16				ConfigRestrictInfoDAPS-r16		OPTIONAL -- Cond HO2

}

AS-Context-v1620 ::=					SEQUENCE {

ueAssistanceInformationNR-SCG-r16		OCTET STRING	OPTIONAL  -- Cond HO2

}

AS-Context-v16xy ::=					SEQUENCE {

source-rrcSegAllowed-r16				ENUMERATED {true}			OPTIONAL

}

ConfigRestrictInfoDAPS-r16 ::=		SEQUENCE {

maxSCH-TB-BitsDL-r16					INTEGER (1..100)			OPTIONAL,	-- Cond HO2

maxSCH-TB-BitsUL-r16					INTEGER (1..100)			OPTIONAL	-- Cond HO2

}

-- ASN1STOP

AS-Context field descriptions

idc-Indication Including information used for handling the IDC problems.

maxSCH-TB-BitsXL Indicates the maximum DL-SCH/UL-SCH TB bits that may be scheduled in a TTI during DAPS HO. Specified as a percentage of the value defined for the applicable UE category.

reestablishmentInfo Including information needed for the RRC connection re-establishment.

sourceContextEN-DC (NG)EN-DC related context information, in particular regarding the UE capability coordination, as defined by the ConfigRestrictInfoSCG IE specified in TS 38.331 [82].

selectedBandCombinationInfoEN-DC Including the BandCombinationInfoSN IE specified in TS 38.331 [82]. See NOTE 1.

sidelinkUEInformationNR Including sidelink UE information as defined by the SidelinkUEInformationNR message specified in TS 38.331 [82].

ueAssistanceInformationNR Including sidelink UE assistance information as defined by the UEAssistanceInformation message specified in TS 38.331 [82].

ueAssistanceInformationNR-SCG Includes for each UE assistance feature associated with the NR SCG as specified in TS 38.331 [82], the information last reported by the UE in the NR UEAssistanceInformation message for the NR SCG, if any.

source-rrcSegAllowed Indicates if source eNB supports sending segmented RRC messages in DL to the UE. If both allow eNB and UE support RRC segmentation, then target eNB/RAN may generate a RRCReconfiguration message exceeding the PDCP size limit, max 45 Kbytes.

| Conditional presence | Explanation |
| --- | --- |
| HO | The field is mandatory present in case of handover within E-UTRA; otherwise the field is not present. |
| HO2 | The field is optional present in case of handover within E-UTRA; otherwise the field is not present. |
| HO3 | The field is optional present in case of handover within E-UTRA, or handover from NR to E-UTRA; otherwise the field is not present. |

NOTE 1:	If the field is present, it is used to help target MN to decide appropriate LTE band for SCell frequency measurement in case of inter-MN handover without SN change.

*** Unchanged text not included ***

End changes