3GPP TSG-RAN WG2 Meeting #112-e	R2-2010541

# Electronic meeting, 2nd - 8th November, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0459 | rev | - | Current version: | 15.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to pdcch-MonitoringSingleOccasion |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on updated RAN1 UE feature list, pdcch-MonitoringSingleOccasion is as follows: 3-2; PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot; For a given UE, all search space configurations are within the same span of 3 consecutive OFDM symbols in the slot; pdcch-MonitoringSingleOccasion. However, current description in 38.306 is still based on a previous version of RAN1 feature list (R1-1801198) and captures the following highlighted restriction: Indicates whether the UE supports receiving PDCCH scrambled with C-RNTI or CS-RNTI in a search space configured to be monitored within a single span of any three contiguous OFDM symbols in a slot with the capability of supporting at least 44 blind decodes in a slot for 15 kHz subcarrier spacing. Hence, the description of pdcch-MonitoringSingleOccasion should be updated to not account with the condition above. The condition concerning a slot with 15 kHz, even though not captured in the updated RAN1 UE feature list, should be kept since it is also captured in TS 38.213: “If the monitoringSymbolsWithinSlot indicates to a UE to monitor PDCCH in a subset of up to three consecutive symbols that are same in every slot where the UE monitors PDCCH for all search space sets, the UE does not expect to be configured with a PDCCH SCS other than 15 kHz if the subset includes at least one symbol after the third symbol.” Impact analysis Impacted 5G architecture options: Standalone, EN-DC Impacted functionality: PDCCH monitoring Inter-operability: If the network implements the CR and the UE does not, there is no inter-operability issue, since it is assumed that the limitation on receiving PDCCH scrambled only with C-RNTI or CS-RNTI is not aplicable to pdcch-MonitoringSingleOccasion. If the UE implements the CR and the network does not, there is no inter-operability issue, since it is assumed that the limitation on receiving PDCCH scrambled only with C-RNTI or CS-RNTI is not aplicable to pdcch-MonitoringSingleOccasion. |
| Summary of change: |  | 4.2.7.10 Phy-Parameters - pdcch-MonitoringSingleOccasion - correct the definition of this field to not refer to the case where PDCCH is scrambled with C-RNTI or CS-RNTI; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The pdcch-MonitoringSingleOccasion will be tied to C-RNTI/CS-RNTI, which may lead to unintended behaviour, e.g. it can be interpreted as support of more spans than intended for this capability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

#### 4.2.7.10	Phy-Parameters

***************** omitted unchanged parts *****************

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| pdcch-MonitoringSingleOccasion Indicates whether the UE supports receiving PDCCH in a search space configured to be monitored within a single span of any three contiguous OFDM symbols in a slot with the capability of supporting at least 44 blind decodes in a slot for 15 kHz subcarrier spacing. | UE | No | No | FR1 only |
| pdcch-BlindDetectionCA Indicates PDCCH blind decoding capabilities supported by the UE for CA with more than 4 CCs as specified in TS 38.213 [11]. The field value is from 4 to 16. NOTE: FR1-FR2 differentiation is not allowed in this release, although the capability signalling is supported for FR1-FR2 differentiation. | UE | No | No | No |

END OF CHANGE