3GPP TSG-RAN WG2 Meeting #112e	R2-2010547

Online, November 2nd - 13th, 2020

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0462 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 1 bit capability for gap requirement info for FR1 and FR2 measurements while UE is EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-10-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The NeedForGap capability was defined for LTE SA and NR SA to NR target bands in Rel.16. the support of this feature didn’t discuss the expected UE/network behaviour when UE transition from LTE SA to EN-DC and whether can use the same capability. this CR proposes to add 1-bit capability for the case when UE is in EN-DC and performing measurement on NR target band. The support of this feature while UE is in EN-DC, is critical for optimum system performance in the field. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Therefore, we propose to add a 1-bit capability that allows UE to inform network about its gap requirement when performing measurements on FR1, FR2 NR bands and on IRAT LTE bands while UE is in EN-DC mode: - adding an optional 1-bit capability (interFreq-NeedForGapsENDC-toFR1-r16) to indicate the support of gap/no-gap requirement when UE is in EN-DC and performing measurement on target NR FR1. - adding an optional 1-bit capability (interFreq-NeedForGapsENDC-toFR2-r16) to indicate the support of gap/no-gap requirement when UE is in EN-DC and performing measurement on target NR FR2. Impact Analysis: Impacted 5G architecture options: EN-DC Impacted functionality: Measurement capaiblity. Interoperability issue: - if the network is implemented according to the CR and the UE is not, i.e. capability information is not provided to the network, no interoperability issue is expected. - if the UE is implemented according to the CR and the network is not, no interoperability issue is expected, as network ignores the new capability and configures gap as needed using legacy capability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inability to perform measurement on NR target bands while UE is in EN-DC without gap being configured |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.9a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.331 CR 2238 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of changes

#### 4.2.9a	MeasAndMobParametersMRDC

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| condPSCellChangeFDD-TDD-r16 Indicates whether the UE supports conditional PSCell change between FDD and TDD cells. | UE | No | No | No |
| condPSCellChangeFR1-FR2-r16 Indicates whether the UE supports conditional PSCell change between FR1 and FR2. | UE | No | No | No |
| interFreq-NeedForGapsENDC-toFR1-r16 Indicates whether the UE can perform inter-frequency SSB based measurements without measurement gaps on FR1 while UE is configured with EN-DC. | UE | No | No | No |
| interFreq-NeedForGapsENDC-toFR2-r16 Indicates whether the UE can perform inter-frequency SSB based measurements without measurement gaps on FR2 while UE is configured with EN-DC. | UE | No | No | No |
| pscellT312-r16 Indicates whether the UE supports T312 based fast failure recovery for PSCell. | UE | No | No | No |

End of changes