3GPP TSG-RAN WG2 # 112- e 	R2-2010606

# Electronic meeting, 2nd - 13th November, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2255 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 38331 on RA report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2020-10-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - The field description of cellID of RA report is as follows: - “This field indicates the CGI of the cell in which the associated random access procedure was performed.” - It is agreed in RAN2#111-e that UE will includes the physical cell ID together with the frequency information of the serving cell as cellID in RA report when the CGI of the cell the Random Access procedure occurs is not exist. However the field description of cellID doesn’t address the agreements correctly. - Also considering for RA initiate in SCell, Msg1 is transmitted in SCell while the monitoring of RAR is done in SpCell, technical speaking the RA procedure performs in both cells, therefore it is ambiguous current field description of cellID refers to which cell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The field description of cellID of RA report is updated to include the case when CGI is not available, and the cellID definition is modified to indicate the cell where the Msg1 is transmitted to avoid misunderstanding. Impact analysis Impacted 5G architecture options: Standalone, MR-DC Impacted functionality: The CR has impact on RA report feature. Inter-operability: - There will be misunderstanding between UE and gNB on the cell the RA procedure performs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be misunderstanding between UE and gNB on the cell the RA procedure performs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change:

-------------------------------------------------- [Start of change] --------------------------------------------

#### –	UEInformationResponse

The UEInformationResponse message is used by the UE to transfer information requested by the network.

Signalling radio bearer: SRB1 or SRB2 (when logged measurement information is included)

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to network

/* Unrelated part omitted /*

RA-Report field descriptions

absoluteFrequencyPointA This field indicates the absolute frequency position of the reference resource block (Common RB 0).

cellID This field indicates the CGI of the cell in which the Msg1 of the associated random access procedure was transmitted if available otherwise the physical cell identity and the carrier frequency of the cell in which the Msg1 of the associated random access procedure was transmitted.

contentionDetected This field is used to indicate that contention was detected for the transmitted preamble in the given random access attempt or not. This field is not included when the UE performs random access attempt is using contention free random-access resources or when the ra-Purpose is set to requestForOtherSI.

csi-RS-Index This field is used to indicate the CSI-RS index corresponding to the random access attempt.

-------------------------------------------------- [End of change] ---------------------------------------------