3GPP TSG-RAN WG2 #112-e	R2-2010608

Electronic meeting, 2rd – 13th November, 2020

Agenda Item:	8.13.2.3

Source:	  NTTDOCOMO, INC.

Title:	Discussion on rel-17 Radio Link Failure Report for CG failure aspects

Document for:	Discussion and Decision

1	Introduction

In [1], one of rel-17 WID objective is to enhance reporting of RLF. This paper discusses the enhancement of RLF-report based on new radio link failure type introduced in the DCCA enhancement WI in rel-16.

------------------------------------------------------Excerpt from [1] ---------------------------------------------------------

- Support of data collection for MDT features for identified use cases, including 2-step RACH optimization and leftovers of Rel-16 SON/MDT WI (MDT enhancements and MDT for MR-DC) [RAN2, RAN3, RAN4]

- Enhancement of logged and immediate MDT (including coexistence with IDC) [RAN2, RAN3]

- Enhancement of reporting e.g. RLF and accessibility measurements, Successful Handover reporting [RAN2, RAN3].

- Specification of MDT for MR-DC [RAN2, RAN3, RAN4]

---------------------------------------------------------------------------------------------------------------------------------------

2	Discussion

2.1 RLF triggered by MCG/SCG failure

In rel-16 DCCA enhancement, MCGFailuerRecovery function is introduced. When radio link failure occurred in the MCG, UE could send MCGFailureInformation to the network via SCG if SCG is not suspended or during PSCell change. While in rel-15, if radio link failure occurred in the MCG, RLF is triggered. Since in current RRC spec, the connectionFailureType is only “rlf”, from network perspective, it could hardly distinguish the following two cases:

- Case A: The RLF is triggered by MCG link failure (UE is not configured with MCGFailureRecovery )

- Case B: The RLF is triggered by MCG link failure and SCG link is suspended or during PSCell change (UE is configured with MCGFailureRecovery)

So we propose to add a new connection failure type of rlfOfBothMCGAndSCG for to distinguish case A and B.

Observation1: From current connectionFailureType in rlf-Report, network could hardly distinguish the difference of case A and B indicated above.

Proposal1: Add a new failure type of rlfOfBothMCGAndSCG in connectionFailureType in RLF report.

Also, if RLF is triggered by SCG failure and MCG transmission is suspended, network needs to know the failed cell information of SCG and measurement result of MCG. Thus, UE should store SCGFailureInformation in VarRLF-Report when T310 is expired in SCG and MCG transmission is suspended. Similarly, if RLF is triggered by MCG failure and SCG transmission is suspended, since failed MCG cell information is already stored in RLF-report when RLF is triggered, UE can store the measurement result of SCG (measResultSCG) in RLF-report.

Observation2: In case RLF is triggered by SCG and MCG transmission is suspended, UE could not report failed cell information of SCG to network.

Observation3: In case RLF is triggered by MCG and SCG transmission is suspended, UE could not reported measurement result of SCG (MeasResultSCG) to the network.

Proposal2: In case RLF is triggered by SCG and MCG transmission is suspended, UE store the SCGFailureInformation in VarRLF-Report.

Proposal3: In case RLF is triggered by MCG and SCG transmission is suspended, UE store the MCGFailureInformation or measResultSCG in VarRLF-Report.

2.2 RLF in NPN cell

Rel-16 NPN WI introduced NPN-Identity which is a combination of PLMN identity and CAG ID (NID). In case UE is accessing NPN cell and radio link failure occurred, UE should be able to report the NPN-identity(List) to the network.

Observation4: Rel-16 NPN WI introduced NPN-Identity which is combination of PLMN identity and CAG ID (NID).

Proposal4: UE report NPN-Identity(List) in RLF-report to the network in case radio link failure is occurred in a NPN cell.

3. Conclusion

Based on the discussion in the previous sections, we made the following proposals:

Observation1: From current connectionFailureType in rlf-Report, network could hardly distinguish the difference of case A and B indicated above.

Observation2: In case RLF is triggered by SCG and MCG transmission is suspended, UE could not report failed cell information of SCG to network.

Observation3: In case RLF is triggered by MCG and SCG transmission is suspended, UE could not reported measurement result of SCG (MeasResultSCG) to the network.

Observation4: Rel-16 NPN WI introduced NPN-Identity which is combination of PLMN identity and CAG ID (NID).

Proposal1: Add a new failure type of rlfOfBothMCGAndSCG in connectionFailureType in RLF report.

Proposal2: In case RLF is triggered by SCG and MCG transmission is suspended, UE store the SCGFailureInformation in VarRLF-Report.

Proposal3: In case RLF is triggered by MCG and SCG transmission is suspended, UE store the MCGFailureInformation or measResultSCG in VarRLF-Report.

Proposal4: UE report NPN-Identity(List) in RLF-report to the network in case radio link failure is occurred in a NPN cell.

4. References

- RP-201281 NEW WID on enhancement of data collection for SON/MDT in NR

- TS 38.331