3GPP TSG-RAN Meeting #112	R2-2010612

# Electronic meeting, 2nd – 13th November 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.314 | CR | 0010 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On the clarification of end time of UL PDCP Packet Average Delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2020-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on the current definition of the UL PDCP packet average delay measurement, there is a risk that the UE always reports PDCP queueing delay to be ‘0’ for all the scenarios where the UE already has pre-scheduled grant available. In such a case, the total RAN delay might be very small and well within the QoS requirements but the perceived delay from the application at the UE for UL. Therefore, the PDCP packet average delay as specified do not reflect the true delay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that the tDeliv of the UL PDCP packet average delay definition is the point in time when the associated UL MAC SDU i is scheduled for transmission. Impact analysis Impacted functionality Immediate MDT and QoS monitoring Inter-operability: If the network is implemented according to the CR and the UE is not, the UL PDCP packet average delay reported by the UE for pre-scheduled grant based scenarios will not reflect the true QoS as perceived. If the UE is implemented according to the CR and the network is not, there will not be any inter-operability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PDCP packet average delay as specified do not reflect the true delay associated to the time spent by the UL packets in UE’s PDCP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Beginning Of Change

#### 4.3.1.1	UL PDCP Packet Average Delay per DRB per UE

The objective of this measurement performed by UE is to measure Packet Delay in Layer PDCP for QoS verification of MDT or for the QoS monitoring as defined in TS 23.501 [4].

Protocol Layer: PDCP

Table 4.3.1.1-1: Definition for UL PDCP Packet Average Delay per DRB per UE

| Definition | PDCP Packet Delay in the UL per DRB. This measurement refers to PDCP queuing delay for DRBs in the UE, which captures the delay from packet arrival at PDCP upper SAP until the UL grant to transmit the packet is available, which has included the delay the UE gets resources granted (from sending SR/RACH to get the first grant). The measurement is done separately per DRB. Detailed Definition: MT,drbid=∀itDelivi, drbid-tArrivali, drbidIT, where explanations can be found in the table 4.3.1.1-2 below. |
| --- | --- |

NOTE:	UE measures UL PDCP queueing delay at DRB level. It is up to gNB to convert DRB level delay to QoS level delay with the assumption that all QoS flows mapped to the same DRB get the same QoS treatment, and it is up to gNB to calculate QoS level delay if multiple DRBs mapped with the same QoS.

Table 4.3.1.1-2: Parameter description for UL PDCP Packet Average Delay per DRB per UE

| M(T,drbid) | PDCP average delay in the UL per DRB, averaged during time period 𝑇. Unit: 0.1 ms. PDCP average delay in the UL per DRB is 1s if the actual value is larger than 1s. |
| --- | --- |
| tArrival(i) | The point in time when the PDCP SDU i arrivals at PDCP upper SAP. |
| tDeliv(i) | The point in time when the UL MAC PDU i including the first part of PDCP SDU i is scheduled for transmission. |
| i | A PDCP SDU that is received by the PDCP during time period 𝑇. |
| I(T) | Total number of PDCP SDUs 𝑖. |
| T | Time Period during which the measurement is performed |
| drbid | The identity of the measured DRB. |

End of Change