# 3GPP TSG-RAN WG2 Meeting#112		       						  R2-2010646

# Electronic, 2nd - 13th November 2020

Title:	[Draft] Reply LS on Cell Configuration within TA/RA to Support Allowed NSSAI

Response to:	R2-2008759/S2-2006526

Release:	Rel-17

Work Item:	FS_NR_slice

Source:	ZTE [to be RAN2]

To:	SA2

Cc:	CT1, RAN3

Contact Person:

Name:				Yuan Gao

E-mail Address:		gao.yuan66@zte.com.cn

- Overall Description:

RAN2 thanks SA2 for the LS on Cell Configuration within TA/RA to Support Allowed NSSAI.

- In Rel-15 and 16, is it expected that each cell in the tracking area supports the same S-NSSAI(s)? (or, said otherwise, do all cells advertising the same TAC support the same set of S-NSSAIs?).

Answer: From RAN2’s perspective, it is not expected that each cell in the tracking area supports the same S-NSSAI(s).

If the answer is "no":

2a) Can RAN WGs and CT1 explain if it can happen that a UE, e.g. due to local radio conditions, can only use a cell in the TA where not all S-NSSAIs are present in the Allowed NSSAI it received (and that the TA supports), and can RAN WGs and CT1 explain how it is handled today in rel-15/16?

2b) If an S-NSSAI can be rejected depending on which cell the UE camps on even though it is supported in the TA, for the reason that it is not supported in the cell, is there in rel-15/16 a CT1 error code to handle this case (i.e. can a S-NSSAI be rejected, with a suitable cause code, depending on which cell of the TA the UE camps on, even though this S-NSSAI is known to be supported in the TA, for the reason that this S-NSSAI is actually not supported in the cell of the TA)? Is there any provisions in the RAN or CT1 specifications to handle this case?

Answer: From RAN2’s perspective, it is possible that UE selects a cell not supporting the UE intended slice due to local radio conditions (e.g. the best cell does not support the intended slice but it is selected by UE following the cell reselection rules). In this case, the R15/R16 mechanism (e.g. CA/DC, HO, release with redirection and UE specific cell reselection priority) can be used to guide UE to a cell supporting the intended slice. It is also possible that UE camp on a cell not supporting the UE intended slice while all the other cells in the TA are not available. UE may initiate service for the intended slice on this cell but the PDU session will not be set up successfully and a cause “slice(s) not supported” will be reported to the core network.

# 2. Actions:

RAN2 kindly asks SA2, CT1 and RAN3 to take the above into account.

# 3. Date of Next TSG-RAN WG2 Meetings:

RAN2#113           	2021-01-25	2021-02-05	Online Meeting

RAN2#113-bis        2021-04-12	2021-04-20	Online Meeting