3GPP TSG-RAN WG2 #112e	R2-2010709

Electronic meeting, November 2nd – 13th 2020

Agenda Item:	6.6.2

Source:	Ericsson

Title:	Summary for RRC Corrections for Positioning

Document for:	Discussion, Decision

# 1	Introduction

The below papers have been submitted in the RRC AI 6.6.2. The paper provides the summary of the submitted paper.

R2-2008806	Corrections on 38.331 to capture agreements of area scope for posSIB validity	CATT,Ericsson	CR	Rel-16	38.331	16.2.0	2014	-	F	NR_newRAT-Core

R2-2008807	Corrections on description of sfn-Offset and sfn0-Offset in SSB-Configuration  in TS 38.331	CATT	CR	Rel-16	38.331	16.2.0	2015	-	F	NR_newRAT-Core

R2-2008808	Correction on the missed description of sfn-SSB-Offset in SSB-Configuration  in TS 38.331	CATT	CR	Rel-16	38.331	16.2.0	2016	-	F	NR_newRAT-Core

R2-2010071	Corrections and update for RRC Positioning	Ericsson	CR	Rel-16	38.331	16.2.0	2172	-	F	NR_pos-Core

R2-2010269	CR on SI window for positioning SI message	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2196	-	F	NR_pos-Core

R2-2010270	Correction on posSRS configuration	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2197	-	F	NR_pos-Core

R2-2010272	Correction on acquisition of MIB and SIB1	Huawei, HiSilicon, Ericsson	CR	Rel-16	38.331	16.2.0	2198	-	F	NR_pos-Core

R2-2010273	Correction on posSIB broadcastStatus	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2199	-	F	NR_pos-Core

# 2	Discussion

## 2.1	posSIB Validity

R2-2008806 provides the changes needed to describe how UE will determine the posSIB validity. It was briefly discussed during last RAN#111e-meeting where companies expressed that it would be more appropriate to capture posSIB validity in TS 38.331 similar to existing SIB validity that already exist in RRC for NR SIBs.

RAN2 to agree the posSIB validity inclusion in RRC and review the CR for posSIB validity check provided in R2-2008806 by email discussion.

## 2.2	sfn-SSB-Offset, sfn-Offset, sfn0-offset

R2-2008807, R2-2008808, R2-2010071, R2-2010270 provides CR for correction of field description for SSB-Configuration related to fields: sfn-Offset and sfn-SSB-Offset and sequence sfn0-offset.

RAN2 to provide correction for field description for fields (sfn-Offset and sfn-SSB-Offset) available in SSB-Configuration. The exact changes are captured via email discussion review.

## 2.3	Removal of PagingSearchSpace And Modification Period for SIB1 acquisition

R2-2010272 suggest simplifying SIB1 acquisition clause by removing unnecessary check such as UE configured with paging search space and to read SIB1 only once during modification period.

In 5.2.2.3.1, there is the following description to check the broadcasting status of UE required SI message (including UE concerned SIBs or posSIBs) in SIB1.

1>	if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and pagingSearchSpace and the UE has not stored a valid version of a SIB, in accordance with sub-clause 5.2.2.2.1, of one or several required SIB(s), in accordance with sub-clause 5.2.2.1, and, UE has not acquired SIB1 in current modification period or if requested by upper layers; or

The reading of the SIB1 is triggered internally by the UE and the configuration of paging search space is not necessary for the condition to read SIB1, i.e., without paging search space, the UE should also be able to read SIB1 with searchSpaceSIB1

The condition “UE has not acquired SIB1 in current modification period” is not required. Removing this will simplify such that UE will acquire SIB1 just before the SI acquisition. If the current modification period for SIB1 is long duration, the broadcast status available in SIB1 may have changed (it may have happened that the UE read the SIB1 at the start of modification period). Removing this condition will allow UE to re-read SIB1 in the same modification period.

Remove “pagingSearchSpace” and “UE has not acquired SIB1 in current modification period” in section 5.2.2.3.1 for SIB1 acquisition. Agree to the CR R2-2010272.

Note: The cover page may need update. The CR number is not in right format and date provided is wrong.

Note: This CR may need to be treated in main session as it is not positioning specific.

## 2.4	posSI Broadcast Status Field Description Update

R2-2010273 provides the missing field description update for posSI-BroadcastStatus. The below text is already available for the legacy si-BroadcastStatus. It should be the same for posSI-BroadcastStatus

posSI-BroadcastStatus

Indicates if the SI message is being broadcasted or not. Change of posSI-BroadcastStatus should not result in system information change notifications in Short Message transmitted with P-RNTI over DCI (see clause 6.5). The value of the indication is valid until the end of the BCCH modification period when set to broadcasting.

RAN2 to agree to update field description of posSI-BroadcastStatus as provided in R2-2010273.

Note: The cover page may need update. The CR number is not in right format and date provided is wrong.

## 2.5	SI message with SIBs and SI message with posSIBs Overlap

R2-2010269 introduces CR with below change.

Each SI message is associated with an SI-window and the SI-windows of different SI messages  with SIBs do not overlap and the SI-windows of different SI messages with posSIBs do not overlap.

However, it is unclear as why such change is required. The current text talks about SI message and the SI message may include SIBs or posSIBs. So, a separate mentioning of posSIBs may not be needed.

The intention seems to clarify that the SI containing SIBs and SI containing posSIBs do not overlap; however this is as such already clear from original text.

No need to capture that the SI message with SIBs do not overlap with SI message with posSIBs.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to agree the posSIB validity inclusion in RRC and review the CR for posSIB validity check provided in R2-2008806 by email discussion.

Proposal 2	RAN2 to provide correction for field description for fields (sfn-Offset and sfn-SSB-Offset) available in SSB-Configuration. The exact changes are captured via email discussion review.

Proposal 3	Remove “pagingSearchSpace” and “UE has not acquired SIB1 in current modification period” in section 5.2.2.3.1 for SIB1 acquisition. Agree to the CR R2-2010272.

Proposal 4	RAN2 to agree to update field description of posSI-BroadcastStatus as provided in R2-2010273.

Proposal 5	No need to capture that the SI message with SIBs do not overlap with SI message with posSIBs.

# References

[1] R2-2008806	Corrections on 38.331 to capture agreements of area scope for posSIB validity	CATT, Ericsson

[2] R2-2008807	Corrections on description of sfn-Offset and sfn0-Offset in SSB-Configuration  in TS 38.331	CATT

[3] R2-2008808	Correction on the missed description of sfn-SSB-Offset in SSB-Configuration  in TS 38.331	CATT

[4] R2-2010071	Corrections and update for RRC Positioning	Ericsson

[5] R2-2010269	CR on SI window for positioning SI message	Huawei, HiSilicon

[6] R2-2010270	Correction on posSRS configuration	Huawei, HiSilicon

[7] R2-2010272	Correction on acquisition of MIB and SIB1	Huawei, HiSilicon, Ericsson

[8] R2-2010273	Correction on posSIB broadcastStatus	Huawei, HiSilicon