3GPP TSG-RAN WG2 Meeting #112e	R2-2010758

# Online, November 02 – 13 2020

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR | 1322 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CP length and reference signal for MBSFN with sub-carrier spacing of 0.375 kHz and 2.5 kHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast-Core |  |  |  |  |  | Date: |  |  | 2020-11-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN1 has endorsed an update to TS 36.300 and sent LS in R2-2008704 (R1-2007154). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification on cyclic prefix length and downlink MBSFN reference signal for sub-carrier spacing of 0.375 kHz and 2.5 kHz in MBMS-dedicated cell and MBMS/Unicast-mixed cell is added in section 5.1.1 and 5.1.4. Impact analysis Impacted functionality: LTE-based 5G terrestrial broadcast Inter-operability: There is no interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cyclic prefix length and downlink MBSFN reference signal for sub-carrier spacing of 0.375 kHz and 2.5 kHz in MBMS-dedicated cell and MBMS/Unicast-mixed cell remain unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1, 5.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## Start of changes

## 5.1	Downlink Transmission Scheme

### 5.1.1	Basic transmission scheme based on OFDM

The downlink transmission scheme is based on conventional OFDM using a cyclic prefix. The OFDM sub-carrier spacing is f = 15 kHz. 12 consecutive sub-carriers during one slot correspond to one downlink resource block. In the frequency domain, the number of resource blocks, NRB, can range from NRB-min = 6 to NRB-max = 110 per CC or per Cell in case of CA or DC.

In addition, there are also four reduced sub-carrier spacings, flow = 7.5 kHz, flow1 = 2.5 kHz, flow2 = 1.25 kHz and flow3 ≈ 0.37 kHz for both MBMS-dedicated cell and MBMS/Unicast-mixed cell.

In case of 15 kHz sub-carrier spacing there are two cyclic-prefix lengths, corresponding to seven and six OFDM symbols per slot respectively.

-	Normal cyclic prefix: TCP = 160Ts (OFDM symbol #0) , TCP = 144Ts (OFDM symbol #1 to #6)

-	Extended cyclic prefix: TCP-e = 512Ts (OFDM symbol #0 to OFDM symbol #5)

where Ts = 1/ (2048  f)

In case of 7.5 kHz sub-carrier spacing, there is only a single cyclic prefix length TCP-low = 1024Ts, corresponding to 3 OFDM symbols per slot.

In case of 2.5 kHz sub-carrier spacing, there is only a single cyclic prefix length TCP-low1 = 3072Ts, corresponding to 1 OFDM symbol per slot.

In case of 1.25 kHz sub-carrier spacing, there is only a single cyclic prefix length TCP-low2 = 6144Ts, corresponding to 1 OFDM symbol per subframe.

In case of 0.37 kHz sub-carrier spacing, there is only a single cyclic prefix length TCP-low3 = 9216Ts, corresponding to 1 OFDM symbol per 3 ms slot as defined in TS 36.211 [4], clause 4.1.

In case of FDD, operation with half duplex from UE point of view is supported.

## Next change

### 5.1.4	Downlink Reference signal and synchronization signals

The downlink cell-specific reference signals consist of known reference symbols inserted in the first and third last OFDM symbol of each slot for antenna port 0 and 1. There is one cell-specific reference signal transmitted per downlink antenna port. The number of downlink antenna ports for the transmission of cell-specific reference signals equals 1, 2, or 4.

Physical layer provides 504 unique cell identities using Synchronization signals and resynchronization signals.

The downlink MBSFN reference signals consist of known reference symbols inserted every other sub-carrier in the 3rd, 7th and 11th OFDM symbol of sub-frame in case of 15 kHz sub-carrier spacing and extended cyclic prefix; every four sub-carriers in the 2nd, 4th and 6th symbol of sub-frame in case of 7.5 kHz sub-carrier spacing; every four sub-carriers in the single symbol of slot in case of 2.5 kHz sub-carrier spacing; every six sub-carriers in the single symbol of subframe in case of 1.25 kHz sub-carrier spacing; and every twelve sub-carriers for MBSFN reference signal pattern type 1 or every six sub-carriers for MBSFN reference signal pattern type 2 in the single symbol of 3 ms slot in case of 0.37 kHz sub-carrier spacing as defined in TS 36.211 [4], clauses 4.1 and 6.10.2.2.4.

In addition to cell-specific reference signals and MBSFN reference signals, the physical layer supports UE-specific reference signals, positioning reference signals, CSI reference signals, and discovery signals.

A UE may assume presence of the discovery signals consisting of cell-specific reference signals, primary and secondary synchronization signals, configurable resynchronization signals, and configurable CSI reference signals.

## End of changes