3GPP TSG-RAN WG2 Meeting #112e	R2-2010846

Electronic, 2 – 13 November 2020

Agenda item:	6.3.3

Source: 	Qualcomm Incorporated

Title:  	Report of [AT112e][508][NRU] on Miscellaneous RRC corrections

Document for:	Discussion and decision

# Introduction

During the first online session of RAN2#112e for NR-U, it was decided to further discuss several RRC CRs in the following email discussion. Other less controversial CRs will be merged into one CR per Chair Notes from the session.

[AT112-e][508][NR-U] Miscellaneous corrections RRC (Qualcomm)

Scope:

    - Captured agreed changes and send out for review

Intended outcome:

    - Agreeable CR

Deadline for providing comments:

    - Companies input:  Nov. 11th

    - Updated CR ready for approval: Nov. 12th

This document will capture feedback from companies and suggest whether or not to agree to the CR, possibly with some modifications.

| Company | Contact Name, Email |
| --- | --- |
| Huawei | Tao Cai, tao.cai@huawei.com |
| Nokia | jarkko.t.koskela@nokia.com |
| Apple | yuqin_chen@apple.com |
| Xiaomi | wuyumin@xiaomi.com |
| Intel Corporation | Seau.s.lim@intel.com |
| MediaTek | pradeep[dot]jose[at]mediatek[dot]com |
| Samsung | jack.jang@samsung.com |
| ZTE | eswar.vutukuri at zte dot com dot cn |
| vivo | yitao.mo@vivo.com |
| Ericsson | mai-anh.phan@ericsson.com |

# Discussion

R2-2009295	Correction on description for extendedRAR-window	Huawei, HiSilicon	CR	Rel-16	38.306	16.2.0	0424	-	F	NR_unlic-Core

=>	General support, move to email discussion 508

This change clarifies the UE capability for monitoring extended RAR window.

Summary of Changes:

Correct the field description for extendedRAR-window.

Do you agree with the changes proposed in this CR?

| Company | Response | Comments |
| --- | --- | --- |
| Huawei | Yes | It is good to be accurate with at least “maximum window” and “LSB”. |
| Nokia | No strong view | We don’t really see much difference with changed text but nothing seems to be wrong with CR either |
| Apple | Yes | Fine to have the change. |
| Xiaomi | Yes |  |
| Intel | Yes | It would be good to update it if we are keeping this capability |
| MediaTek | Yes |  |
| Samsung | No strong view | but fine to have these non-technical changes |
| ZTE | Discussion needed Also related to CB [507] ?? | We are not sure if this discussion also includes the CB 507 that was mentioned in the notes (since that thread has not yet been triggered). But these are related issues. So, we want to express our views here in general. - As discussed online, we have agreed that this capability is not needed, but RAN1 indicate that this is needed for LAA style operation. Since there is no LAA capability, we think we should first discuss how to clarify that this field is only applicable if UE only supports LAA style operation in NR-U (i.e. non-standalone only mode). - It should be clarified that this is only applicable for NR-U but not for 2-step RACH, the simplest way would be to modify as follows: Indicates whether the UE supports the configuration of maximum length of RAR window with a value larger than 10ms and up to 40ms by decoding of the 2 LSB bits of SFN indication in DCI 1_0 scrambled with RA-RNTI. In general, we think we should first wait for the issue 1 to be decided before fine tuning this wording here. |
| vivo | Yes with comments | It looks a bit redundant to say “2 LSB bits”. Thus, we propose the terminology “2 LSB bits of SFN indication” is replaced by “2 LSBs of SFN”, which is also used in 38.212 (i.e. LSBs of SFN). Besides, we share a similar view with ZTE’s second proposal. Based on the above, we propose the following text: Indicates whether the UE supports the configuration of maximum length of RAR window with a value larger than 10ms and up to 40ms by decoding of the 2 LSBs bits of SFN indication in the DCI format 1_0 with CRC scrambled by RA-RNTI. |
| Ericsson | Yes with further change proposals. | We think that the proposed clarification is useful. Further proposals: - It would be clearer to point directly to the RRC field ra-ResponseWindow (with v1610 suffix and without suffix) instead of “RAR window”. Then it also becomes clear that this capability is not related to the RAR extension for 2-step RACH reflected by msgB-ResponseWindow. With vivo’s proposal, we would have the following description: Indicates whether the UE supports the configuration of ra-ResponseWindow or ra-ResponseWindow-v1610 maximum length of RAR window with a value larger than 10ms and up to 40ms by decoding of the 2 LSBs bits of SFN indication in the DCI format 1_0 with CRC scrambled by RA-RNTI. 2. Naming: the current capability is called “extendedRAR-Window”. However, the corresponding field is called “ra-ResponseWindow”. It would be nice to align the name as follows for easier parameter search: “extRA-ResponseWindow” (“ext” is listed in Table A.3.1.2-1, Examples of typical abbreviations used in ASN.1 identifiers) |

Summary: 5 companies support the CR while 1 company (Vivo) suggests some edits to improve the change. ZTE thinks that we should wait for the conclusion of the discussion on whether to keep this capability. Assuming the capability is kept, the CR can be agreed. The editorial part of the suggestions by Vivo are also reasonable.

Proposal 1: Assuming the extendedRAR-window-r16 is not removed from Rel-16 specifications (per offline discussion 507), agree to the CR in R2-2009295 with the change: “2 LSBs bits of SFN indication in the DCI format 1_0”.

R2-2009296	Correction of field description for ra-ResponseWindow	Huawei, HiSilicon, Ericsson	CR	Rel-16	38.331	16.2.0	2052	-	F	NR_unlic-Core, NR_2step_RACH-Core

=>	Need to check if RAN1 text is sufficient and whether it is new or not. Note that RAN2 has discussed this last meeting and decided to keep the sentence. Move to email discussion 508

Summary of Changes:

Remove the sentence for the content of DCI when the length of the rar-window is larger than 10ms

Do you agree with the changes proposed in this CR?

| Company | Response | Comments |
| --- | --- | --- |
| Huawei | Yes | During last meeting, some commented that RAN1 has not yet finalized their work and we should postpone this to the future meetings. Apparently RAN1 has finalized this now, according to the current spec 38.212, it reads as follows for the LSBs of SFN in the DCI Based on this, we think it safe and clear enough to remove the redundant description in the RRC spec about the DCI content. |
| Nokia | Yes |  |
| Apple | Yes |  |
| Xiaomi | Yes |  |
| Intel | Yes |  |
| MediaTek | Yes |  |
| Samsung | Yes |  |
| ZTE | Yes |  |
| vivo | Yes | There is no need to have duplication. |
| Ericsson | Yes | Better to refer to 38.212 instead of duplicating text. |

Summary: All companies support the change.

Proposal 2: Agree to the CR in R2-2009296.

R2-2009349	Clarification on HARQ processes sharing	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.2.0	2055	-	F	NR_unlic-Core

=>	Move to email discussion

Summary of Changes:

Change the field description for configuredGrantTimer as: “When cg-RetransmissonTimer is configured, if HARQ processes are shared among different configured grants on the same BWP, configuredGrantTimer * periodicity is set to the same value for the configurations that share HARQ processes on this BWP.”

Do you agree with the changes proposed in this CR?

| Company | Response | Comments |
| --- | --- | --- |
| Huawei | Yes |  |
| Nokia | Yes |  |
| Apple | Yes |  |
| Xiaomi | Yes |  |
| Intel | Yes |  |
| MediaTek | Yes |  |
| Samsung | Yes | (unnecessary restrictions) |
| ZTE | Yes |  |
| vivo | Yes |  |
| Ericsson | Yes |  |

Summary: All companies support the change.

Proposal 3: Agree to the CR in R2-2009349.

# Conclusion

This report captures the feedback for some of the RRC CRs submitted for NR-U and, based on feedback from the companies, the following are proposed:

Proposal 1: Assuming the extendedRAR-window-r16 is not removed from Rel-16 specifications (per offline discussion 507), agree to the CR in R2-2009295 with the change: “2 LSBs bits of SFN indication in the DCI format 1_0”.

Proposal 2: Agree to the CR in R2-2009296.

Proposal 3: Agree to the CR in R2-2009349.