3GPP TSG-RAN2 Meeting # 112	R2-2010946

# E-meeting, November 2nd-13th

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1786 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on V2X UE capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-10-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Capture further Release-16 UE capabilities based on the RAN1 UE feature list and LS reply from RAN1 as follows: - R2-2011116, LS on updated Rel-16 RAN1 UE features lists for LTE (R1-2009351; contact: NTT DOCOMO, AT&T) - R2-2011205, Reply LS on UE capability for V2X (R1-2009635; contact: OPPO) - R2-2011206, Reply LS on maximum data rate for NR sidelink (R1-2009643; contact: OPPO) - R2-2008713, Reply LS on maximum data rate for NR sidelink (R1-2007353; contact: Samsung) |  |  |  |  |  |  |  |  |
| Summary of change: |  | New Release-16 capabilities from RAN1 are added based on the latest RAN1 feature list and LS reply from RAN1 as follows - R2-2011116, LS on updated Rel-16 RAN1 UE features lists for LTE (R1-2009351; contact: NTT DOCOMO, AT&T) - R2-2011205, Reply LS on UE capability for V2X (R1-2009635; contact: OPPO) - R2-2011206, Reply LS on maximum data rate for NR sidelink (R1-2009643; contact: OPPO) - R2-2008713, Reply LS on maximum data rate for NR sidelink (R1-2007353; contact: Samsung) Impact analysis Impacted functionality: V2X UE capability Inter-operability: - If UE implements according to the CR and the network does not, there is no inter-operability issues since network would not configure UE for the additional V2X capability. - If the network implements according to the CR and the UE does not, there is no inter-operability issues since the UE would not report the additional V2X capability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New RAN1 related UE capabilities will not be captured in specifcations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.5, 4.3.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 36.331 CR 4449 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Start

#### 4.3.21.32	v2x-SupportedBandCombinationListEUTRA-NR-r16

This field indicates the band combination(s) on which the UE supports NR sidelink communication-only, as defined in TS 23.287 [42] and specified in TS 38.331 [35], or simultaneously with V2X sidelink communication, as defined in TS 23.285 [29] and specified in TS 36.331 [5].

Next Change

4.3.5.X	v2x-SupportedTxBandCombListPerBC-v16xy, v2x-SupportedRxBandCombListPerBC-v16xy

This field indicates, for a particular band combination of EUTRA, the supported band combination list among v2x-SupportedBandCombinationListEUTRA-NR on which the UE supports simultaneous transmission or reception of EUTRA and NR sidelink communication respectively, or simultaneous transmission or reception of EUTRA and mixed V2X sidelink and NR sidelink communication respectively.

Next Change

4.3.5.X	scalingFactorTxSidelink-r16, scalingFactorTxSidelink-r16

This field indicates, for a particular band combination of EUTRA, the scaling facor, as defined in TS 38.306 [32], for the PC5 band combination(s) v2x-SupportedBandCombinationListEUTRA-NR on which the UE supports simultaneous transmission/reception of EUTRA and NR sidelink communication respectively, or simultaneous transmission or reception of EUTRA and joint V2X sidelink communication and NR sidelink communication respectively (as indicated by v2x-SupportedTxBandCombListPerBC-v16xy / v2x-SupportedRxBandCombListPerBC-v16xy). The leading / leftmost value corresponds to the first band combination included in v2x-SupportedBandCombinationListEUTRA-NR which is indicated with value 1 by v2x-SupportedTxBandCombListPerBC-v16xy / v2x-SupportedRxBandCombListPerBC-v16xy, the next value corresponds to the second band combination included in v2x-SupportedBandCombinationListEUTRA-NR which is indicated with value 1 by v2x-SupportedTxBandCombListPerBC-v16xy / v2x-SupportedRxBandCombListPerBC-v16xy and so on.

Next Change

4.3.21.X	tx-Sidelink-r16, rx-Sidelink-r16

This parameter indicates whether the UE supports sidelink transmission/reception on the band in the band combination. For NR sidelink transmission, tx-Sidelink-r16 is only applicable if the UE supports at least one of sl-TransmissionMode1-r16 and sl-TransmissionMode2-r16 on the band as specified in TS 38.331 [82]. For NR sidelink reception, rx-Sidelink-r16 is only applicable if the UE supports sl-Reception-r16 on the band as specified in TS 38.331 [82].

End of Change