3GPP TSG-RAN WG2 Meeting #112 Electronic	R2-2011036

Elbonia, 02 – 13 November 2020

Agenda item:	5.2

Source:	Nokia (Rapporteur)

Title:	Offline 001 on Stage 2 Corrections

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

This document is the report of the following email discussion:

[AT112-e][001][NR15] Stage-2 Corrections (Nokia)

Treat R2-2008816, R2-2008817, R2-2008818, R2-2008819, R2-2008820, R2-2009308, R2-2009309, R2-2009310, R2-2009311, R2-2008821, R2-2008822

Intended outcome: Intermediate: Determine agreeable parts. Final: For agreeable parts, agreed CRs.

Deadline: Intermediate deadline(s) by Rapporteur, Final: Discussion stop at Wed Nov 11, 1200 UTC

# 2	Clarification of SCell setup during inter-RAT HO

CRs on Clarification of SCell setup during inter-RAT HO were contributed in R2-2008817, R2-2008818, R2-2008819, R2-2008820 with a companion contribution in R2-2008816.

Question 1: do you agree with the observations made in that paper, specifically that:

1) 	Stage-3 specifications allow the target RAT to add SCells for usage with the target PCell in inter-RAT handover scenarios (LTE SA to NR SA and vice-versa); and

2) The intention of LTE and NR Stage-2 is not to restrict SCells addition only for intra-RAT scenarios.

| Answers to Question 1 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical justification |
| Qualcomm Incorporated | Yes |  |
| Ericsson (Tony) | Yes but | We are in principle okay with the first observation, but we think that the second one is not entirely true. We think this is not the intention of the current text in stage-2. |
| Lenovo | Yes |  |
| OPPO | Yes |  |
| Intel | Yes |  |
| Huawei, HiSilicon | Yes but | The observation looks OK. However the current text does not exclude such use cases and we don’t see it is essential to change the text. |
| CATT | Yes |  |
| Nokia | Yes |  |
| MediaTek | Yes |  |
| vivo | Yes but | Agree with Ericsson and Huawei. And CR version is also not latest version. |
| Apple | Yes |  |
| Sequans | Yes |  |
| Samsung | Yes | We in general agree with observations. |

Summary 1: 13 companies have expressed an opinion and all agreed with the observations.

Proposal 1: acknowledge that current Stage-3 specifications allow the target RAT to add SCells for usage with the target PCell in inter-RAT handover scenarios (LTE SA to NR SA and vice-versa); and the intention of LTE and NR Stage-2 is not to restrict SCells addition only for intra-RAT scenarios.

The paper then makes one proposal: “Align Stage-2 with Stage-3 to allow the target RAT to add SCells for usage with the target PCell in inter-RAT handover scenarios (LTE SA to NR SA and vice-versa).”

Question 2: do you agree to align Stage-2 with Stage-3 to allow the target RAT to add SCells for usage with the target PCell in inter-RAT handover scenarios (LTE SA to NR SA and vice-versa).

| Answers to Question 2 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical justification |
| Qualcomm Incorporated | Yes |  |
| Ericsson (Tony) | No | We agree long time ago that stage-3 and stage-2 specification do not need to be fully aligned as far as the stage3 discuss is clear (or if in stage-2 there is something that is stated wrongly) This does not look the case and we would like to stick to this principle in order to avoid more CRs of this type in the future. We thinks the issue is not critical in this case. |
| Lenovo | Yes |  |
| OPPO | Yes |  |
| Intel | May be | We don’t need to capture every scenario in stage 2. Unless there is a risk of misunderstanding, an update to stage 2 is not so essential here. |
| Huawei, HiSilicon | No | We have similar views as Ericsson and Intel. |
| CATT | No | We share the similar view as Ericsson, Intel and HW. |
| Nokia | Yes | To Ericsson: please quote that agreement as this contradicts working procedures. The Stage 2 is not a TR and is therefore as relevant as others specifications. This has been clarified on numerous occasions in the past when such claims were made. [Ericsson] We do not have time to dig into very old agreements of (more) than two years ago. However, as an example, according to the AI of RAN2#105 (when late drop still needed to be frozen) you can clearly see (read) the principle we are talking about: ### 10.2.1 Stage 2 corrections for TS 38.300 CRs to correct errors in stage 2 are still appropriate, but CRs to tidy up the specifications or add additional cases covered by stage 3 but not stage 2 are no longer appropriate for Rel-15. As at previous meetings you should discuss your stage 2 CRs with the specification rapporteurs before submission. ### 10.2.2 Stage 2 corrections for TS 37.340 CRs to correct errors in stage 2 are still appropriate, but CRs to tidy up the specifications or add additional cases covered by stage 3 but not stage 2 are no longer appropriate for Rel-15. As at previous meetings you should discuss your stage 2 CRs with the specification rapporteurs before submission. True that now the RAN2 Chairman has changes, but we don’t think that these principles changed together with him. Anyway, we can clarify once again this with the current Chairman online, if needed. |
| MediaTek | Maybe not | As the stage 3 SPEC is clear, we don’t have to capture every details in stage 2. |
| vivo | no | And CR version is also not latest version. |
| Apple | Yes |  |
| Sequans | Yes |  |
| Samsung | No | It is nice to have aligned stage 2 and stage 3. However stage 3 is always more important and if it is already clear, alignment may not be essential. |

Summary 2: 13 companies have expressed an opinion and 6 companies believe Stage 2 should be aligned with Stage 3, 5 companies think no alignment is required, while 2 companies remain undecided. Thus, although there is a slight majority for clarifying something in the specification, it is not large enough to warrant Rel-15 CRs. Instead it is suggested to rely on capturing proposal 1 above in the Chairman’s minutes.

Proposal 2: rely on Chairman’s minutes capturing proposal 1 above and do not consider CRs further.

Finally, the paper suggests capturing in the Stage 2 (38.300 and 36.300) a single statement reading “At inter-RAT handover, the target network can add SCells for usage with the target PCell.”

Question 3: do you agree adding a statement to the Stage 2 as suggested in R2-2008817, R2-2008818, R2-2008819, R2-2008820

| Answers to Question 3 |  |  |
| --- | --- | --- |
| Company | Yes/No | Comments e.g. which Release, suggested updates, technical concerns… |
| Qualcomm Incorporated | Yes |  |
| Ericsson (Tony) | No | We agree long time ago that stage-3 and stage-2 specification do not need to be fully aligned as far as the stage3 discuss is clear (or if in stage-2 there is something that is stated wrongly) This does not look the case and we would like to stick to this principle in order to avoid more CRs of this type in the future. We thinks the issue is not critical in this case. |
| Lenovo | Yes |  |
| OPPO | Yes |  |
| Intel | May be with comments | As in our response to Q2, we don’t think it is essential. If a change is to be agreed, we suggest making it clear which scenario is really covered by the new sentence. For example, the proposed statement in 38.300 that “target RAT can add Scells” is not clear whether it is covering an inter-RAT HO to NR or from NR or both. |
| Huawei, HiSilicon | No | See our response to Q2. |
| CATT | No | See our response to Q2. |
| Nokia | Yes | See Q2. |
| MediaTek | Maybe not |  |
| vivo | No | And CR version is also not latest version. |
| Apple | Yes |  |
| Sequans | Yes |  |
| Samsung | No | It seems not an essential correction. |

Summary 3: 13 companies have expressed an opinion and 6 companies believes the CRs can be agreed, 5 companies think the opposite, while 2 companies remain undecided. Given proposal 2, the suggestion is not to consider these CRs further as there isn’t a strong enough majority to warrant Rel-15 changes.

# 3	UE Capabilities Description

CRs on UE Capabilities Description for 38.300 were contributed in R2-2009308, R2-2009309.

Question 4: do you agree with the need to have a high level description of UE Capabilities in the Stage 2.

| Answers to Question 4 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical justification |
| Qualcomm Incorporated | Yes | Proponent |
| Ericsson (Tony) | Yes | Proponent |
| Lenovo | No | The high level description as such looks ok, but out of context in stage 2. Therefore, we prefer to have such description in an informative annex in TS 38.331. |
| OPPO | Yes | Would be good to have such high level description, either in annex 331 or 300 is fine for us. |
| Intel | Yes |  |
| Huawei, HiSilicon | No | We think the original description in stage 2 has nothing broken, and it is not essential to add new text. |
| CATT | No | We don’t see any issue with current stage 2, so it’s not necessary to add new description. |
| Nokia | Yes | The UE capabilities are one of the most complex aspects of NR to grasp. Having Stage 3 text only has led to more confusion than necessary. |
| MediaTek | Yes | Good to have general introduction as the capability design is complicate in NR |
| vivo | Yes | We are ok with the high-level description; however, CR templet version should be updated to the latest version. |
| Apple | Yes | A high-level capability description is indeed somewhat missing in the current stage 2 spec. The LTE stage 2 spec has capability information as well, we think it is good to have it added to 38.300. |
| Sequans | Yes | We agree with the need to have such description. |
| Samsung | No | We are fine with having the high level description. But it seems requiring considerable effort for companies to converge. We don’t think we should spend our effort for nice-to-have. |

Summary 4: 13 companies have expressed an opinion and 9 companies believe inhaving a high level description of UE Capabilities in the Stage 2, while 4 companies do not.

Proposal 4: a high level description of the UE Capability framework is introduced in the Stage 2.

Question 5: if you agree with the need, do you agree with the CRs contributed in R2-2009308, R2-2009309.

| Answers to Question 5 |  |  |
| --- | --- | --- |
| Company | Yes/No | Comments e.g. which Release, suggested updates, technical concerns… |
| Qualcomm Incorporated | Yes | Proponent |
| Ericsson (Tony) | Yes | Proponent |
| Lenovo | No | See comments to Q4 above. Furthermore, in the description we think it might be good to clarify that a Feature Set refers to a set of baseband processing capabilities which are related to a band. |
| OPPO | No | “Those capability parameters are sent in feature set per component carrier (for UL and DL) and they are signalled in the corresponding FSs (per Band) i.e. for the corresponding block of contiguous serving cells in a band.” It’s not clear the different between “sent” and “signalled” “NOTE: For intra-band non-contiguous CA, there are as many feature sets per band signalled as the number of (contiguous groups of) carriers that the UE is able to aggregate non-contiguously in the corresponding band.” Not clear the intention of the note |
| Intel | No | We have a few comments on the CR text proposal and we think requires further discussion. For example: - The first part (“The UE capabilities in NR rely on a hierarchical structure..) is overlapped with the description in TS38.306 section 4.2.1. Furthermore, the description with “per band combinations, …” is more confusing. - The peak data rate is per carrier sounds misleading to us. TS 38.306 says “For NR, the approximate data rate for a given number of aggregated carriers in a band or band combination is computed as follows:”. In addition, we are not sure the following description is needed: “to the capabilities supported for that carrier which in turn depend (at least partly) on the band where that carrier is located”. It is confusing and the peak date rate equation in TS38.306 is clear enough. - Regarding the paragraph after the Figure 14.1: Is it about FSPC? If yes, we think it should be explicitly mentioned. - The Note is difficult to understand. |
| Huawei, HiSilicon | No | We think stage 2 should be kept in a high level description, and quite a few details is already covered in 38.306. We do not understand the note. |
| CATT | No | Agree with Huawei, since quite a few details is already covered in 38.306. |
| Nokia | Yes | Few answers to some of the points above: - OPPO: the purpose of the Note is to explain how intra band non-contiguous CA works since in the above paragraph we only mention contiguous CA. Do you think this is not correct? - Intel: removing that sentence (similar to 306) would give us something illegible. - Intel: more confusing than? It is a high level descriptions giving all the variables. Do you have a better wording in mind? - Intel: arguing that we have all details in Stage 3 to avoid a Stage 2 description would mean that we don’t need a Stage 2 at all. - Intel: the text already reads “feature set per component carrier” - Huawei and CATT: since it is important to establish a common understanding, please let us know if you think the suggested text is technically correct. As it stands now, it reflects the understanding of the co-sourcing companies of how the capabilities framework works. |
| MediaTek | Yes |  |
| vivo | Yes with comments | 1 the Text is technically correct; however, CR templet version should be updated to the latest version. 2 I understand the intention; however, I want to clarify which part in stage3 specification can map to the below note? “NOTE: For intra-band non-contiguous CA, there are as many feature sets per band signalled as the number of (contiguous groups of) carriers that the UE is able to aggregate non-contiguously in the corresponding band.” 3 Considering multiple Feature set, “s” should be added in the picture, i.e., “Feature sets per band”, |
| Apple | Almost | We see this as a first step to cover the high-level conceptual view of UE capabilities and the stage 2 can further enhance over time. - We are generally fine with a list of parameter signalling associations like per UE, per FR, per band, per BC, per FS, per FSPC, but the “…” is a little confusing and we suggest removing it. - The first note could be more specific, essentially all UE capabilities are UE specific – it may as well go in a second sentence instead of a note; however, we are fine with the intention. - The text around the feature set combinations is useful. |
| Sequans | Yes |  |
| Samsung | No | See our previous response. |

Summary 5: amongst the companies willing to update the Stage 2, 4 companies are happy with the suggested CR (R2-2009308, R2-2009309) while 5 would like to work further on it.

Proposal 5: continue working on R2-2009308 in order to obtain an agreeable version.

Another set of CRs was also contributed to 37.340 to update the existing wording (R2-2008821, R2-2008822)

Question 6: do you agree with the CRs contributed in R2-2008821 and R2-2008822.

| Answers to Question 6 |  |  |
| --- | --- | --- |
| Company | Yes/No | Comments e.g. which Release, suggested updates, technical concerns… |
| Qualcomm Incorporated | Yes |  |
| Ericsson (Tony) | Yes |  |
| Lenovo | Yes, but | A pointer to Stage-3 NR should be added, see comments to Q4 above. Furthermore, the category of the R16 CR should be cat A. |
| OPPO | Yes |  |
| Intel | Yes |  |
| Huawei, HiSilicon | Yes |  |
| CATT | Yes |  |
| Nokia | Yes |  |
| MediaTek | Yes |  |
| vivo | Yes | CR templet version should be updated to the latest version. |
| Apple | Yes |  |
| Sequans | Yes |  |
| Samsung | No | It is neither an essential correction nor important clarification. |

Summary 6: 13 companies have expressed an opinion and all agreed but one with the suggested CRs.

Proposal 6: agree R2-2008821 as such and R2-2008822 after updating the category (A instead of F).

# 3	Cell Terminology

The Stage 2 Rapporteurs (Nokia & ZTE) were tasked at the last meeting to review the cell terminology. The result of that exercise are the two CRs contributed in R2-2009310 and R2-2009311.

Question 7: do you agree with the CRs contributed in R2-2009310 and R2-2009311.

| Answers to Question 7 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical justification |
| Qualcomm Incorporated | No | We tend to think that the notion of SpCell is rather important terminology in the context of lower layer procedures, like MAC. Also in some cases, mentioning of PSCell indicates that the feature is supported in SCG, e.g. the change to 5.6.1 in release-16 CR somehow make the SCG context disappear. Release-16 CR should be a Cat.F CR. |
| Ericsson (Tony) | No | We think that CR is not critical and does not introduce any clear benefits. Further, some changes are wrong as for that one in the PDCP duplication section. The CA duplication can indeed be enabled at the MCG and SCG. When CA happens on the SCG one of the logical channel should be linked to the PSCell. |
| OPPO | No | We understand the intention is to make 38.300 only capture non-DC part and let 37.340 capturing DC related part. However, we think even with the statement in the beginning ,we may lost some information if we remove PSCell or replacing SpCell with PCell. So, is it possible we just add the statement in 4.5, and keep other parts unchanged. |
| Intel | No (as proposed) | We think the proposed text does not add clarity. For example “does not necessarily apply” is too vague. The reference to “This statement” is also not clear which statement is being referred to. |
| Huawei, HiSilicon | No | We think the update is not essential and might create new confusions. For CA duplication, we have same understanding that it can be configured at MCG or SCG. |
| CATT | No | We also think the update of the cell terminology is not necessary and cause some confusions. For example of the changes to 5.4.1 and 5.6.1 in Rel-16 which remove the terminology PSCell, it seems noting related with SCG. |
| Nokia | Yes | To Ericsson and CATT: please read the addition to 4.5, not individual changes in isolation. To others: what is now the understanding for all statements related to PCell only? What should we do with the SpCell ones? The aim was to remove all ambiguities. [Ericsson] Even with the addition in section 4.5, still is a bit odd that we say the CA duplication is applied only to the MCG. We believe that current text is much clearer and does not leave any room for ambiguity. Same applies to other parts of the spec where is mentioned PCell, but in reality, the same principle applies also to PSCell. This just create more confusion. |
| MediaTek | May not | The intention is fine but we think it is not critical. |
| vivo | Yes | We agree with the intention. CR templet version should be updated to the latest version. |
| Apple | No | While we agree with the intention to keep 38.300 agnostic to DC, we think it is a bit late for a change of terminology. The term SpCell is also widely used in 38.331, it may create confusion to use PCell in a slightly different way in 38.300. Further, the note added to sub-clause 4.5 can be easily missed by a reader already familiar with the existing terminology, which can lead to misinterpretation. To avoid ambiguities where e.g. PCell can be equally understood as SpCell, perhaps 38.300 could use SpCell directly and otherwise use PCell for “PCell only” (i.e. PCell in SA or PCell in NR-DC), “PCell/SpCell” for statements common to NR SA and NR-DC, “PSCell” for the PSCell only, etc. |
| Sequans | Maybe | Agree with the intention to remove ambiguity, but no strong view, as this change requires some additional wording when we want to refer to PCell of MCG only. |
| Samsung | No | We appreciate Rapporteur companies effort to make the spec clean. However, there seem very little room for misunderstanding and it is not right time to enhance stage 2 spec quality, which is already good. |

Summary 7: 13 companies have responded and 8 companies do not support the CRs, two companies do while two companies remain undecided.

Proposal 7: leave 38.300 as is and do not update the Cell Terminology.

# 4	Conclusion

The proposals from this email discussions are:

Proposal 1: acknowledge that current Stage-3 specifications allow the target RAT to add SCells for usage with the target PCell in inter-RAT handover scenarios (LTE SA to NR SA and vice-versa); and the intention of LTE and NR Stage-2 is not to restrict SCells addition only for intra-RAT scenarios.

Proposal 2: rely on Chairman’s minutes capturing proposal 1 above and do not consider CRs further.

Proposal 4: a high level description of the UE Capability framework is introduced in the Stage 2.

Proposal 5: continue working on R2-2009308 in order to obtain an agreeable version.

Proposal 6: agree R2-2008821 as such and R2-2008822 after updating the category (A instead of F).

Proposal 7: leave 38.300 as is and do not update the Cell Terminology.