3GPP TSG-RAN2 Meeting #112e	R2-2011063

# Electronic, 2nd November - 13th November 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.323 | CR | 0291 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on LTE PDCP re-establishment for UM DRB when t-Reordering is used |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT-Core |  |  |  |  |  | Date: |  |  | 2020-11-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In RAN2#111e, RAN2 agreed to specify the behavior for AM DRB (as specified in R2-2008541) at PDCP re-establishment procedure when t-Reordering is used since t-Reordering had been allowed to be used for normal DRBs configured with rlc-OutOfOrderDelivery by R2-2004818 and R2-2004826, which can be also configured with Ethernet header compression. In the current PDCP specification, for UM DRB, PDCP re-establishment procedure when t-Reordering is used does not specify the process of EHC protocol, which is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For UM DRB, to specify the process of EHC protocol for PDCP re-estasblishment when t-Reordering is used if the associated RLC entity is configured with rlc-OutOfOrderDelivery. Impact Analysis for archicture option The impacted 5G architecture option is EN-DC. Impacted functionality: PDCP re-establishment for UM DRB when t-Reordering is used Inter-operability: If the network is implemented according to the CR while the UE is not, the EHC protocol will not be reset at UE side if drb-ContinueEHC-DL is not configured and if the associated RLC entity is configured with rlc-OutOfOrderDelivery. If the UE is implemented according to the CR while the network is not, the network may assume that the EHC protocol cannot be reset if the associated RLC entity is configured with rlc-OutOfOrderDelivery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For UM DRB, PDCP re-establishment when t-Reordering is used will not reset the EHC protocol at UE side if drb-ContinueEHC-DL is not configured and if the associated RLC entity is configured with rlc-OutOfOrderDelivery, which is not aligned with the network configuration and its intention and may have a bad impact on EHC protocol. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

5.2.2.2a	Procedures for DRBs mapped on RLC UM when the reordering function is used

When upper layers request a PDCP re-establishment when the reordering function is used, the UE shall:

-	process the PDCP Data PDUs that are received from lower layers due to the re-establishment of the lower layers, as specified in the clause 5.1.2.1.4;

-	stop and reset t-Reordering, if running;

-	deliver all stored PDCP SDUs, if any, to upper layers in ascending order of associated COUNT values;

- 	if the PDCP entity is associated with at least one RLC entity configured with rlc-OutOfOrderDelivery:

-	reset the EHC protocol for downlink (if configured) if drb-ContinueEHC-DL is not configured, see TS 36.331 [3];

-	set Next_PDCP_RX_SN, and RX_HFN to 0 and Last_submitted_PDCP_RX_SN to Maximum_PDCP_SN;

-	apply the ciphering algorithm and key provided by upper layers during the re-establishment procedure.

<End of modification>