3GPP TSG-RAN2 Meeting #112e	R2- 2011080

# Electronic, 2nd November - 13th November 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | 1511 | rev | 1 | Current version: | 15.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on lch-CellRestriction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15, LTE_HRLLC-Core |  |  |  |  |  | Date: |  |  | 2020-11-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In RAN2#100, RAN2 made the following agreements from Rel-15 HRLLC WI: Agreements: 1 PDCP data duplication for LTE shall assume NR PDCP data duplication as baseline. 2 RAN2 works on PDCP data duplication for both CA and DC. 3a At least UM bearers are supported for PDCP duplication via CA. 4 PDCP enables reordering and duplication detection when PDCP duplication is configured. 6 MAC CE is used for activation and deactivation of PDCP duplication for each RB configured with duplication. 7 For CA case, LCP applies configured LCH to carriers/cells restriction for LCHs of a duplication RB and the restriction is lifted when duplication is deactivated as agreed in NR. 8 PDCP duplication is configured by RRC. The configuration also indicates whether the duplication is immediately started, which is the same as NR. 9 LCH to carriers/cells restriction is configured for CA duplication.. In the current MAC specification, the agreements (i.e. 7 and 9 above) have not been specified correctly. Rather, it specified unintended behavior as follows: “if a logical channel has been configured with lch-CellRestriction and if PDCP duplication is activated, for this logical channel the MAC entity shall not consider the cells indicated by lch-CellRestriction to be restricted for transmission.” |
| Summary of change: |  | To make lch-CellRestriction related behaviour aligned with the agreements. Impact Analysis for archicture option The impacted 5G architecture options are EN-DC and NE-DC. Impacted functionality: lch-CellRestriction in LCP procedure. Inter-operability: If the network is implemented according to the CR while the UE is not, UE does not apply lch-CellRestriction according to RRC configuration, if PDCP duplication is activated. If the UE is implemented according to the CR while the network is not, the network assuming the current UE behavior may configure lch-CellRestriction, incorrectly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE does not follow network configuration regarding lch-CellRestriction, if PDCP duplication is activated |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>\

5.4.3.1	Logical channel prioritization

The Logical Channel Prioritization procedure is applied when a new transmission is performed.

RRC controls the scheduling of uplink data by signalling for each logical channel: priority where an increasing priority value indicates a lower priority level, prioritisedBitRate which sets the Prioritized Bit Rate (PBR), bucketSizeDuration which sets the Bucket Size Duration (BSD), and optionally allowedTTI-Lengths which sets the allowed TTI lengths. For NB-IoT, prioritisedBitRate, bucketSizeDuration and the corresponding steps of the Logical Channel Prioritisation procedure (i.e., Step 1 and Step 2 below) are not applicable.

The MAC entity shall maintain a variable Bj for each logical channel j. Bj shall be initialized to zero when the related logical channel is established, and incremented by the product PBR × TTI duration for each TTI, where PBR is Prioritized Bit Rate of logical channel j. However, the value of Bj can never exceed the bucket size and if the value of Bj is larger than the bucket size of logical channel j, it shall be set to the bucket size. The bucket size of a logical channel is equal to PBR × BSD, where PBR and BSD are configured by upper layers.

The MAC entity shall perform the following Logical Channel Prioritization procedure when a new transmission is performed on an UL grant with a certain TTI length:

-	The MAC entity shall allocate resources to the logical channels that are allowed to transmit using the TTI length of the grant, in the following steps:

-	Step 1: All the allowed logical channels with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to "infinity", the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s);

-	Step 2: the MAC entity shall decrement Bj by the total size of MAC SDUs served to logical channel j in Step 1;

NOTE 1:	The value of Bj can be negative.

-	Step 3: if any resources remain, all the allowed logical channels are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally.

-	The UE shall also follow the rules below during the scheduling procedures above:

-	the UE should not segment an RLC SDU (or partially transmitted SDU or retransmitted RLC PDU) if the whole SDU (or partially transmitted SDU or retransmitted RLC PDU) fits into the remaining resources of the associated MAC entity;

-	if the UE segments an RLC SDU from the logical channel, it shall maximize the size of the segment to fill the grant of the associated MAC entity as much as possible;

-	the UE should maximise the transmission of data.

-	if the MAC entity is given an UL grant size that is equal to or larger than 4 bytes while having data available for transmission, the MAC entity shall not transmit only padding BSR and/or padding (unless the UL grant size is less than 7 bytes and an AMD PDU segment needs to be transmitted);

-	for transmissions on serving cells operating according to Frame Structure Type 3, the MAC entity shall only consider logical channels for which laa-UL-Allowed has been configured;

-	if a logical channel has been configured with lch-CellRestriction and if PDCP duplication within the same MAC entity (i.e. CA duplication) is activated, for this logical channel the MAC entity shall consider the cells indicated by lch-CellRestriction to be restricted for transmission.

-	for NB-IoT UEs, BL UEs or UEs in enhanced coverage, if edt-SmallTBS-Enabled is set to TRUE for the corresponding PRACH resource, the UE shall choose a TB size among the set of possible TB sizes as described in clauses 8.6.2 and 16.3.3 of TS 36.213 [2]

The MAC entity shall not transmit data for a logical channel corresponding to a radio bearer that is suspended (the conditions for when a radio bearer is considered suspended are defined in TS 36.331 [8]).

If the MAC PDU includes only the MAC CE for padding BSR or periodic BSR with zero MAC SDUs and there is no aperiodic CSI requested for this TTI, as specified in TS 36.213 [2], the MAC entity shall not generate a MAC PDU for the HARQ entity in the following cases:

-	in case the MAC entity is configured with skipUplinkTxDynamic and the grant indicated to the HARQ entity was addressed to a C-RNTI; or

-	in case the MAC entity is configured with skipUplinkTxSPS and the grant indicated to the HARQ entity is a configured uplink grant activated by the MAC entity's Semi-Persistent Scheduling C-RNTI or by the MAC entity's UL Semi-Persistent Scheduling V-RNTI; or

-	in case the grant indicated to the HARQ entity is a configured uplink grant activated by the MAC entity's AUL C-RNTI.

NOTE 1a:	If at least one MAC PDU is to be generated for the HARQ entity for this TTI, the MAC entity generates MAC PDUs corresponding to all UL grants indicated to the HARQ entity for this TTI.

For the Logical Channel Prioritization procedure, the MAC entity shall take into account the following relative priority in decreasing order:

-	MAC control element for C-RNTI or data from UL-CCCH;

-	MAC control element for DPR;

-	MAC control element for SPS confirmation;

-	MAC control element for AUL confirmation;

-	MAC control element for BSR, with exception of BSR included for padding;

-	MAC control element for PHR, Extended PHR, or Dual Connectivity PHR;

-	MAC control element for Sidelink BSR, with exception of Sidelink BSR included for padding;

-	data from any Logical Channel, except data from UL-CCCH;

-	MAC control element for Recommended bit rate query;

-	MAC control element for BSR included for padding;

-	MAC control element for Sidelink BSR included for padding.

NOTE 2:	When the MAC entity is requested to transmit multiple MAC PDUs in one TTI, steps 1 to 3 and the associated rules may be applied either to each grant independently or to the sum of the capacities of the grants. Also the order in which the grants are processed is left up to UE implementation. It is up to the UE implementation to decide in which MAC PDU a MAC control element is included when MAC entity is requested to transmit multiple MAC PDUs in one TTI. When the UE is requested to generate MAC PDU(s) in two MAC entities in one TTI, it is up to UE implementation in which order the grants are processed.

<End of modification>