3GPP TSG-RAN WG2 Meeting #112e	R2-2011110

# Electronic Meeting, November 2nd – 13th, 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0475 | rev | - | Current version: | 15.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Dummify UE capability of crossCarrierScheduling-OtherSCS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-10-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In Rel-15 the UE capability of crossCarrierScheduling-OtherSCS was not used and noted as “Cross-carrier scheduling with different numerologies is not supported in this release of specification.”. In Rel-16 the UE capabilities of crossCarrierSchedulingDL-DiffSCS/crossCarrierSchedulingUL-DiffSCS are introduced for Cross-carrier scheduling with different numerologies. Therefore, RAN2 agree to dummify the Rel-15 UE capability of crossCarrierScheduling-OtherSCS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Dummifying Rel-15 UE capability of crossCarrierScheduling-OtherSCS in FeatureSetDownlink parameters and FeatureSetUplink parameters. Impact Analysis Impacted 5G architecture options: NR-SA, NR-DC, (NG)EN-DC, NE-DC Impacted functionality: Cross-carrier scheduling with different numerologies in NR Cell Group Inter-operability: There is no inter-operability problems as the Rel-15 UE capability of crossCarrierScheduling-OtherSCS was not used since Rel-15. This CR is considered mandatory to support the impacted functionality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how UE to set the Rel-15 and Rel-16 UE capabilty bits for Cross-carrier scheduling with different numerologies. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.5, 4.2.7.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS 38.331 CR 2286 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 4.2.7.5	FeatureSetDownlink parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |

Next Change

#### 4.2.7.7	FeatureSetUplink parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |