3GPP TSG-RAN WG2 #112 electronic	R2-2011194

Electronic Meeting, Nov 2-13, 2020

Agenda Item:	6.8.4

Source:	vivo

Title:	[AT112-e][225][NR][DCCA] Correction on FR2 maximum power for NR-DC power control (vivo)

Document for:	Discussion, Decision

1	Introduction

This contribution summarizes the following discussion:

[AT112-e#225][NR][DCCA] Correction on FR2 maximum power for NR-DC power control (vivo)

Scope:

    - Provide CRs on FR2 power limit based on RAN4 LS band Tdocs R2-2010291, R2-2010112, and R2-2010340.

Intended outcome:

    - Agreeable CRs to 38.331 in R2-2010743 (revision of R2-2010291)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Final CRs: 2nd week Wed, UTC 1100

The rapporteur proposes to split this email discussion in two phases:

- Companies are invited to provide feedback on the questions proposed in this document.

Deadline: 2020-11-10

- The rapporteur proposes a draft CR to capture the agreements based on companies’ feedback. Then companies can comment on the draft CR.

Deadline: 2020-11-11

Contact from companies

| Company | Email |
| --- | --- |
| OPPO | wangshukun@oppo.com |
| Nokia | Jarkko.t.koskela@nokia.com |
| ZTE | liu.jing30@zte.com.cn |
| vivo | wenjuan.pu@vivo.com |
| Ericsson | Stefan.wager@ericsson.com |

- Discussion

In the RAN4 LS R2-2008736 (R4-2011721), it is stated that:

“For newly introduced p-UE-FR2 in the RRCReconfiguration message, RAN4 decides not to use this parameter in Rel-16. Further discussion on this topic would be postponed to Rel-17.”

During the online discussion, the following agreement for p-UE-FR2 was made, and there are two remain issues need to be discussed.

We keep the field (i.e. not dummify) Discuss whether inter-node message information is needed Discuss if the same should apply also for p-NR-FR2

Thus, this offline will separately discuss whether p-maxUE-FR2 in the inter-node message and p-NR-FR2 in RRCReconfiguration message also need some similar clarification.

2.1	p-maxUE-FR2 in inter-node message

Since p-UE-FR2 and p-maxUE-FR2 has the same meaning / usage for NR-DC power control on FR2, but in the different message, i.e., the former is included in RRCRecofniguration to be sent to the UE and the latter is used to be indicated to SN by MN via CG-ConfigInfo inter-node message. Thus, as proposed in R2-2010340, it is straightforward to add the similar clarification for the p-maxUE-FR2 within the CG-ConfigInfo, as follows:

p-maxUE-FR2 Indicates the maximum total transmit power to be used by the UE across all serving cells in frequency range 2 (FR2). In this release of the specification, it shall be ignored if received.

Q1: Do companies agree with the change above?

| Company | Yes or No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| Nokia | Yes |  |
| ZTE | Yes with comment | Besides p-maxUE-FR2, should we also clarify the below two fields (relates to Q2)? powerCoordination-FR2-r16 SEQUENCE { p-maxNR-FR2-MCG-r16 P-Max OPTIONAL, p-maxNR-FR2-SCG-r16 P-Max OPTIONAL, p-maxUE-FR2-r16 P-Max OPTIONAL } OPTIONAL, |
| vivo | Yes |  |
| Ericsson | Yes, with comment | We agree with ZTE that the same sentence shall also be added to p-maxNR-FR2-MCG and p-maxNR-FR2-SCG, alternatively to the whole powerCoordination-FR2. |

Summary：

According to the companies’ feedback, the above change for p-maxUE-FR2 can be agreed.

Proposal 1: to add the same clarification for p-maxUE-FR2, i.e., this field is not used in this release of the specification.

2.2	p-NR-FR2 in RRCReconfiguration message

NR-DC power control on FR2 was discussed and agreed in RAN1. For P-NR-FR2, it was introduced in PhysicalCellGroupConfig for NR-DC power control on FR2 to configure the maximum transmit power allowed in MCG FR2 or SCG FR2, which is based on the below RAN1#99 meeting agreement.

| LTE_NR_DC_CA_enh-Core | NR-DC | PhysicalCellGroupConfig | P-NR-FR2 | New | Maximum transmit power allowed in a cell group's FR2 carriers |
| --- | --- | --- | --- | --- | --- |

And RAN1 has specified this filed P-NR-FR2 and the corresponding NR-DC power control on FR2 mechanism in TS 38.213-g30, as follows. More detail can be found in the Appendix.

### 7.6.2 NR-DC <Omit> If a UE is configured with an MCG and a SCG using NR radio access in FR1 and/or in FR2, the UE is configured a maximum power PMCG for transmissions on the MCG by p-NR-FR1 and/or by p-NR-FR2-r16 and a maximum power PSCG for transmissions on the SCG by p-NR-FR1 and/or by p-NR-FR2-r16 and with an inter-CG power sharing mode by nrdc-PCmode-FR1-r16 for FR1 and/or by nrdc-PCmode-FR2-r16 for FR2. The UE determines a transmission power on the MCG and a transmission power on the SCG per frequency range. <Omit>

We think this field is still valid and would be used to support NR-DC power control on FR2, and thus there seems no need to modify the description/usage of this filed.

Q2: Do companies agree that there is no need to modify the description/usage of p-NR-FR2 in RRCReconfiguration?

| Company | Yes or No | Comments |
| --- | --- | --- |
| OPPO | Yes | Confirmed with our RAN4 colleague. |
| Nokia | Yes |  |
| ZTE | Not sure | Our RAN4 actually consider this is similar to p-UE-FR2, that will not be used as well. |
| vivo | Yes | Confirmed with our RAN4 colleague, RAN4 have no concern for p-NR-FR2. Confirmed with our RAN1 colleague, p-NR-FR2 was defined in RAN1 and has been captured in RAN1 specification. If the similar clarification of p-UE-FR2 is applied to p-NR-FR2, then RAN1 specified NR-DC PC on FR2 will not work. Thus, there is no need to modify the description/usage of p-NR-FR2. Also, there is no need to modify the description/usage of p-maxNR-FR2-MCG-r16/ p-maxNR-FR2-SCG-r16 in the inter-node message. |
| Ericsson | No | Our understanding is same as ZTE after discussion with our RAN4 colleagues. This relates also to offline discussion #005, in which it is clarified that p-max is not used for FR2. Our understanding is that the UE will not be able to limit its FR2 output power according to a configured p-max limit in Rel-15/16. It does not matter if it’s per cell, cell group or UE. Only the power limits in 38.101-2 apply. |

Summary:

According to the companies’ feedback, there is no consensus on whether the same clarification should be applied to p-NR-FR2. And if it is necessary, the clarification should be also applied for p-maxNR-FR2-MCG-r16 and p-maxNR-FR2-SCG-r16 in powerCoordination-FR2. It seems RAN2 cannot conclude it since the field p-NR-FR2 was defined in RAN1 and it is not clear whether RAN4 has any concern about the field. Thus, rapporteur suggests:

Proposal 2: send a Ls to RAN1 and RAN4, to ask for whether there is any concern about p-NR-FR2 to be used (or not used) in Rel-16. The Ls can also Cc RAN1, for RAN1 may update their specification accordingly.

3	Conclusion

Proposal 1: to add the same clarification for p-maxUE-FR2, i.e., this field is not used in this release of the specification.

Proposal 2: send a Ls to RAN4 to ask for whether they have any concern about p-NR-FR2 to be used in Rel-16. The Ls can also Cc RAN1, for RAN1 may update their specification accordingly.

4	References

[1] R2-2008736, Reply LS on power control for NR-DC (R4-2011721; contact: vivo), RAN4 (R4-2011721)

[2] R2-2010112, Correction on p-UE-FR2 for NR-DC power control	Ericsson, NTTDOCOMO	CR	Rel-16	38.331	16.2.0	2165	1	F	LTE_NR_DC_CA_enh-Core	R2-2010027

[3] R2-2010340, Correction on p-UE-FR2 for NR-DC power control in FR2	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2207	-	F	LTE_NR_DC_CA_enh-Core

[4] R2-2010291, Correction on p-UE-FR2 in NR-DC power control	vivo	CR	Rel-16	38.331	16.2.0	2201	-	F	LTE_NR_DC_CA_enh-Core

5	Appendix

7	Uplink Power control (TS 38.213)

<Omit>

7.6	Dual connectivity

<Omit>

7.6.2	NR-DC

If a UE is configured with an MCG using NR radio access in FR1 or in FR2 and with a SCG using NR radio access in FR2 or in FR1, respectively, the UE performs transmission power control independently per cell group as described in Clauses 7.1 through 7.5.

If a UE is configured with an MCG and a SCG using NR radio access in FR1 and/or in FR2, the UE is configured a maximum power PMCG for transmissions on the MCG by p-NR-FR1 and/or by p-NR-FR2-r16 and a maximum power PSCG for transmissions on the SCG by p-NR-FR1 and/or by p-NR-FR2-r16 and with an inter-CG power sharing mode by nrdc-PCmode-FR1-r16 for FR1 and/or by nrdc-PCmode-FR2-r16 for FR2. The UE determines a transmission power on the MCG and a transmission power on the SCG per frequency range.

If a UE is provided semi-static-mode1 for nrdc-PCmode-FR1-r16 or for nrdc-PCmode-FR2-r16, or semi-static-mode2 for nrdc-PCmode-FR1-r16 or for nrdc-PCmode-FR2-r16, the UE does not expect PMCG and PSCG to be configured such that PMCG+PSCG>PTotalNR-DC, where PMCG is the linear value of PMCG, PSCG is the linear value of PSCG, and PTotalNR-DC is the linear value of a configured maximum transmission power for NR-DC operation in FR1 or FR2 as defined in [8-3, TS 38.101-3].

If a UE is provided semi-static-mode1 for nrdc-PCmode-FR1-r16 or for nrdc-PCmode-FR2-r16, the UE determines a transmission power for the MCG or for the SCG as described in Clauses 7.1 through 7.5 using PMCG or PSCG as the maximum transmission power, respectively.

If a UE is provided semi-static-mode2 for nrdc-PCmode-FR1 or for nrdc-PCmode-FR2

-	if at least one symbol of slot i1 of the MCG or of the SCG that is indicated as uplink or flexible to a UE by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, if provided, overlaps with a symbol for any ongoing transmission overlapping with slot i2 of the SCG or of the MCG, respectively, the UE determines a power for the transmission on the SCG or the MCG overlapping with slot i2 as described in Clauses 7.1 through 7.5 using PSCG or PMCG, respectively, as the maximum transmission power

-	otherwise, the UE determines a power for the transmission on MCG or the SCG overlapping with slot i2, as described in [8-3, TS 38.101-3] and in Clauses 7.1 through 7.5 without considering PMCG or PSCG, respectively

The UE expects to be provided semi-static-mode2 for nrdc-PCmode-FR1-r16 or for nrdc-PCmode-FR2-r16  only for synchronous NR-DC operation [10, TS 38.133].

If a UE

-	is provided dynamic for nrdc-PCmode-FR1-r16 or for nrdc-PCmode-FR2-r16, and

-	indicates a capability to determine a total transmission power on the SCG at a first symbol of a transmission occasion on the SCG by determining transmissions on the MCG that

-	are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by more than Toffset from the first symbol of the transmission occasion on the SCG, or are configured by higher layers, and

-	overlap with the transmission occasion on the SCG

the UE determines a maximum transmission power on the SCG at the beginning of the transmission occasion on the SCG as

-	minPSCG,PTotalNR-DC- PMCGactual, if the UE determines transmissions on the MCG with a PMCGactual total power

-	PTotalNR-DC, if the UE does not determine any transmissions on the MCG

where

-	Toffset=max⁡{Tproc,MCGmax,Tproc,SCGmax},

-	Tproc,MCGmax and Tproc,SCGmax is the maximum of Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux, and Tproc,CSImux based on the configurations on the MCG and the SCG, respectively, when the UE indicates the value of 'long' for the capability,

-	Tproc,MCGmax and Tproc,SCGmax is the maximum of Tproc,2, Tproc,releasemux, Tproc,2mux based on the configurations on the MCG and the SCG, respectively, when the UE indicates the value of 'short' for the capability, and

-	PMCGactual is the total power for the transmissions on the MCG that overlap with the transmission occasion on the SCG where PMCGactual is determined based on transmissions configured by higher layers and on transmissions scheduled by DCI formats in PDCCH receptions with a last symbol that is at least Toffset before the first symbol of the transmission occasion on the SCG.

The UE does not expect to have PUSCH, PUCCH, or SRS transmissions on the MCG that

-	are scheduled/triggered by DCI formats in PDCCH receptions with a last symbol that is earlier by less than or equal to Toffset from the first symbol of the transmission occasion on the SCG, and

-	overlap with the transmission occasion on the SCG

The UE does not expect to receive a positive TPC command value in a DCI format 2_2 or a DCI format 2_3 in a PDCCH reception with a last symbol that is less than Toffset before the first symbol of the transmission occasion on the SCG, if the transmission on the MCG overlaps with the transmission occasion on the SCG.

The UE is not required to apply a TPC command the UE receives in a DCI format 2_2 or a DCI format 2_3 in a PDCCH reception with a last symbol that is less than Toffset before the first symbol of the transmission occasion on the SCG, if the transmission on the MCG overlaps with the transmission occasion on the SCG.