3GPP TSG-RAN WG2 Meeting #112 electronic	R2-20xxx

Online, November 2nd - 13th, 2020

Agenda Item:	6.2.4

Source: 	Huawei, HiSilicon

Title:	Summary of [AT112-e][019][IAB] NR RRC 38331

Document for:	Discussion and Decision

# Introduction

This paper aims at capturing the summary of the following offline discussion:

- [AT112-e][019][IAB] NR RRC 38331 (Huawei)

Treat 38331 tdocs under 6.2.4

Intended outcome: Intermediate: Determine agreeable parts. Final: For agreeable parts, agreed CRs.

Deadline: Intermediate deadline(s) by Rapporteur, Final: Discussion stop at Wed Nov 11, 1200 UTC

# Discussion on correction or clarification

- 2.1: Miscellaneous

R2-2010149	RRC Miscellaneous Corrections	Ericsson, Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2184	-	F	NR_IAB-Core

R2-2009323	Miscellaneous corrections to TS 38.331 for IAB	vivo	CR	Rel-16	38.331	16.2.0	2054	-	F	NR_IAB-Core

R2-2010638	Miscellaneous corrections for IAB  	Samsung R&D Institute UK	CR	Rel-16	38.331	16.2.0	2266	-	F	NR_IAB-Core

R2-2009005	Correction on RRC function description for IAB	Fujitsu	CR	Rel-16	38.331	16.2.0	2025	-	F	NR_IAB-Core

R2-2009750	Corrections on intra-donor CU RLF recovery and RLF cause determination	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2125	-	F	NR_IAB-Core

[Changes in 5.3.7.2 and 5.3.10.4 will be discussed separately]

Rapporteurs’ view: Those above changes can be considered as miscellaneous corrections and be merged as one CR. The wording will be checked in phase2 offline.

Except following changes:

Change 1: (R2-2009323) Add “This field is also used to indicate the minimum IAB-MT capabilities set that the IAB-MT shall support as defined in TS 38.306 [26].” to the field description of iab-NodeIndication.

[Rapp]: The minimum mandatory IAB-MT capabilities is clear in 38.306. And the current field description is clear that this field is to identify the IAB-MT. Companies’ views are welcome to see if the clarification is needed.

Change 2: (R2-2009323) Change “Need R” to “Need S” for iab-Support-r16

[Rapp]: The intention is correct. But it is worth for companies to check whether this is a BC change and agreeable.

Change 3: (R2-2009005) Clarify the allocation of a priority and a prioritised bit rate (PBR) is not for each RB in IAB-MT.

[Rapp]: The 1st change is not needed, since “UE” also refers to IAB-MT unless specific statement. For the “RB” change, in order to avoid any changes to R15 UEs, rapporteur proposes to make the clarification only for IAB-MT, i.e. “allocation of a priority and a prioritised bit rate (PBR) for each RB of UE and logical channel of IAB-MT.”

Change 4: (R2-2009750) Add missing “Need R” for the condition TDD_IAB

[Rapp]: Worth to collect companies’ view.

Question 1: Do you agree with the above 4 changes.

| Company | Agree to Change 1? | Agree to Change 2? | Agree to Change 3 (Rapp’s wording)? | Agree to Change 4? | Comments |
| --- | --- | --- | --- | --- | --- |

- 2.2: RLF cause

R2-2009390	CR for TS38.331 on RLF cause for IAB BH RLF	ZTE, Sanechips	CR	Rel-16	38.331	16.2.0	2062	-	F	NR_IAB-Core

R2-2010602	Cause value due to the reception of BH RLF indication	Lenovo, Motorola Mobility	CR	Rel-16	38.331	16.2.0	2257	-	F	NR_IAB-Core

R2-2009750	Corrections on intra-donor CU RLF recovery and RLF cause determination	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2125	-	F	NR_IAB-Core

[Only the change in 5.3.10.4]

Rapporteur’s view: Those 3 CRs seem address the similar issue of RLF cause determination. The intention is clarify the BH RLF is due to “link recovery failure at the IAB node’s parent node”, while for this IAB it is just RLF.

We have two parts of the changes. If only part 1 is agreeable, we can use R2-2009750 as baseline. If only part 2 is agreeable, we can use R2-2009390 as baseline. If both parts are agreeable, we can use R2-2010602 as baseline in phase 2.

Part 1: Procedure text: “1>	else if the IAB-MT declares radio link failure due to the reception of BH RLF indicationRLF recovery failure over the backhaul link”;

Part 2: ASN.1 naming: In rlf-Cause-r16, change the value “bh-rlfRecoveryFailure” into, for example, “bh-rlf”.

Question 2: Do you agree the intention of above part 1/2 changes, assuming the wording can be updated, if needed in phase 2 discussion?

| Company | Agree to part 1? | Agree to part 2? | Comment |
| --- | --- | --- | --- |

- 2.3: Other separated issues

- Issue: Suspend BH RLC at MCG/SCG Failure

R2-2010635	Transmission suspension on BH RLC channel upon IAB-MT failure 	Samsung R&D Institute UK	CR	Rel-16	38.331	16.2.0	2265	-	F	NR_IAB-Core

Rapporteur’s view: This is a new behaviour to suspend BH RLC channel in case MCG/SCG failure. Some questions, e.g. how to specify the behaviour of suspending at RLC layer and what’s the difference with suspending BAP layer, need to be clarified. Also, companies may need to clarify why this is not an implementation issue.

Question 3: Do you agree the intention of R2-2010635, i.e. suspend BH RLC at MCG/SCG failure?

| Company | Agree? | Comment |
| --- | --- | --- |

- Issue: availabilityIndicator configuration

R2-2009747	Correction on configuration of availabilityIndicator	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2123	-	F	NR_IAB-Core

Rapporteur’s view: The issue, as described in the cover page, is that the availabilityIndicator is a IAB node specific parameter rather than IAB-MT’s serving cell specific. One availabilityIndicator configuration is sufficient to configure all the IAB-DU’s cell of this IAB node. To avoid the any NBC ASN.1 change, the proposed change is to clarify that only one servicing cell will provide this configuration.

Question 4: Do you agree the intention of R2-2009747, i.e. clarify only one serving cell provides availabilityIndicator?

| Company | Agree? | Comment |
| --- | --- | --- |

- Issue: non-DRB in inter-RAT HO

R2-2009746	Correction on non-DRB for IAB-MT	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2122	-	F	NR_IAB-Core

Rapporteur’s view: The issue, as described in the cover page, is that the IAB-MT specific feature, i.e. non-DRB operation, does not apply to inter-RAT HO case, since it is even not supported by IAB-MT.

Question 5: Do you agree the intention of R2-2009746, i.e. remove the non-DRB operation in inter-RAT HO in sec. 5.4.3.2?

| Company | Agree? | Comment |
| --- | --- | --- |

- Issue: BH RLC bearer

R2-2009749	Corrections on BH RLC bearer	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2124	-	F	NR_IAB-Core

Change A:

- 5.3.5.2 Initiation The Network may initiate the RRC reconfiguration procedure to a UE in RRC_CONNECTED. The Network applies the procedure as follows: - the establishment of RBs (other than SRB1, that is established during RRC connection establishment) is performed only when AS security has been activated; - the addition of Secondary Cell Group and SCells is performed only when AS security has been activated; - the reconfigurationWithSync is included in secondaryCellGroup only when at least one RLC bearer or BH RLC bearer is setup in SCG; - the reconfigurationWithSync is included in masterCellGroup only when AS security has been activated, and SRB2 with at least one DRB or, for IAB, SRB2, are setup and not suspended; - the conditionalReconfiguration for CPC is included only when at least one RLC bearer is setup in SCG; - the conditionalReconfiguration for CHO is included only when AS security has been activated, and SRB2 with at least one DRB or, for IAB, SRB2, are setup and not suspended.

Rapporteur’s view: The issue, as described in the cover page, is that the reconfigurationWithSync can also be included in secondaryCellGroup when at least one BH RLC bearer is setup in SCG, for the BH link case. The intention is to add this missing case for IAB-MT.

Question 6: Do you agree the intention of 1st change in R2-2009749, i.e. add the missing case of “at least one BH RLC bearer is setup in SCG” to allow NW to include the reconfigurationWithSync in secondaryCellGroup?

| Company | Agree? | Comment |
| --- | --- | --- |

Change B:

- 5.3.5.4 Secondary cell group release The UE shall: 1> as a result of SCG release triggered by E-UTRA (i.e. (NG)EN-DC case) or NR (i.e. NR-DC case): 2> reset SCG MAC, if configured; 2> for each RLC bearer that is part of the SCG configuration: 3> perform RLC bearer release procedure as specified in 5.3.5.5.3; 2> for each BH RLC bearer that is part of the SCG configuration: 3> perform BH RLC bearer release procedure as specified in 5.3.5.5.10; 2> release the SCG configuration; 2> if CPC was configured, 3> remove all the entries within VarConditionalReconfig, if any; 2> stop timer T310 for the corresponding SpCell, if running;

Rapporteur’s view: The issue, as described in the cover page, is that the BH RLC beraer release procedure is missed in the SCG release procedure.

Question 7: Do you agree the intention of 2nd change in R2-2009749, i.e. add “perform BH RLC bearer release procedure” in case SCG release?

| Company | Agree? | Comment |
| --- | --- | --- |

- Issue: Inter-node message

R2-2010229	Support of Rel-16 features for SCG in EN-DC and NR-DC	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2192	-	F	NR_IAB-Core, NR_Mob_enh-Core

Moved from 6.1.3

[This is related to the R2-2010230 treated in [AT112-e][020][IAB] LTE RRC 36331 (vivo)]

Rapporteur’s view: The new fields, including bap-Config and iab-IP-AddressConfigrationList for IAB WI and conditionalReconfiguration for NR Mobility Enhancement WI, can be configured by the SgNB in EN-DC case. However, those are missed in the field description of scg-CellGroupConfig in the CG-Config inter-node message. Also, the procedure to release those configurations in case of SCG release in EN-DC is missing.

Please note that the Uu interface message in EN-DC is clarified/changed in R2-2010230 for TS 36.331.

(Based on some offline discussion, we believe only the 1st and 3rd changes are needed for EN-DC)

1st Change:

- 5.3.5.10 MR-DC release The UE shall: 1> as a result of MR-DC release triggered by E-UTRA or NR: 2> release SRB3, if established, as specified in 5.3.5.6.2; 2> release measConfig associated with SCG; 2> if the UE is configured with NR SCG: 3> release the SCG configuration as specified in clause 5.3.5.4; 3> release otherConfig associated with the SCG; 3> stop timers T346a, T346b, T346c, T346d and T346e associated with the SCG, if running; 3> release bap-Config associated with the SCG, if configured; 3> release iab-IP-AddressConfigurationList associated with the SCG, if configured; 3> release conditionalConfiguration associated with the SCG, if configured; 2> else if the UE is configured with E-UTRA SCG: 3> release the SCG configuration as specified in TS 36.331 [10], clause 5.3.10.19 to release the E-UTRA SCG;

3rd change:

## 11.2 Inter-node RRC messages scg-CellGroupConfig Contains the RRCReconfiguration message (containing only secondaryCellGroup and/or measConfig and/or otherConfig and/or bap-Config and/or iab-IP-AddressConfigurationList and/or conditionalReconfiguration): - to be sent to the UE, used upon SCG establishment or modification, as generated (entirely) by the (target) SgNB. In this case, the SN sets the RRCReconfiguration message in accordance with clause 6 e.g. regarding the "Need" or "Cond" statements. or - including the current SCG configuration of the UE, when provided in response to a query from MN, or in SN triggered SN change in order to enable delta signaling by the target SN. In this case, the SN sets the RRCReconfiguration message in accordance with clause 11.2.3. The field is absent if neither SCG (re)configuration nor SCG configuration query nor SN triggered SN change is performed, e.g. at inter-node capability/configuration coordination which does not result in SCG (re)configuration towards the UE. This field is not applicable in NE-DC..

Question 8: Do you agree the intention of 1st change in R2-2010229, i.e. add the release of bap-Config, iab-IP-AddressConfigurationList, and conditionalConfiguration in case of SCG release in MR-DC? (Assuming the intention of R2-2010230 for 36.331 is to be agreed).

| Company | Agree? | Comment |
| --- | --- | --- |

Question 9: Do you agree the intention of 3rd change in R2-2010229, i.e. add bap-Config, iab-IP-AddressConfigurationList, and conditionalConfiguration in the filed description of scg-CellGroupConfig? (Assuming the intention of R2-2010230 for 36.331 is to be agreed).

| Company | Agree? | Comment |
| --- | --- | --- |

- Issue: Intra-CU recovery

R2-2009750	Corrections on intra-donor CU RLF recovery and RLF cause determination	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2125	-	F	NR_IAB-Core

[Changes in 5.3.7.2]

Rapporteur’s view: The issue, as described in the cover page, is that the IAB node can only perform the intra-donor RLF recovery in R16, if it suffers BH RLF. The inter-donor RLF recovery should be avoided (not supported) in R16 specification by proper implementation. Some clarification is worth to be added to indicate that in the RRC connection re-establishment related clause to avoid the IAB-MT selecting the inter-donor cell.

#### 5.3.7 RRC connection re-establishment #### 5.3.7.2 Initiation ….. 2> release the physical channel configuration for the source SpCell; 2> discard the keys used in the source SpCell (the KgNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key), if any; 1> perform cell selection in accordance with the cell selection process as specified in TS 38.304 [20], clause 5.2.6. NOTE: IAB-MT should attempt to select the cell belongs to same IAB-donor by implementation.

Question 10: Do you agree the intention of R2-2009750 for sec. 5.3.7.2, i.e. add NOTE “IAB-MT should attempt to select the cell belongs to same IAB-donor by implementation”?

| Company | Agree? | Comment |
| --- | --- | --- |

# Conclusion and proposals

Based on the above summary, following proposals are given.

# Reference

- R2-2010149	RRC Miscellaneous Corrections	Ericsson, Huawei, HiSilicon

- R2-2009323	Miscellaneous corrections to TS 38.331 for IAB	vivo

- R2-2009747	Correction on configuration of availabilityIndicator	Huawei, HiSilicon

- R2-2009746	Correction on non-DRB for IAB-MT	Huawei, HiSilicon

- R2-2009749	Corrections on BH RLC bearer	Huawei, HiSilicon

- R2-2010229	Support of Rel-16 features for SCG in EN-DC and NR-DC	Huawei, HiSilicon

- R2-2009005	Correction on RRC function description for IAB	Fujitsu

- R2-2010638	Miscellaneous corrections for IAB  	Samsung R&D Institute UK

- R2-2009390	CR for TS38.331 on RLF cause for IAB BH RLF	ZTE, Sanechips

- R2-2010602	Cause value due to the reception of BH RLF indication	Lenovo, Motorola Mobility

- R2-2009750	Corrections on intra-donor CU RLF recovery and RLF cause determination	Huawei, HiSilicon

- R2-2010635	Transmission suspension on BH RLC channel upon IAB-MT failure 	Samsung R&D Institute UK

# Contact

| Company | Name | Email Address |
| --- | --- | --- |