3GPP TSG-RAN WG2 Meeting #112 electronic	R2-20xxx

Online, November 2nd - 13th, 2020

Agenda Item:	8.7.3.1

Source: 	Huawei, HiSilicon

Title:	Summary of [AT112-e][611][Relay] Open issues on L2 relay

Document for:	Discussion and Decision

# Introduction

This paper aims at capturing the summary of the following offline discussion:

- [AT112-e][611][Relay] Open issues on L2 relay (Huawei)

Scope: Discuss the remaining open issues on L2 relay architecture, including:

- PC5 adaptation layer

- RRC procedures (including paging)

- Remaining issues from email discussion [627]

- Remaining open items in the current TR

Intended outcome: Summary in R2-2010870

Deadline:  Wednesday 2020-11-11 1200 UTC

# Discussion

- L2 architecture and adaptation

- PC5 Adaptation layer for U2N

We have the leftover of majority view from email discussion [627] as below:

Proposal-12: RAN2 discuss the support of N:1 mapping by PC5 adaptation layer between Remote UE Uu Radio Bearer and PC5 RLC channel for relaying for L2 UE-to-NW relay.

We have the EN related to the adaptation layer in the TR.

Editor note: It is FFS if the adaptation layer is also supported at the PC5 interface between Remote UE and Relay UE.

Based on the inputs from email discussion and last meeting on-line discussion, rapporteur believes we’d better not make rush decision in the SI phase. BTW, it is difficult to make the decision since no clear evidence to show either of those is infeasible from the previous discussion.

Based on the following observations, rapporteur tries to propose a way forward on this.

Observation 1: The need of PC5 adaptation layer in L2 U2N relay depends on the need of N:1 mapping between Remote UE Uu Radio Bearer and PC5 RLC channel.

Observation 2: The system indeed works without the feature of above “N:1 mapping” and the benefit can be deeply evaluated in WI phase.

Observation 3: Based on the agreements we have, the identity information of Remote UE Uu Radio Bearer and Remote UE is included in the adaptation layer anyway. The bearer mapping function and remote UE identification function in adaptation layer seems common, regardless the protocol stack with or without PC5 adaptation.

Way forward:

- Capture both the protocol stacks with and without PC5 adaptation layer for L2 UE-to-Network relay as candidate solutions in the TR, leave the down selection to WI phase.

- In the TR sec. 4.5.1.1, remove the Editor Note: “It is FFS if the adaptation layer is also supported at the PC5 interface between Remote UE and Relay UE.”. Add normal text “Whether the adaptation layer is also supported at the PC5 interface between Remote UE and Relay UE is left to WI phase.”

Question 1: Do you have strong concern to agree the above bullets as the compromised way forward?

| Company | Any concern to bullet 1/2? | Comment |
| --- | --- | --- |
| MediaTek | No | We agree to take 1 and 2 as the WF for PC5 adaptation layer for L2 UE-to-Network relay |
| InterDigital | No | We are fine with the way forward to capture both candidate solutions since we agree that more study of the benefits of N:1 would be beneficial. There is no need to do the downselection now, as work can continue in parallel regardless of the final decision. |
| OPPO | No |  |
| Samsung | No | The way forward is sensible – to us the need for Adapt on PC5 is unclear, but we understand some companies feel differently. The best thing is to conclude on this in the WI phase, as suggested by discussion rapporteur. |
| Sony | No |  |
| Ericsson (Min) | No |  |
| Futurewei | No | It is a sensible way forward. |
| vivo | No | To make progress the way forward is fine to us. |
| Qualcomm | Yes | We still have concern: - Our current main concern: Adding adaptation layer in PC5 is purely based on forward compatibility design (e.g. multi-hop, flow control and RLF indication notification as proposed by multiple companies), and we don’t have a clear scope on what they should be. It could result in scope expansion and make it hard to complete the WI (the scope may even enlarge if we put two protocol stacks on the table). Thus, no matter whether we agree adaptation layer in PC5, we hope that PC5 adaptation layer in L2 U2N relay is clearly defined with function and operation, i.e. only supports the functions of 1-hop bearer mapping in Rel-17. - From technique view, we believe that there is no LCID issue in L2 U2N relay. Multi-hop support (as forward compatibility design) requires holistic design review, and is not appropriate to add fragmented function/features here and there that may not be useful for multi-hop at all in future releases. Hence, we prefer to make a clear conclusion in SI phase, with main concern to avoid WI scoping uncontrollable. We are positive to discuss how to close this issue in SI. |
| Spreadtrum | No |  |
| Sharp | No |  |
| CATT | No |  |
| Xiaomi | No | Although we have similar concern as QC, we could accept the WF as compromise. |
| ZTE | No |  |
| Lenovo&MM |  | We prefer to keep the current FFS since we anyway need to further discuss in WI whether the PC5 adaptation layer can be added. For the observation 3, the remote UE ID is optional in the PC5 adaptation layer if PC5 adaptation layer is added because the relay UE can identify the transmitting remote UE based on the legacy physical layer and MAC layer including the 24-bit layer 2 transmitting UE ID. |
| Apple | No |  |
| Convida | No | We support PC5 adaptation layer over for forward compatibility, e.g. support multi-hop, and unify design with U2U relay. |
| Intel | No |  |
| Nokia | No with comments | We agree that this is a reasonable compromise to move forward. However, we echo Qualcomms concerns on how to handle this discussion resonably in the WI without the time dissipating.. |

Summary: It is clear majority view the rapporteur’s way forward is acceptable. To address the concern from QC and Lenovo, we can add further clarification.

Proposal 1a: Capture both the protocol stacks with and without PC5 adaptation layer for L2 UE-to-Network relay as candidate solutions in the TR, leave the down selection to WI phase (with the discussion focuses on the N:1 mapping function for the PC5 adaptation layer).

Proposal 1b: In the TR sec. 4.5.1.1, remove the Editor Note: “It is FFS if the adaptation layer is also supported at the PC5 interface between Remote UE and Relay UE.”. Add normal text “Whether the adaptation layer is also supported at the PC5 interface between Remote UE and Relay UE is left to WI phase.”

- Adaptation layer for U2U

We will try to solve the leftover as below:

Proposal of majority from email discussion [627] Proposal-16: For L2 UE-to-UE relay, the adaptation layer over second PC5 hop can be used to support N:1 bearer mapping and data multiplexing between multiple ingress PC5 RLC channels over first PC5 hop and one egress PC5 RLC channel over second PC5 hop. Proposal-17: For L2 UE-to-UE relay, the second hop PC5 adaptation layer needs to support Remote UE identification for relaying traffic. Proposal-19: For L2 UE-to-UE relay, The identity information of Source Remote UE end-to-end Radio Bearer and the identity information of Source Remote UE needs be put into the second PC5 hop adaptation layer by Relay UE in order for Destination Remote UE to correlate the received data packets for the specific PDCP entity associated with the right end-to-end SL Radio Bearer. Proposal-20a: For L2 UE-to-UE relay, the source Remote UE identity, end-to-end RB identity, Destination Remote UE identity are carried by first hop PC5 Adaptation layer.

TR EN Editor Note: It is FFS on the details to support the N-to-1 mapping between the ingress RLC channels from multiple transmitting Remote UEs to egress RLC channels (going to the same Destination UE) at Relay UE.

Agreement: Proposal-15: [Easy] agree the following description for L2 UE-to-UE relay For L2 UE-to-UE relay, the second hop PC5 adaptation layer can be used to support bearer mapping between the ingress RLC channels over first PC5 hop and egress RLC channels over second PC5 hop at Relay UE. Proposal-21: agree the following description for L2 UE-to-UE relay Support the N:1 mapping by first hop PC5 adaptation layer between Remote UE SL Radio Bearers and first hop PC5 RLC channels for relaying. Proposal-22: agree the following description for L2 UE-to-UE relay Support the adaptation layer over first hop PC5 between Source Remote UE and Relay UE in order to identify traffic destined to different Destination Remote UEs.

Considering all above, we can simplify the proposal and try to go with the following way forward:

Way forward for L2 UE-to-UE relay:

- Adaptation layer support the N:1 bearer mapping between multiple ingress PC5 RLC channels over first PC5 hop and one egress PC5 RLC channel over second PC5 hop and support the Remote UE identification function.

- In the TR sec. 5.5.1, remove the Editor Note: “It is FFS on the details to support the N-to-1 mapping between the ingress RLC channels from multiple transmitting Remote UEs to egress RLC channels (going to the same Destination UE) at Relay UE.”

- The identity information of Source Remote UE end-to-end Radio Bearer, the identity information of Source Remote UE are included in the adaptation layer in first and second PC5 hop.

- The identity information of Target Remote UE is included in the adaptation layer in the first PC5 hop.

Question 2: Do you have strong concern to agree the above bullets as the compromised way forward?

| Company | Any concern to bullet 1/2/3/4? | Comment |
| --- | --- | --- |
| MediaTek | No | We agree to take 1/2/3/4 as the WF for L2 UE-to-UE relay, which actually reflect the majority view during post-RAN2#111e email discussion |
| InterDigital | No | We agree with the way forward. Since we agreed to support N:1 mapping for the first hop and routing of traffic to different destinations, we think it is natural to also support N:1 mapping over the second hop as well as routing of traffic from different source remote UEs to the same destination. |
| OPPO | No | We are also fine to have target remote UE ID in the second PC5 hop, it is wired to have asymmetric format for the two hops. |
| Samsung | No |  |
| Sony | No |  |
| Ericsson (Min) | Yes | We support to include target remote UE ID in the first hop, we also support to include target remote UE in the second hop. It means that we can have a fixed format for the adaptation layer header. |
| Futurewei | No | The details of adaptation layer PDU format should be worked out in normative phase. |
| Vivo | Concern on bullet 4 | We are wondering the reason why target remote UE ID is excluded in the second PC5 hop. If the target remote UE ID is not included in the adaptation layer, how can the UE know that the traffic is terminated in itself or needs to be relayed to next hop. Maybe some clarification to exclude target remote UE ID in the second PC5 hop is needed. |
| Qualcomm | No for 1)/2)/4) Yes for 3) | We don’t have concern for 1)/2)/4) on adaptation layer of L2 U2U. From technique perspective, adaptation layer is needed in 1st hop to include destination UE ID, support N:1 mapping (LCID space may be issue in case of multiple destinations) and bi-directional transmission. For 3), we don’t agree to include Source Remote UE identification info in adaptation layer of 1st hop for now, because its necessary is not clear to us. In our understanding, the relay knows: - The source Remote UE identification info from the PC5 link on which it is received even for N:1 mapping case. The source UE ID is obtained during unicast PC5 link establishment - The destination Remote UE identification info can’t be implicitly known, so adaptation layer header should include it, as 4) indicated - For 2nd hop, we agree to include source Remote UE identification in adaptation layer. For this issue, we are fine to further discuss in WI phase. |
| Spreadtrum | No | But we think the target remote UE ID should be included in the second PC5 hop as well. |
| Sharp | No |  |
| CATT | No |  |
| Xiaomi | Yes for 4) | We understand the reason to include source UE ID in the first hop is to handle the case one UE has multiple source UE IDs in different unicast connections. Following the same logic, target UE ID should also be included in the second hop. |
| ZTE | See the comments | We totally agree with 1.2 and 4. But for 3, we hold the followingf comments - Second hop PC5 adaptation layer: Identity information of Source Remote UE end-to-end RBs and destination Remote UE is necessary , but the Identity information of Source Remote UE is not neede. |
| Lenovo&MM | No for 1/2/4. yes for 3 | We are fine with 1/2/4. For 3: Source Remote UE is not needed in the first adaptation layer since layer 2 ID has been included in the legacy physical and MAC layer. the relay UE can identify the transmitting remote UE based on the legacy sidelink. In addition, the relay UE is aware of the remote UE based on PC5 RRC connection. |
| Apple | No (with a minor comment on 3) | For P3, why it is mentioned as “Source Remote UE end-to-end Radio Bearer”, I think the end-to-end bearer identify is known between Soruce remote UE and Target Remote UE for U2U case, we suggest to remove “Soruce remote UE” wording in P3 for the end-to-end bearer. |
| Convida | Yes with Comments | For 3) we propose to change “Source Remote UE end-to-end Radio Bearer” to “end-to-end Radio Bearer” since the radio bearer is between the source and the destination. The identification may locally unique within source remote UE, within the destination remote UE or within the source and destination remote UE pair. For 4), agree with OPPO that the identity information of Target Remote UE is included in the adaptation layer in the second PC5 hop for forward compatibility reason, e.g. multi-hop. For this issue, we are fine to further discuss in WI phase. |
| Intel | No | We are open to other companies’ views on including target UE ID in the second hop as well. |
| Nokia | No, with minor comment | We should at least have a decent discussion online on the view on 3 |

Summary: The concerns from companies focus on the Source Remote UE ID presence in the 1st hop and Target Remote UE ID presence in the 2nd hop. So, we can consider those as open for the candidate information to be include in the adaptation layer. The decision on the absence of some information in 1st/2nd hop is left to WI phase, as commented by companies.

Proposal 2a: For L2 UE-to-UE relay, Adaptation layer support the N:1 bearer mapping between multiple ingress PC5 RLC channels over first PC5 hop and one egress PC5 RLC channel over second PC5 hop and support the Remote UE identification function.

Proposal 2b: In the TR sec. 5.5.1, remove the Editor Note: “It is FFS on the details to support the N-to-1 mapping between the ingress RLC channels from multiple transmitting Remote UEs to egress RLC channels (going to the same Destination UE) at Relay UE.”

Proposal 2c: For L2 UE-to-UE relay, the identity information of Remote UE end-to-end Radio Bearer is included in the adaptation layer in first and second PC5 hop.

Proposal 2d: In addition, the identity information of Source Remote UE and/or the identity information of Target Remote UE are candidate information to be included in the adaptation layer, which is decided in WI phase.

- U2U RRC procedure

We have the following left proposals from email discussion [627] for the U2U connection establishment procedure.

“Proposal-29: The solution 8 and solution 9 as captured within SA2 TR23.752 are considered as baseline for the connection establishment procedure for L2 UE-to-UE Relay. RAN2 can further discuss the details of RAN2 impact, if any.”

Rapporteur would like to check companies’ view on the following formulation. Since this has been discussed in the email discussion in multiple phases, so we here try to agree on this by checking if any company has strong concern.

Way forward: For L2 UE-to-UE relay connection establishment procedure, capture in the TR that “R2 consider the SA2 solution in TR 23.752 as baseline (e.g. Solution #8 and Solution #9)”. Further R2 impacts can be discussed in WI phase, if any.

Question 3: Do you have strong concern to agree the above as the compromised way forward?

| Company | Any concern? | Comment |
| --- | --- | --- |
| MediaTek | No |  |
| InterDigital | See comments | We agree with the proposed way forward. However, we should remove solution 8 from the statement (and reference only solution 9), since solution 8 deals with relay selection. |
| OPPO | No | RAN2 design for U2U relay is tightly coupled with SA2 design, hard for RAN2 to progress without SA2 conclusion first (SA2 has not made any conclusion on U2U relay yet). |
| Samsung | No | Same understanding as OPPO. |
| Sony | No |  |
| Ericsson (Min) | No | Don’t agree with InterDigital. Selection of which SA2 solution is not RAN2 responsibility. |
| Futurewei | No | RAN support of SA2 selected solution can be worked out in normative phase. No blocking issue is identified from RAN perspective. |
| Vivo | No | For InterDigitatal’s comments, we prefer to keep both solution #8 and #9 as examples in RAN TR 23.752 since SA2 has not made any conclusion on this issue yet. It is not appropriate for RAN2 to exclude either solution. |
| Qualcomm | No | We agree with InterDigital that solution 8 covers relay selection. Thus, we think just listing solution 9 should be sufficient. But we can follow majority if most companies want to include solution 8. |
| Spreadtrum | No |  |
| Sharp | No | We share the same view with OPPO. |
| CATT | No |  |
| Xiaomi | No |  |
| ZTE | No |  |
| Lenovo&MM | No |  |
| Apple | No |  |
| Convida | No |  |
| Intel | No |  |
| Nokia | No |  |

Summary: The only minor wording comment from InterDigital is address by “and/or”, also based on other comments that we should not exclude any solutions for SA2. In addition, it is just “e.g.” as some example.

Proposal 3: For L2 UE-to-UE relay connection establishment procedure, capture in the TR that “R2 consider the SA2 solution in TR 23.752 as baseline (e.g. Solution #8 and/or Solution #9)”. Further R2 impacts can be discussed in WI phase, if any.

- U2N RRC procedure

- System information delivery

This is also to check companies’ view on the leftover from email discussion [627]. Since this has been discussed in the email discussion in multiple phases, so we here try to agree on this by checking if any company has strong concern.

Proposal-30: Relay UE can support the relaying of the system information to the Remote UE(s) and what system information can be relayed to Remote Ues can be discussed at normative phase. Proposal-33: Agree the following on-demand SI delivery principles for Remote UE for L2 UE-to-NW relay - On-demand SI request is supported for Remote UE for all RRC states (Idle/Inactive/Connected state). - Only Msg3 based on-demand SI request is supported for Remote UE during Idle or Inactive mode; For connected Remote UE, only on-demand SIB request (i.e. dedicatedSIBRequest) is supported as Rel-16. - The legacy Uu RRC procedure is reused to support the Remote UE’s on-demand SI request. - On-demand SI delivery is supported for the Remote UE(s) regardless of out-of-coverage or in-coverage, when connected with Relay UE.

Question 4: Do you have strong concern to agree the above P30/33?

| Company | Any concern to P30/33? | Comment |
| --- | --- | --- |
| MediaTek | No | We agree P30/33, which actually reflect the majority view during post-RAN2#111e email discussion |
| InterDigital | No concern for P30 Comment for P33 | We are ok with the P33 if we remove “The legacy Uu RRC procedure is reused to support the remote UE’s on-demand SI request”. While we agree with the intent of re-using MSG3-based solution as much as possible, whether the legacy procedure is re-used as is needs to be further studied in the WI phase. |
| OPPO | No |  |
| Samsung | No concern with P30 Some concern with P33 | With regards to P33, we have concerns over the fourth bullet. Our strong preference is for RAN2 to first confirm the meaning of relayed on-demand SI – whether it is the same as for the Uu link, or whether it can e.g. include a sub-set of minimum SI, or even a scenario-specific subset of overall SI. Additionally, we need to discuss what on-demand SI request itself for remote UE means. In our understanding, remote UE can just inform the SIBs needed to Relay UE. Relay UE – depending on its state, and SI acquisition mechanism (broadcast or SI request) supported by the cell to which relay UE is camped – will acquire those SIBs and provide them to remote UE. And finally, the need for system information (for which purpose the system information is helpful) is not clear for the case of an OOC remote UE. Additional clarity on this matter is needed before we can agree on principles outlined in P33. We also share InterDigital’s concerns on the third bullet. |
| Sony | Comment for P33 | We share the same view with InterDigital. Whether legacy Uu RRC procedure is reused can be discussed in WI phase. |
| Ericsson (Min) | No concern for P30 concern for P33 | For remote UE in coverage, it may be sufficient for remote UE to obtain the SI via the direct connection although the remote is also connected to the relay UE. This option is at least not excluded. |
| Futurewei | No | If there are still different views on P-33, P-30 should be enough to conclude discussions on system information in SI phase, with the understanding that “what system information can be relayed to Remote Ues can be discussed at normative phase”. |
| Vivo | No concern on P30 Concern on P33 | The On-demand SI procedure for remote UE is consisted of two parts: PC5 RRC procedure between remote UE and relay UE; and on-demand SI procedure by relay UE in the Uu backhaul. There may be some ambiguity regarding bullet 3 in P33, we think the intention is to support the legacy Uu on-demand SI procedure by relay UE to acquire SI for remote UE in the Uu backhaul. |
| Qualcomm | No concern for P30 Some concern for P33 | For P33, we also have concern on 3rd bullet: legacy Uu RRC procedure actually can’t work for IDLE/INACTIVE remote UE’s on-demand SIB acquisition. The reason is that legacy procedure of IDLE/INACTIVE UE’s on-demand SIB acquisition (specified in Rel-15) needs to receive Msg4 addressed with TC-RNTI as response before monitoring SIB. However, if remote UE is connected to relay, there is no Msg1/2 and thereby TC-RNTI can’t be obtained in Msg2. Thus, some spec change is needed if supporting IDLE/INACTIVE UE. Because CONENCTED remote UE can work, we don’t think this issue is essential, and thereby fine to further discuss the solution in WI. |
| Spreadtrum | No |  |
| Sharp | No | We agree P30/33 |
| CATT | Yes | We don’t agree principles 2 and 3 in P33. PC5 on-demand SI procedure for remote UE should be introduced to instead legacy Uu on-demand SI. We proposed a holistic analysis in our Tdoc R2-2008922 for this topic. |
| Xiaomi | Concern for P33 | We still don’t understand why OOC UE would require on-demand SIBs, considering the functionality of the SIBs are used for in-coverage UEs. |
| ZTE | Concern for P33 | For p33, we concern the 3 Although we support the on-demand SI request and delivery for Remote UE for all RRC states, whether the legacy Uu procedures can be reused directly for idle/inactive Remote UE is needed more discussion. So the wording of “The legacy Uu RRC procedure is reused to support the Remote UE’s on-demand SI request” may bring some confusion in this proposal. So we suggest to add a FFS: if the legacy Uu RRC procedure can be reused directly for all RRC states Remote UE. |
| Lenovo&MM | No | We are fine with the P30 and P33. |
| Appple | Comment on P33.3 | We prefer to change “The legacy Uu RRC procedure is reused to support the Remote UE’s on-demand SI request “ to “The legacy Uu RRC procedure is used as a starting point for the Remote UE’s on-demand SI request work in normative phase”. |
| Convida | No concern for P30 Comment for P33 | Whether the legacy procedure is re-used needs to be further studied in the WI phase. |
| Intel | No | We are generally fine with both the proposals. We have a comment regarding reusing legacy Uu procedure in P33 similar to ZTE’s; we wonder if an editor’s note or FFS can be added about the direct applicability. For e.g. can the gNB broadcast the SI upon receiving msg3 based On-demand SI request from the remote UE especially when the remote UE is out of coverage? |
| Nokia | No, with online discussions for P33 |  |

Summary: No concern to P30. Some concerns to P33. So, rapporteur propose to only agree the consensus part in SI phase.

Proposal 4: For L2 UE-to-NW relay, relay UE can support the relaying of the system information to the Remote UE(s) and what system information can be relayed to Remote UEs can be discussed at normative phase. On-demand SI request is supported for Remote UE for all RRC states (Idle/Inactive/Connected state).

- Paging

“The Option 2 as studied in TR36.746 [7] for FeD2D paging is selected as the baseline paging relaying solution for L2 UE-to-Network relaying case (i.e. Relay UE monitors the Remote UE’s Paging Occasion(s) in addition to its own Paging Occasion(s).)”

By considering the contribution R2-2009230 and the current TR, it seems no further details are essential in SI phase. Please note: 1) current TR does not exclude any of those “CN paging or RAN paging”; 2) More details (e.g. how to forward the paging message) can refers to the TR36.746; 3) P8-P15 in R2-2009230 are not excluded by the current TR.

Question 5: Do you confirm that, in L2 U2N relay, the paging relaying solution apply to both CN paging and RAN paging via option 2?

| Company | Y/N | Comment |
| --- | --- | --- |
| MediaTek | Yes | We agree that both CN paging and RAN paging can be performed via option 2. The support of RAN paging is also conditional to the support RRC-Inactive state as discussed in #610. |
| InterDigital | Yes | We see no significant difference between the two cases, as the relay UE still monitors the paging occasions for the remote UE whether the remote UE is reached via RAN paging or core network paging. |
| OPPO | Yes |  |
| Samsung | Yes |  |
| Sony | Yes |  |
| Ericsson (Min) | Yes |  |
| Futurewei | Yes |  |
| vivo | Yes |  |
| Qualcomm | Yes | We raised the issue in our contribution (R2-2008966). Besides the reasons explained by MediaTek and IDC, we think restricting to only CN paging will make further restriction on RRC state of remote UE and relay (e.g. if only support CN paging, it will imply remote UE can’t be in INACTIVE state). Furthermore, we think Rapporteur’s wording seems to leave some ambiguity, i.e. whether relay can monitor CN/RAN paging in all RRC state? So, can we reformulate as below to make it more clear? Relay UE in all RRC states (i.e. IDLE/INACTIVE/CONNECTED) can monitor both CN paging and RAN paging for IDLE/INACTIVE remote UE. |
| Spreadtrum | Yes |  |
| Sharp | Yes |  |
| CATT | Yes |  |
| Xiaomi | Yes |  |
| ZTE | Yes |  |
| Lenovo&MM | Yes |  |
| Apple | Yes |  |
| Convida | Yes |  |
| Intel | Yes | We provided related text and figure in our contribution R2-2008983 that can be considered for the TR because compared to FeD2D TR, we have to consider RAN paging in addition to CN paging. |
| Nokia | Yes |  |

Summary: All companies are fine with the proposal. For the relationship with RRC state, we can come back to this after the discussion is concluded in [610].

Proposal 5: In L2 U2N relay, the paging relaying solution apply to both CN paging and RAN paging via option 2.

Question 6: Do you have any other essential issue to be decided in SI phase for paging in L2 U2N?

| Company | Y/N | Comment |
| --- | --- | --- |
| MediaTek | No |  |
| InterDigital | No | Details of how the relay UE determines the paging occasions, and the forwarding of the paging over PC5-RRC can be handled in the WI phase. |
| OPPO | No |  |
| Samsung | No |  |
| Sony | No |  |
| Ericsson (Min) | Yes | For remote UE in coverage, it may be sufficient for remote UE to obtain the paging message from the direct connection although the remote UE is connected to relay UE. This option is at least not excluded. How will remote UE perform TAU/RNA update procedure when connected to relay UE. |
| Futurewei | No | After U2N relay is established, the remote UE should be considered as camping/connecting to the serving cell of the relay UE, through the relay UE. The scenarios suggested by Ericsson would unnecessarily cause confusion on both the remote UE and the network. |
| Vivo | Yes | The following issues may be further studied for remote UE: - RNA configuration, and its relationship with relay UE’s RNA configuration; - RNA update procedure, and its relationship with relay UE’s RNA update procedure. |
| Qualcomm | Yes | We see some contributions to propose share remote UE ID (e.g. 5G-S-TMSI) with relay for PO calculation (R2-2008983, R2-2009202). However, we have some security concern on sending 5G-S-TMSI in air interface. If this concern is not resolved, we are not sure whether paging forwarding can be supported. As solution, either of below 2 solutions are fine with us: - Alt-1: Share calculated PO directly (i.e. not share remote UE ID) - Alt-2: RAN2 send LS to SA3 We prefer Alt-1 to avoid sending LS to SA3 |
| Spreadtrum | No |  |
| Sharp | No |  |
| CATT | No |  |
| Xiaomi | No |  |
| ZTE | No |  |
| Lenovo&MM | Yes | The TAU/RNAU of the remote UE should be discussed. Furthermore, the TAU/RNAU relationship between the remote UE and the relay UE should be discussed. |
| Apple | No |  |
| Convida | Yes | When a remote UE is in coverage of the same serving cell as its relay UE, the remote UE needs to choose whether to receive the paging from the direct path and/or indirect path. We also support to send LS to SA3 to clarity whether there is a security issue in sharing the Remote UE ID to the relay UE. |
| Intel | No | We think that we can agree on the basic principle (as per option 2 with some updates) during the SI and come back with detailed solutions during WI phase. For the RNA update, the remote UE has to do it, but whether the relay UE does it on behalf of the remote UE and related details can also be discussed during WI phase. |
| Nokia | No |  |

Summary: There is no clear majority view on any other paging related issue to be solved in SI phase. That means we can come back to check on those issue mentioned by companies in next meeting or in WI phase.

For the mentioned “security issue in sharing the Remote UE ID”, it can be evaluated by SA3 on whether there is an issue. And, we always have the alternates (e.g. sharing the calculated PO) in R2 to avoid the issue, if needed.

For TAU/RNAU, no company mentioned which specific issue needs to be discussed. The issue that relay UE does the procedure on behalf of the remote UE can be considered as enhancement. Considering the baseline is always to reuse the legacy procedure for relay UE and remote UE separately, we don’t need to discuss the enhancement in SI phase for the procedure only occurring occasionally.

- RRC Basic procedures

Other than SI delivery, Paging and UAC, in L2 U2N relay, we also have the RRC procedures including RRC connection establishment, RRC reconfiguration, RRC connection re-establishment, RRC connection release, RRC connection resume.

Rapporteur has following observations:

Observation 1: For those legacy RRC procedures not requiring RA (e.g. RRC reconfiguration and RRC connection release), we can reuse the legacy RRC procedure, since L2 U2N relay supports the E2E RRC connection between remote UE and gNB. The delta part with legacy Uu procedure is only the detail on the message content design, which is stage3 work.

Observation 2: For those legacy RRC procedure requiring RA (e.g. connection establishment, connection re-establishment, connection resume), we can re-use the agreed “RRC connection establishment procedure for L2 U2N” as the baseline, to handle the relay specific part different from legacy Uu procedure (e.g. the relay discovery, and PC5 and Uu configuration/RLC bearer for relaying those RRC message, etc.).

Way forward for L2 UE-to-Network relay:

- The RRC reconfiguration and RRC connection release procedures can reuse the legacy RRC procedure, with the message content/configuration design left to WI phase.

- The RRC connection re-establishment and RRC connection resume procedures can reuse the legacy RRC procedure as baseline, by considering the agreed “connection establishment procedure of L2 UE-to-NW relay” to handle the relay specific part, with the message content design left to WI phase.

Question 7: Do you have strong concern to agree the above bullets as the compromised way forward?

| Company | Any concern to bullet 1/2? | Comment |
| --- | --- | --- |
| MediaTek | No | We agree to take 1/2 as the WF for additional RRC procedures. |
| InterDigital | No | We agree that given the connection establishment procedure is now defined taking into account the relay, the other RRC procedures for the remote UE can be simply built on top of this basic procedure. |
| OPPO | No |  |
| Samsung | No |  |
| Sony | No |  |
| Ericsson (Min) | No |  |
| Futurewei | No |  |
| vivo | No |  |
| Qualcomm | No (with one comment for confirmation) | Because these RRC procedures are essential, we think RAN2 TR should capture something like “these RRC procedures (reconfiguration, release, re-establishment and resume) are supported, and its details left to WI phase.”. Not sure whether Rapporteur has the same understanding. |
| Spreadtrum | No |  |
| Sharp | No |  |
| CATT | No |  |
| Xiaomi | No |  |
| ZTE | No with comments | Since the Observation 2 said “ handle the relay specific part different from legacy Uu procedure (e.g. the relay discovery, and PC5 and Uu configuration/RLC bearer for relaying those RRC message, etc.).”, but the configuration of PC5/ Uu for relaying those RRC message has not achieved an agreements yet . Hence, we suggest to align 2) and 1). For 2) , “ by considering the agreed ‘connection establishment procedure of L2 UE-to-NW relay’ to handle the relay specific part, with the message content/ configuration design left to WI phase.”. |
| Lenovo&MM | No |  |
| Apple | No |  |
| Convida | No |  |
| Intel | No |  |
| Nokia | No |  |

Summary: Companies have no concern on the proposals. The only update is the minor wording alignment based on ZTE’s comments.

Proposal 6a: For L2 UE-to-Network relay, the RRC reconfiguration and RRC connection release procedures can reuse the legacy RRC procedure, with the message content/configuration design left to WI phase.

Proposal 6b: For L2 UE-to-Network relay, the RRC connection re-establishment and RRC connection resume procedures can reuse the legacy RRC procedure as baseline, by considering the agreed “connection establishment procedure of L2 UE-to-NW relay” to handle the relay specific part, with the message content/configuration design left to WI phase.

- Miscellaneous EN in the TR

Followings are some Ens in the TR to be discussed.

- EN1

Editor Note: It is FFS if this PC5 L2 configuration is a default configuration that can be overridden.

Considering the agreed “connection establishment procedure of L2 UE-to-NW relay”, we can remove the EN, which means such details can be further considered in WI phase.

Step 2. The Remote UE sends the first RRC message (i.e. RRCSetupRequest) for its connection establishment with gNB via the Relay UE, using a default L2 configuration on PC5.

Question 8: Do you have strong concern to “In the TR sec. 4.5.5.1, remove the Editor Note: It is FFS if this PC5 L2 configuration is a default configuration that can be overridden.”?

| Company | Any concern? | Comment |
| --- | --- | --- |
| MediaTek | No | We agree to remove the FFS |
| InterDigital | No | The EN is no longer needed. |
| OPPO | No | Currently for sidelink we only use specified configuration, and especially this configuration may be used for SRB0 as the first RRC message, for which even Uu interface does not apply default configuration which can be overridden, so that we believe there is still uncertainty here, and we can just leave that to WI phase. |
| Samsung | No |  |
| Sony | No |  |
| Ericsson (Min) | No |  |
| Futurewei | No |  |
| vivo | No |  |
| Qualcomm | No | We think we still need to discuss whether it is: - Specified configuration (i.e. can’t be overridden by RRC as specified in section 9.1 of 38.331); - or default configuration (i.e. can be overridden by RRC as specified in section 9.2 of 38.331). This discussion can be left to WI. |
| Spreadtrum | No |  |
| Sharp | No |  |
| CATT | No |  |
| Xiaomi | No |  |
| ZTE | No with comments | We think that we are not able to ignore “ PC5 L2 configuration is a default configuration that can be overridden ” and the wording “using a default L2 configuration on PC5” in step2 still raises a confusion. Because it does not achieve an agreement about the PC5 L2 configuration, isn’t it? Although we prefer using the specified L2 configuration on PC5, which is the simplest one (such as CCCH configuration in section 9.1 of 38.331) and cannot be overridden/re-configure, we are willing to leave that to WI phase , Besides, it is noted that the description (The details for Relay UE to forward the RRCSetuprequest/RRCSetup message for remote UE at this step can be discussed in WI phase) in step 2 aims to give a suggestion to discuss the “Uu transmission configuration of SRB0”. But the PC5 L2 configuration is ignored. Hence, we suggest to reword the sentence and clarify both PC5 L2 configuration and Uu configuration for SRB0 should be left in WI. |
| Lenovo&MM | No |  |
| Apple | No | I understand the details can be decided in WI. |
| Convida | No |  |
| Intel | No |  |
| Nokia | No |  |

Summary: Companies have no concern on the proposals and are ok to left the ambiguity part to WI phase.

Proposal 7: In the TR sec. 4.5.5.1, remove the Editor Note: “It is FFS if this PC5 L2 configuration is a default configuration that can be overridden.”

- EN2

Editor note: It is FFS if the adaptation layer is also supported over the first PC5 link (i.e. the PC5 link between the transmitting Remote UE and Relay UE).

Considering the agreement below, the EN should be removed.

Agreement Proposal-22: agree the following description for L2 UE-to-UE relay Support the adaptation layer over first hop PC5 between Source Remote UE and Relay UE in order to identify traffic destined to different Destination Remote UEs.

Question 9: Do you have strong concern to “In the TR sec. 5.5.1, remove the Editor Note: It is FFS if the adaptation layer is also supported over the first PC5 link (i.e. the PC5 link between the transmitting Remote UE and Relay UE)”?

| Company | Any concern? | Comment |
| --- | --- | --- |
| MediaTek | No | We agree to remove the FFS |
| InterDigital | No | The EN is no longer needed. |
| OPPO | No |  |
| Samsung | No |  |
| Sony | No |  |
| Ericsson (Min) | No |  |
| Futurewei | No |  |
| vivo | No |  |
| Qualcomm | No | We agree to remove the FFS |
| Spreadtrum | No |  |
| Sharp | No |  |
| CATT | No |  |
| Xiaomi | No |  |
| ZTE | No |  |
| Lenovo&MM | No |  |
| Apple | NO |  |
| Convida | No |  |
| Intel | No |  |
| Nokia | No | We agree to move the FFS |

Summary: Companies have no concern on the proposals.

Proposal 8: In the TR sec. 5.5.1, remove the Editor Note: “It is FFS if the adaptation layer is also supported over the first PC5 link (i.e. the PC5 link between the transmitting Remote UE and Relay UE).”

- EN3

Editor note: It is FFS if N-to-1 bearer mapping from PC5 RLC channels to Uu interface RLC channel is supported for this case.

Considering the agreement below, the EN should be removed.

Agreement Proposal-1: [Easy] agree the following description for L2 UE-to-NW relay For L2 UE-to-NW relay, the Uu adaptation layer at Relay UE supports UL bearer mapping between ingress PC5 RLC channels for relaying and egress Uu RLC channels over the Relay UE Uu path. Proposal-2: [Easy] agree the following description for L2 UE-to-NW relay The different RBs of the same Remote UE and/or different Remote UEs can be subject to N:1 mapping and data multiplexing over Uu RLC channel. Proposal-25 [Easy]: agree the following description for L2 UE-to-NW relay gNB implementation can handle the QoS breakdown over Uu and PC5 for the end-to-end QoS enforcement of a particular session established between Remote UE and network in case of L2 based UE to Network relaying. Details of handling in case PC5 RLC channels with different e2e QoS are mapped to the same Uu RLC channel can be discussed in WI phase.

Question 10: Do you have strong concern to “In the TR sec. 4.5.1.2, remove the Editor Note: “It is FFS if N-to-1 bearer mapping from PC5 RLC channels to Uu interface RLC channel is supported for this case.”?

| Company | Any concern? | Comment |
| --- | --- | --- |
| MediaTek | No | We agree to remove the FFS |
| InterDigital | No | The EN is no longer needed. |
| OPPO | No |  |
| Samsung | No |  |
| Sony | No |  |
| Ericsson (Min) | No |  |
| Futurewei | No |  |
| vivo | No |  |
| Qualcomm | No (with one comment for clarification) | We agree to remove the FFS. But we think removing is doesn’t mean that the use of 1:1 mapping on both PC5 and Uu is precluded, right? We are not sure whether rapporteur have the same understanding. We have some concern that 1:1 mapping will never be configured, which may be useful in some cases. Thus, we request to capture the clarification. |
| Spreadtrum | No |  |
| Sharp | No |  |
| CATT | No |  |
| Xiaomi | No | This relates to offline discussion 610. But either way, this FFS should be removed. |
| ZTE | No |  |
| Lenovo&MM | No |  |
| Apple | No |  |
| Convida | No |  |
| Intel | No |  |
| Nokia | No | We agree to move the FFS |

Summary: Companies have no concern on the proposals. For the comments for QC, rapporteur clarify that it is pretty clear that we will support 1:1 mapping.

Proposal 8: In the TR sec. 4.5.1.2, remove the Editor Note: “It is FFS if N-to-1 bearer mapping from PC5 RLC channels to Uu interface RLC channel is supported for this case.”

# Conclusion and proposals

Based on the above summary, following proposals are given.

# Contact

| Company | Name | Email Address |
| --- | --- | --- |
| OPPO | Qianxi Lu | qianxi.lu@oppo.com |
| Samsung | Milos Tesanovic | m.tesanovic@samsung.com |
| Futurewei | Hao Bi | Hao.bi@futurewei.com |
| Qualcomm | Peng Cheng | chengp@qti.qualcomm.com |
| Lenovo | Lianhai Wu | Wulh5@lenovo.com |
| Apple | Zhibin Wu | Zhibin_wu@apple.com |
| Convida | Zhuo Chen | Chen.Zhuo@convidawireless.com |
| Intel | Ansab Ali | Ansab.ali@intel.com |
| Nokia | Jakob Buthler | Jakob.buthler@nokia.com |