3GPP TSG-RAN WG2 Meeting #112 electronic				R2-20xxxxx

Online, 2nd – 13th November, 2020

Agenda Item:		8.13.3	MDT

Source:			Huawei (Summary rapporteur)

Title:				Report of [AT112-e][804][NR/R17 SON/MDT] MDT enhancements (Huawei)

WI code(s):			NR_ENDC_SON_MDT_enh-Core

Document for:		Discussion and Decision

## 1	Introduction

This document is related to the following email discussion:

* [AT112-e][804][NR/R17 SON/MDT] MDT enhancements (Huawei)

Scope:

1. The following proposals should be discussed and concluded:

Discuss the proposal 1 and 2 of M6 in R2-2011011.

Proposal: NR MDT support IDC mechanism according to LTE baseline, including:

- upon detection of IDC, the UE suppress logging and tag MDT report with InDeviceCoexDetected flag.

- UE resumes the measurement logging when the IDC problem is resolved

Intended outcome: Report

Proposal 1: For EN-DC, choose one of the three directions:

- LTE and NR logged MDT configurations are independent, and UE performs logging based on the logged MDT configuration of the same RAT it camps.

- In EN-DC where UE cannot camp on NR cells, UE logs the NR measurements based on network configurations.

- No need to introduce SN configuration for logged MDT. R17 MRDC enh covers SN configuration fo early measurements on non-camping frequencies.

2. Based on R2-2011012, to figure out all the logged MDT enhancements raised in the documents and collect companies’ interest on each. No need to do technical discussion through and just show your interest on the topics.

Intended outcome: Report

Deadline:  23:00, Thursday, 2020-11-12

In section 2, some tables are put to collect companies’ comments. In addition, please also add the contact information in the following table.

| Company | Email |
| --- | --- |
| Ericsson | pradeepa.ramachandra@ericsson.com |
| OPPO | linxue@oppo.com |

## 2	Summary

### 2.1	M6 discussions (from [1])

P1 and P2 are from [1].

Proposal 1: For split bearer (i.e. MN terminated split bearer, SN terminated split bearer) for D1, basically there are the following options:

- No differentiation and UE reports a single D1 to network. ([5], Qualcomm; [10], ZTE)

- Need differenation between MN and SN

  - Option 1: D1 measurement for MN terminated bearers is configured by and reported to MN. Vice versa, i.e. only the node hosting the PDCP entity configures the D1 measurement. UE reports two D1s to the node hosting the PDCP entity in one RRC message. ([4], OPPO; [9], Huawei; [6], Nokia (In [6], P2 seems to be aligned with Option 1))

  - Option 2: D1 measurement is configured by and reported to the node with lower layer configurations, i.e. MN and SN can independently configure the UE with D1 measurements in the split bearer. UE reports the D1 to each node ([4], OPPO; [3], CATT; [8], Ericsson)

Proposal 2: For MN terminated SCG bearer and SN terminated MCG bearer, basically there are the following options:

  - Alt 1: The terminated node, e.g., MN in case of MN terminated SCG bearer,configures the configuration to UE ([10], ZTE; [9], Huawei; [6], Nokia (In [6], P2 seems to be aligned with Alt 1))

  - Alt 2: The serving node, e.g., SN in case of MN terminated SCG bearer, configures the configuration to UE ([10], ZTE; [8], Ericsson)

For P1, please provide your comments in the following table if any.

| Company | Preferred option | Comments |
| --- | --- | --- |
| Ericsson | Option-2 | The D1 measurement is about the time difference between the time of arrival of a packet in the PDCP layer of the UE (Tstart) to the time of sending the corresponding packet based on the scheduling grant (Tend). Tstart is not impacted by whether the packet is sent over MCG or over SCG. However, Tend is dependent on the scheduling grant provided by MCG or the SCG. Therefore, it is important to separate the impact of this scheduling delay as observed by the UE via two different D1 measurements, one towards the MCG and the other towards the SCG. Further, for us we have to retain the principle that if the associated measurement is based on the MCG related configurations, then the measurement should be reported to the MN and if the associated measurement is based on the SCG related configurations, then the measurement should be reported to the SN. In the case of split bearer, the packets can be sent over both MCG and SCG. Therefore, the measurements related to the MCG associated scheduling impacts should be reported to the MN (Tend is based on the packets sent over MCG) and the SCG associated scheduling impacts should be reported to the SN (Tend is based on the packets sent over SCG). Based on this, we prefer Option-2. |
| OPPO | Option1/Option2 | In our understanding, the scheduling efficiency of two nodes is different, which will result in different transmission latency. For split bearers where both the two paths are involved, D1 measured for each node would be different. We think it is beneficial to reflect this differentiation in the report, which can be used to evaluate the delay impacted by different nodes. Accordingly, the optimization from network can be more precise based on these separate measurements. Regarding the configuration and reporting, we are fine with either Option1 or Option2. But for option2, we are not very clear whether the D1 measurement configuration is actually acquired from the node with PDCP entities before the node with lower layer configurations provides the configurations to the UE? |

For P2, please provide your comments in the following table if any.

| Company | Preferred option | Comments |
| --- | --- | --- |
| Ericsson | Alt2 | For us we have to retain the principle that if the associated measurement is based on the MCG related configurations, then the measurement should be reported to the MN and if the associated measurement is based on the SCG related configurations, then the measurement should be reported to the SN. Based on this principle, we prefer alternative-2. |
| OPPO | Alt1/Alt2 | Both the two alternatives can work. Same as P1, we think it needs to be clarified whether there is interaction between MN and SN before the node with lower configurations indicates the measurement to UE. |

### 2.2	IDC in logged MDT (from [2])

The following proposal is from [2].

Proposal: NR MDT support IDC mechanism according to LTE baseline, including:

- upon detection of IDC, the UE suppress logging and tag MDT report with InDeviceCoexDetected flag.

- UE resumes the measurement logging when the IDC problem is resolved

Please provide your comments in the following table if any.

| Company | Agree? | Comments |
| --- | --- | --- |
| Ericsson | Agree |  |
| OPPO | Yes |  |

### 2.3	Logged MDT in EN-DC (from [2])

The following proposal is from [2].

Proposal 1: For EN-DC, choose one of the four directions:

(1) LTE and NR logged MDT configurations are independent, and UE performs logging based on the logged MDT configuration of the same RAT it camps.

(2) In EN-DC where UE cannot camp on NR cells, UE logs the NR measurements based on network configurations.

(3) No need to introduce SN configuration for logged MDT. R17 MRDC enh covers SN configuration fo early measurements on non-camping frequencies.

(4) Enhanced areaConfiguration wherein a list of areaConfig could be provided to the UE wherein one list is associated to LTE and the other is associated is NR.

Please provide your comments in the following table if any.

| Company | Preferred option | Comments |
| --- | --- | --- |
| Ericsson | Option-4 | As we have explained in our contribution (R2-2010035), the biggest limitation with the current MDT configuration. The main issue is that unless a UE is currently configured NR SA, the network cannot configure the UE with any logged MDT configuration associated to NR. The operator has to wait until the UE is in connected mode in NR to configure NR specific logged MDT configuration. We believe that we should discuss solutions that enable the network to configure the UE with LTE and/or NR logged MDT configuration independent of UE’s connected mode of operation at a given point in time i.e., independent of whether the UE is in LTE SA, NR SA, NR-DC, EN-DC. With option-1, option-2 or option-3 the limitation as described above remains. With some of these options, the UE can be configured with NR specific logged MDT configuration only when the UE is connected to NR in the SA setup or EN-DC setup. This still hampers the NR logged MDT configuration possibility for the operators and also these solutions required MN-SN coordination, thus increasing network overhead and larger specification impacts. Based on this, we prefer option-4. |
| OPPO | Option1 | We think the method in option1 is feasible for the UE to provide the network with a set of continuous and elaborated measurement results. UE would collect the measurements according to the MDT configuration corresponding to the RAT where it camps. For Option2, we think the information contained in SIB24 is sufficient for the UE to collect the NR measurements when UE camps on LTE cells. For Option3, the logging might be discontinuous when UE camps on NR cells since there is no related SN configuration. |

### 2.4	Collect interests for logged MDT enhancements (from [2])

In [2], there are lots of enhancements for logged MDT, and as indicated by the session chair:

- Based on R2-2011012 [2], to figure out all the logged MDT enhancements raised in the documents and collect companies’ interest on each. No need to do technical discussion through and just show your interest on the topics.

Since section 2.2 and 2.3 have covered logged MDT in EN-DC and IDC related enhancements, so the left enhancements in [2] will be discussed in this section 2.4.

Please provide your comments in the following table if any.

| Feature Name | Support to pursue this feature (please add your company name in the column if this is your company’s preference) | No interest in this feature (please add your company name in the column if this is your company’s preference) |
| --- | --- | --- |
| Logged of early measurements (section 2.2.3 in [2]) | Ericsson |  |
| MDT and NR-U (section 2.2.4.1 in [2]) | Ericsson, OPPO |  |
| MDT and SNPN (section 2.2.4.2 in [2]) |  | Ericsson, OPPO |
| UE power savings on logged MDT (section 2.2.4.3 in [2]) | Ericsson, OPPO |  |
| MDT and On-demand SI (CMCC) (section 2.2.4.4 in [2]) | Ericsson, OPPO |  |
| MDT and NTN (section 2.2.4.5 in [2]) |  | Ericsson, OPPO |
| MDT logging in RRC states (section 2.2.5.1 in [2]) | Ericsson (this was discussed in Rel16 but we can discuss this again in Rel17 if there is any benefit based on RAN4 work) | OPPO |
| Enhanced OOC (section 2.2.5.2 in [2]) |  | Ericsson |
| Neighbour frequency coverage hole (section 2.2.5.3 in [2]) | Ericsson |  |
| Beam measurement (section 2.2.5.4 in [2]) | Ericsson, OPPO |  |
| Logged MDT measurements on overlapping carriers and non-overlapping carriers (section 2.2.5.5 in [2]) | Ericsson (Related to early measurements) |  |
| UE-based positioning and MDT (section 2.2.5.6 in [2]) | Ericsson | OPPO |
| Management based logged MDT overwriting signalling based logged MDT (section 2.2.5.7 in [2]) | Ericsson (could be fixed in Rel16 itself) | OPPO |
| DataInactivityTimer related discussions (section 2.2.5.8 in [2]) |  | Ericsson (not MDT related), OPPO |

## 3	Conclusion

[To be added]

## 4	Relevant Tdocs

[1] R2-2011011, Summary on 8.13.3.1 Immediate MDT enhancements

[2] R2-2011012, Summary on 8.13.3.2 Logged MDT enhancements