3GPP TSG RAN WG2#112-e	R2-2008727

Online meeting, 2nd - 13th November, 2020

3GPP TSG-RAN WG3 #109-e	R3-205765

17-28 August 2020

Online

Title:	Band selection and indication on single connectivity

Response to:	-

Release:	Rel-16

Study Item:	NR_newRAT-Core

Source:	TSG RAN3

To:	TSG RAN2

Cc:

Contact Person:

#### Name:	Yin Gao

E-mail Address:	gao.yin1@zte.com.cn

1. Overall Description:

RAN3 has discussed the band selection scenario as described below:

There is a cell with multiple overlapping bands, e.g., two, n2 and n25. Also, let’s assume there are UEs that can access both bands. A UE selects the first band in the frequencyBandList (broadcast in SIB1) that it supports, according to supportedBandListNR reported in UE-NR-Capability. It may be needed to select for this UE another band in the current serving cell during RRC connection setup phase or after RRC connection is established. If this happens, in a split gNB the gNB-DU needs to inform the gNB-CU of this change (note that the gNB-CU is not assumed to decode the CellGroupConfig IE), which may be used to update the measurement configuration for the UE.

RAN3 has discussed whether the new band selected for the UE can be indicated over F1 using the Selected BandCombinationIndex as defined in TS 38.331.

Regardless of the split gNB case, RAN3 kindly requests RAN2 to confirm that it is possible for the network to change the band on which the UE is served in the scenario above.

RAN3 would also like to ask whether the Selected BandCombinationIndex, as defined in TS38.331, can be used to indicate the new single band selected in the scenario above.

2. Actions:

To RAN2 group:

ACTION: 	RAN3 kindly asks RAN2 to address the questions in this LS.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #110-e	2nd- 12th  Nov. 2020