3GPP TSG-RAN WG2 Meeting #112-e	R2-2010933

Online, 2 – 13 November 2020

Title:	LS on per-table MCS range for mode-2

Response to:

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	RAN2

To:	RAN1

Cc:

Contact Person:

Name:	Qianxi Lu

Tel. Number:

E-mail Address:	qianxi.lu@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

Following RAN1 request on RRC parameter definition, RAN2 defined

- Per-pool configuration of additional MCS table, for the following parameter (in R1-2003190), and

| additional-mcs-Table-SL | new | mcs-Table-SL | Indicates MCS table(s) additionally used in a resource pool. FFS in RAN1 on whether multiple MCS tables can be configured in a resource pool. 64QAM table is (pre-)configured as default. Zero, one or two additional can be additionally (pre-)configured using the 256QAM and/or low-SE MCS tables | {qam256}, {qam64LowSE}, {qam256, qam64LowSE} |
| --- | --- | --- | --- | --- |

- Per-table configuration of MCS range for each additional MCS table for mode-1 operation (in R1-2005050)

| minMcs-Mode1 | new | minMcs-Mode1 | Indicates the minimum MCS value for Mode 1 configured and dynamic grants when using the associated MCS table. If no MCS is configured, UE autonomously selects MCS from the full range of values. | 0..27 |
| --- | --- | --- | --- | --- |
| maxMcs-Mode1 | new | maxMcs-Mode1 | Indicates the maximum MCS value used for Mode 1 configured and dynamic grants when using the associated MCS table. If no MCS is configured, UE autonomously selects MCS from the full range of values. | 0..31 |

For mode-2 operation, MCS-ranges specified in RRC , i.e.,

- sl-MinMCS-PSSCH and sl-MaxMCS-PSSCH included in sl-PSSCH-TxConfigList, which is the PSSCH transmission parameter table based on UE speed and synchronization source; and

- sl-MinMCS-PSSCH and sl-MaxMCS-PSSCH indicated in sl-CBR-PSSCH-TxConfigList, which is the PSSCH transmission parameter table based on CBR and priority

do not depend on the MCS-tables configured for each pool. In other words, when mode-2 UE derives MCS ranges, the selection of MCS-table is not taken into account.

RAN2 would like to request feedback from RAN1 on the following question:

Q1:  Should the MCS ranges for mode-2 operation (i.e., in the PSSCH transmission parameter table based on UE speed and synchronization source, and the PSSCH transmission parameter table based on CBR and priority) be defined per MCS table?

2. Actions:

To RAN1 group

RAN2 respectfully requests RAN1 to feedback on Q1 as above.

3. Date of Next TSG-RAN2 Meetings:

3GPPRAN2#113e	25 January – 5 February 2021	E-meeting

3GPPRAN2#113-bis-e 	12-21 April 2021	E-meeting