3GPP TSG RAN WG2#113-e	R2-2100036

e-Meeting, 25th January - 5th February, 2021

3GPP TSG-RAN3 #110-e	R3-207176

Online, 2nd November – 13th November 2020

Title:	LS on UE based solution related to Logged MDT

Release:	Release 16

Work Item:	TEI16

Source:	RAN3

To:	RAN2

Cc:

Contact Person:

#### Name:	Angelo Centonza

E-mail Address:	angelo.centonza@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN3 has discussed the RAN2 requirement that the management-based MDT configuration should not overwrite signalling based MDT configuration in all types of connectivity scenarios.

During those RAN3 discussions network-based solution is considered to be complex because:

-	For an inactive UE resuming in a new serving RAN, the old serving RAN needs to store the last known logged MDT configuration and forward it to the new serving RAN. Moreover, the remaining logging duration are required to be signaled to the new serving RAN in order to inform it of when the UE would become available for a new configuration. This implies that the old serving RAN needs to maintain a timer running for the UE even when the UE has moved to inactive state

-	For Idle UEs that move to connected mode in a new serving RAN, the last serving RAN needs to signal information to the last serving CN (such as the remaining logging duration and/or the time when the logged MDT configuration was activated). The CN needs to a running timer counting the remaining logging duration for the UE. Such information stored in the last serving CN needs to be retrieved by the new serving RAN at the time the UE moves to Connected Mode, so to allow the new serving RAN to understand whether a UE is on a signalling based logged MDT configuration and how long is left before such configuration expires. 
It is worth noticing that if the UE moves to RRC_Connected in an area served by a different AMF, the old serving AMF will need to signal the logged MDT configuration information to the new serving AMF.

-	In case of inter RAT and inter system mobility in connected and Idle mode, coordination is needed between the different RATs/System, so that information about the last active MDT configuration is signalled from source RAT/System to target RAT/System to enable an educated decision on any new logged MDT configuration for the UE. For example, if a UE in RRC_Idle is configured with a logged MDT configuration for NR, and if the UE re-selects in E-UTRAN, the old serving AMF will need to transfer MDT configuration information to the new serving MME, so that if the UE returns to RRC_Connected in the NG-RAN, the MDT configuration information can be sgnalled from the MME to the new serving AMF and then be provided to the serving NG RAN.

The above network based solutions impact multiple interfaces and therefore companies think they are complex.

Therefore, RAN3 suggests to reconsider UE based solutions in Rel17 to fulfil the RAN2 requirement asking that management based logged MDT should not overwrite signalling based logged MDT.

2	Actions

RAN3 WG respectfully asks RAN2 to analyse the issues highlighted above and to provide feedback on a Rel17 UE based solution that could be used to fulfil the requirement that management based logged MDT configurations should not overwrite signalling based logged MDT configurations.

3	Date of Next RAN3 Meetings:

RAN3#111e	25th January – 5th February 2021	Online