3GPP TSG-RAN WG2 Meeting #113-e                                                                R2-2100086

Online, Jan 25 – Feb 5, 2021

Source:	CATT

Title:	Discussion on Group Scheduling

Agenda Item:	8.1.2.4

Document for:	Discussion and Decision

Introduction

Group scheduling mechanism is one of the objectives of RAN WI [1] as below,

- Specify a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service [RAN1, RAN2] - This objective includes specifying necessary enhancements that are required to enable simultaneous operation with unicast reception.

In this contribution, we discuss the key issues related to group scheduling. More specifically, the discussions cover,

- Mapping between Group RNTI and MBS session.

- Support DRX for MBS reception

- Support SPS group-common PDSCH for MBS for RRC_CONNECTED UEs

- Simultaneous operation with unicast reception.

Discussion

Issue 1: Mapping between Group RNTI and MBS session.

Regarding the group-common PDCCH and group-common PDSCH used for MBS, RAN1#102e has made the following agreement,

- For RRC_CONNECTED UEs, at least support group-common PDCCH with CRC scrambled by a common RNTI to schedule a group-common PDSCH, where the scrambling of the group-common PDSCH is based on the same common RNTI.

Based on the above agreement, there will be common RNTI (similar as G-RNTI in LTE) used for group scheduling of MBS.

In case of G-RNTI is used for group scheduling of PTM transmission, whether the mapping between G-RNTI and MBS session is 1:1 still needs to be discussed further.

In SC-PTM, there is one to one mapping between MBMS session and MBMS traffic logical channel (SC-MTCH). G-RNTI is used to identify the transmissions of a SC-MTCH. So there is one to one mapping between TMGI and G-RNTI.

When it comes to NR MBS, LTE SC-PTM mechanism cannot be reused directly as the QoS model in NR is different from LTE.

In [3], SA2 has made the following conclusion,

- The network shall support QoS control per MBS session instead of per user. - The network shall support one or multiple QoS flow for an MBS session.

And RAN2 has also agreed that,

The function of mapping from QoS flows to MBS RBs in SDAP is needed for NR MBS. TBD whether any SDAP header is needed.

Based on the above agreement, some general principles can be summarized as following,

- MBS QoS flows of a specific MBS session could be mapped to different MRBs.

- One MRB is mapped to one MBS traffic logical channel (e.g. MTCH). So based on 1, there could be more than one MBS traffic logical channel (e.g. MTCH) used for the specific MBS session. This is different from that in SC-PTM. The difference is that in SC-PTM there is one to one mapping between MBMS session and MBMS traffic logical channel (e.g. MTCH).

To address the mapping between G-RNTI and the MBS session (TMGI), there are several potential solutions.

Solution 1: One to one mapping between G-RNTI and MBS session.

All the MBS traffic logical channels (e.g. MTCH) within the MBS session share the same G-RNTI if there are multiple MBS traffic logical channels (e.g. MTCH) in one MBS session.

Solution 2: One to one mapping between G-RNTI and MBS traffic logical channel(e.g. MTCH).

UE is configured with more than one G-RNTIs within one MBS session if there are more than one MBS traffic logical channel (e.g. MTCH) used within the MBS session. In this case UE is required to monitor more than one G-RNTI simultaneously for a specific MBS service. It seems that option 2 will increase the complexity and UE power consumption.

Based on above analysis, option 1 will be more reasonable compared to option 2.

Proposal 1: There is one to one mapping between common RNTI (i.e. G-RNTI) and MBS session.

Issue 2: Multiplexing of MBS traffic channels.

With the assumption that there is one to one mapping between G-RNTI and MBS session (TMGI), all the MBS traffic channel (e.g. MTCH) within the same MBS session will use the same G-RNTI.

UE interested in one MBS session needs to receive data from all the related MBS traffic channels of the specific MBS session. So there will be no problem if data from different MBS traffic channels of the same MBS session is multiplexed into one transport block (i.e.MAC PDU). This will enable a flexible scheduling in NG-RAN and could be easily supported by reusing legacy unicast function.

Proposal 2: Multiplexing of MBS traffic channels within one specific MBS session is supported.

Issue 3: Support DRX for MBS reception.

In LTE SC-PTM, the benefit of applying DRX pattern for MBS reception and the general principles for the MBS DRX configuration has been discussed extensively as in [2]. When it comes to NR MBS, the need to support DRX pattern for MBS reception are same as LTE MBMS, therefore we have no reason to deviate from the general principle for DRX in SC-PTM.

Proposal 3: For MBS reception, the following general principle for DRX pattern in SC-PTM is taken as baseline,

-The MBS reception opportunities are independent of the unicast DRX scheme.

-MBS DRX pattern is configured on a per MBS service basis.

Furthermore, whether the MBS DRX pattern is applicable to PTP transmission should be considered further.If the defined MBS DRX pattern is also applicable to PTP transmission, potentially complexity of UE operation in connected mode will be increased. As illustrated in Figure 1, for UE operating in PTP mode and unicast simultaneously, it needs to monitor C-RNTI with two DRX cycles at the same time, i.e. UE need to monitor C-RNTI during “on duration 1” and “on duration 2” in Figure 2 at least . The increasing complexity at UE side is unnecessary as no benefit is seen with this mechanism. So to simplify the design, when UE is operating in PTP mode, the unicast DRX configuration should be reused.

Figure 1

Proposal 4: MBS specific DRX pattern is only applicable to PTM mode. For PTP mode, unicast DRX pattern is reused.

Issue 4: Support SPS group-common PDSCH for MBS for RRC_CONNECTED UEs

RAN1 has agreed to support SPS group-common PDSCH for MBS for RRC_CONNECTED UEs. However, the detailed SPS configuration is to be determined further by RAN1.

Agreements: Support SPS group-common PDSCH for MBS for RRC_CONNECTED UEs - FFS: use group-common PDCCH or UE-specific PDCCH for SPS group-common PDSCH activation/deactivation - FFS: whether to support more than one SPS group-common PDSCH configuration per UE - FFS: whether and how uplink feedback could be configured - FFS: retransmission of SPS group-common PDSCH

RAN2’ work on SPS may mainly focus on stage-3 aspects which rely on RAN1 further conclusion on the detailed SPS configuration. So we should wait for RAN1 further progress on SPS.

Proposal 5: RAN2 should wait for RAN1 further conclusion on detailed SPS configuration.

Issue 5: Simultaneous operation with unicast reception

How to define the common frequency resource for MBS transmission will significantly impact the solution on the support simultaneous reception of unicast and multicast.

For the common frequency resource for PTM transmission, RAN1 has made a working assumption the common frequency resource for group-common PDCCH / PDSCH is confined within the frequency resource of a dedicated unicast BWP, which will ease the simultaneous reception of unicast and multicast in the same slot.

Working assumption: For multicast of RRC-CONNECTED UEs, a common frequency resource for group-common PDCCH / PDSCH is confined within the frequency resource of a dedicated unicast BWP to support simultaneous reception of unicast and multicast in the same slot - Down select from the two options for the common frequency resource for group-common PDCCH/ PDSCH - Option 2A: The common frequency resource is defined as an MBS specific BWP, which is associated with the dedicated unicast BWP and using the same numerology (SCS and CP) - FFS BWP switching is needed between the multicast reception in the MBS specific BWP and unicast reception in its associated dedicated BWP - Option 2B: The common frequency resource is defined as an ‘MBS frequency region’ with a number of contiguous PRBs, which is configured within the dedicated unicast BWP. - FFS: How to indicate the starting PRB and the length of PRBs of the MBS frequency region

RAN1 also agreed that common frequency resource for MBS could be the initial BWP or specific common frequency resources.

Agreements: For RRC_IDLE/RRC_INACTIVE UEs, define/configure common frequency resource(s) for group-common PDCCH/PDSCH. - the UE may assume the initial BWP as the default common frequency resource for group-common PDCCH/PDSCH, if a specific common frequency resource is not configured. - FFS: the relation of the common frequency resource(s) (if configured) and initial BWP.

However whether the common frequency resource is defined as an MBS specific BWP is to be decided by RAN1further. This will prevent RAN2 to discuss the detailed configuration in RRC signaling for the common frequency resource (e.g. MBS specific BWP), so RAN2 should wait for RAN1 conclusion on the MBS specific BWP.

RAN2 cannot proceed on aspects related to BWP with these undecided aspects. Therefore we should wait for further conclusion from RAN1 on BWP.

Proposal 6: RAN2 should wait for further conclusion from RAN1 on MBS specific BWP.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: There is one to one mapping between common RNTI (i.e. G-RNTI) and MBS session.

Proposal 2: Multiplexing of MBS traffic channels within one specific MBS session is supported.

Proposal 3: For MBS reception, the following general principle for DRX pattern in SC-PTM is taken as baseline,

-The MBS reception opportunities are independent of the unicast DRX scheme.

-MBS DRX pattern is configured on a per MBS service basis.

Proposal 4: MBS specific DRX pattern is only applicable to PTM mode. For PTP mode, unicast DRX pattern is reused.

Proposal 5: RAN2 should wait for RAN1 further conclusion on detailed SPS configuration.

Proposal 6: RAN2 should wait for further conclusion from RAN1 on MBS specific BWP.

Reference

- RP-201038,WID revision: NR Multicast and Broadcast Services; Huawei, HiSilicon

- R2-154195, Summary of email discussion: [91#22][LTE/SC-PTM] Scheduling pattern, DRX and Change Notification, Huawei

- TR 23.757,Study on architectural enhancements for 5G multicast-broadcast services;v1.2.0